3GPP TSG-RAN WG4 RAN4-84Bis	R4-1710933

Dubrovnik, Croatia, 9-13 October, 2017

Source:		Ericsson

Title:			Follow up paper on OOB blocking challenges

Agenda item:		8.27.3.4

Document for:	Discussion

# Introduction

This contribution continues the discussion aspects that has been brought in RAN4 #83 [1] and RAN4 #84 [2] of the OOB blocking requirement when translating into an OTA requirement. Based upon feedback and discussions with other companies, it has been noted the following observations:

1.	The required EIRP level by the closest possible interferer in order to reach -15 dBm received power level.

2.	Considering testing aspects realistic a fixed distance should be considered

3.	Considering testing aspects realistic a fixed blocking value should be considered

This contribution looks for providing elements to the discussion on OOB blocking considerations and the definition of a TS in this matter. A companion paper [3] a draft text proposal is presented as a starting point to capture such agreements discussed in this contribution.

# Discussion

Figure 1, presents the minimum required EIRP (dBm) of interferer to reach -15 dBm received power of victim. This representation considers the assumptions proposed by Huawei in [4] for the WA case, i.e. a minimum FSPL based on a minimum distance of 42 m (BS height 25 m and closest UE 35 m distant along ground at 1.5 m height), and calculating the required EIRP level by the closest possible interferer in order to reach -15 dBm received power level.

Figure 1:  Minimum required EIRP (dBm) of interferer to reach -15 dBm received power of victim.

As an example, the FSPL at 6 GHz is approximately 80 dB, and the required EIRP of a hypothetical interferer would then need to be at least 65 dBm (approximately 3.2 kW). This is a very high output power level for frequencies > 6 GHz, and represents very unrealistic scenario. In that sense, we think it is reasonable to limit the WA OOB blocking requirement to certain frequencies. Note that no antenna gains have been considered in this example (seems reasonable for the victim since the analysis is for OOB), however no other losses than the FSPL are considered either.

Limiting the OOB blocking requirement for WA OTA AAS BS would significantly simplify the specification of an OTA test method and align with the radiated immunity (RI) frequency range. It is worth mentioning that the OOB blocking requirement is the same (-15 dBm) for WA, MR, and LA cases and that the OOB blocking requirement is constant over the entire frequency range (1 MHz – 12.75 GHz) of the requirement.

# Conclusion

Based on the analysis above, we think that for the WA case, the OTA OOB blocking requirement shall be limited. One approach to consider would to be used a fixed level and a fixed distance. Definition on the approach and required limits will require discussion and agreements among the stakeholders.

# References

R4-1705495, General OOB blocking challenges, Ericsson.

R4-1707477, Further discussion on OOB blocking challenges”, Ericsson

R4-1711157, DRAFT TS 37.105 Specification text for Section 10.6 OTA RX Blocking, Ericsson.

R4-1705649, Receiver out of band blocking, Huawei.