3GPP TSG-RAN WG4 Meeting #84bis	R4-1710957

Dubrovnik, Croatia, 9 – 13 October 2017

Source: 	Nokia

Title: 					TP to TR 38.817-01: Futher ACLR agreements

Agenda Item: 			9.4.3.8.2

Document for:	Approval

# 1	Introduction

In this contribution we provide a TP to capture the further agreements on NR range 1 ALCR [1].

# 4	References

[1] R4-1709963, Further NR UE Range 1 ACLR agreements, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4#NR AH3

******************************* Start of the TP ******************************************

### 5.5.3		Adjacent Channel Leakage ratio

Concerning NR Range 1 UE ACLR requirement following agreements have been made

#### 5.5.3.1	NR ACLR

ACLR is the ratio of power of wanted signal to the power falling into Adjacent Channel. ACLR measurement bandwidth for both the wanted and adjacent channels is the maximum transmission bandwidth among the different SCSs of CP-OFDM SU for a channel BW with addition of one SCS to account for half SCS shift due to SCS alignment to DC, this measurement bandwidth is centered within the channels.

Offset for the adjacent measurement BW centre is +/- Channel BW from wanted channel centre (in the middle of the Channel).

For PC3 requirement is NRACLR = 30 dBc for channel bandwidths up to 100 MHz.

For PC2 requirement is NRACLR = 31 dBc for channel bandwidths up to 100 MHz.

Table below provides measurement BWs in the last row

Table 5.5.3.1-1: ACLR measurement bandwidth

| Sub-6GHz | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| SU_CP-OFDM [#RB] | 15 | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N.A | N.A | N.A |
|  | 30 | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
|  | 60 | N.A | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |
| RX&TXBW CP [MHz] | 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 38.88 | 48.6 | NA | NA | NA |
|  | 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.4 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
|  | 60 | NA | 7.92 | 12.96 | 17.28 | 22.32 | 36.72 | 46.8 | 56.88 | 77.04 | 97.2 |
| TXBWsym [MHz] | 15 | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 38.895 | 48.615 | NA | NA | NA |
|  | 30 | 3.99 | 8.67 | 13.71 | 18.39 | 23.43 | 38.19 | 47.91 | 58.35 | 78.15 | 98.31 |
|  | 60 | NA | 7.98 | 13.02 | 17.34 | 22.38 | 36.78 | 46.86 | 56.94 | 77.1 | 97.26 |
| maxTXBWsym [MHz] | lowest | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

#### 5.5.3.2	E-UTRA ACLR

For E-UTRA ACLR it has been agreed that if EUTRA CHBW = NR CHBW only the NR ACLR of 30 dBc shall be measured. This means that for channel bandwidths up to 20 MHz it is not necessary to measure E-UTRA ACLR as NR ACLR is tighter because the requirements in same 30 dBc but measurement bandwidth is larger.

Considering recent agreement on NR ACLR which stated that ACLR MBW is same as the bandwith of transmitted NR signal it is eveident that E-UTRA ACLR is always more relaxed thus is is not necessary.

Therefore no E-UTRA ACLR requirement is not specified for NR as NR ACLR requirement will be always more stringent.

#### 5.5.3.3	UTRA ACLR

For bands defined also for UTRA, adopt UTRAACLR1 = 33dB and UTRAACLR2 =36dB for power class 3.

UTRA ACLR requirement is indicated to the UE by NS-signalling when applicaple. Dedicated NS-value is assigned to UTRA ACLR requirement and it is used when no other additional requirement needs to indicated to the UE.

In case power reduction is necessary to meet the UTRA ACLR requirement it will be defined as A-MPRUTRA_ACLR.  A-MPRUTRA_ACLR is only allowed only in case other power reductions such as MPR and A-MPR coming from other requirement than UTRA ACLR are not sufficient. Allowed power reduction = Max (A-MPRUTRA_ACLR, (MPR+A-MPRother).

******************************* End of the TP ******************************************