3GPP TSG-RAN WG4 Meeting #84bis	R4-1711485

Dubrovnik, Croatia, 9th-13th October 2017

Source: 	Ericsson

Title:  	TP to TR 38.817-02 v0.2.0: BS ACLR for sub6GHz NR BS

Agenda Item:	9.5.3.3.1

Document for:	Approval

# Background

A skeleton for TR 38.817-02 has been approved in [1].

During RAN4#NR AH3, a way forward has been agreed on sub6GHz ACLR definition in [2]. Based on this way forward, we provided a contribution where we proposed our understanding related to ACLR definition for sub6GHz in [3].

# Proposal

We propose to adopt the text in this contribution to TR 38.817-02.

# References

R4-1710085, TR 38.817-02 v0.2.0: General aspects for BS RF for NR

R4-1710025, WF on BS ACLR for NR, NTT DoCoMo

R4-1711484, BS ACLR measurement BW for sub6GHz NR BS, Ericsson

# Text proposal for TR 38.817-02

# <Start of Text proposal>

### 6.6.3	Adjacent Channel Leakage Power Ratio (ACLR)

#### 6.6.3.1	General

The following is agreed for conducted ACLR requirement for FR1:

- Adopt the E-UTRA BS ACLR (i.e. 45dB) as ACLR1 and ACLR2 for all FR1 NR BS classes

- For ACLR definition, the same CBW with wanted signal can be assumed for adjacent CBW.

- For FR1, adopt each E-UTRA BS absolute limit to the corresponding NR BS class, whichever is less stringent compared to the ACLR.

- For NR vs NR ACLR in FR1, Transmission BW configuration of wanted channel (it would differ depend on SCS and/or CBW) should be used as a measurement BW for wanted signal power measurement.

- For NR vs E-UTRA ACLR in FR1, Specify BS 45 dB ACLR conducted requirement for 5MHz adjacent channel bandwidth only.

- For NR vs UTRA ACLR in FR1, no need to specify ACLR since it is already taken care of NR vs E-UTRA case.

In general, this is also agreed that:

- Consider the adjacent carrier bandwidth which corresponds to largest transmission BW configuration when more than one subcarrier spacing is considered.

- Consider only the test which corresponds to smallest guard band between wanted and adjacent carrier.

#### 6.6.3.2	Basic limits

Table with basic limits for NR BS in FR1.

Based on the agreements mentioned in Section 6.6.3.1, the ACLR requirement is specified in Table 6.6.3.2-1.

# Table 6.6.3.2-1: Base Station ACLR for FR1

| Channel bandwidth of E-UTRA lowest/highest carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Subcarrier spacing (kHz) at wanted carrier | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth |  | ACLR limit |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 40, 50, …, 100 | BWChannel | 15, 30, 60 | NR of same BW with the SCS that provides largest transmission BW (BWConfig) configuration | Square (BWConfig) | 45 dB |
|  | 2 x BWChannel |  |  | Square (BWConfig) | 45 dB |
|  | BWChannel/2 + 2.5MHz | 15, 30, 60 | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 1) |
|  | BWChannel/2 + 2.5MHz | 15, 30, 60 | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 1) |
| Note 1: Only applicable when the NR band is refarmed E-UTRA band. |  |  |  |  |  |

#### 6.6.3.3	non-AAS conducted ACLR requirement

Minimum conducted requirement for 1-C type NR BS in FR1 is defined as basic limit, i.e. no scaling and no antenna considered.

#### 6.6.3.4	AAS conducted ACLR requirement

Minimum conducted requirement for 1-H type NR BS in FR1 is defined based on basic limit with the emissions scaling applied.

# <End of Text proposal>