3GPP TSG-RAN WG4 Meeting #84bis 											      R4- 17114601

Dubrovnik, Crotia, 9 – 13 Oct, 2017

Source: 	Huawei, HiSilicon

Title: 	TP for TR 36.758: Specific issue for TDD operation in 3300-3400MHz band – UE sensitivity

Agenda Item:	8.12.1

Document for:	Approval

# Introduction

During last 3GPP TSG RAN Meeting #76 in West Palm Beach, USA June 5 - 8, 2017, a new WI about the study of the TDD in the C-band (3300-3400 MHz) has been approved.

This contribution provides the TP for the TR 36.758, about the UE sensitivity.

# Discussion

## Background

The text below is to define the UE sensitivity in the 3300-3400MHz band for Africa countries.

# Text proposal for TR

<Unchanged sections omitted>

# 8	Study of E-UTRA specific issues

## 8.2    UE sensitivity for E-UTRA

Taking into account that B52 and B42 have very similar technical characteristics, it is proposed to apply to the B52 the same UE reference sensitivity than the B42. Thus, for B52 the reference sensitivity is specified in Table 8.6-1 for general case:

Table 8.6-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 52 |  |  | -99 | -96 | -94.2 | -93 | TDD |

For specific case, UE(s) equipped with 4 antenna ports, the following additional requirements described in the table 8.6-2 is applied for B52:

Table 8.6-2: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 52 |  |  | -101.2 | -98.2 | -96.4 | -95.2 | TDD |

<Unchanged sections omitted>

# Conclusion

It is proposed:

Proposal: Add the text as proposed in the paragraph “Text proposal for TR” in the TR [4]

# References

[1] RP-171506 “New LTE band for 3.3-3.4 GHz for Africa”, Huawei, HiSilicon

[2] R4-1708452 “Regulation background for TDD C-Band (3300-3400MHz) for Africa”; Huawei, HiSilicon

[3] R4-1709188 “LS to African Telecommunications Union about Regulation background to protect incumbent users in the band: 3.3-3.4GHz”, 3GPP RAN WG4

[4] TR 36.756 “3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; TDD operating in 3300-3400MHz band in Africa for LTE (Release 15)”

[5] R4-1710124 “Response to the LS R4-1709188 from 3GPP to ATU about regulations background to protect incumbent users in the band”, African Telecommunications Union