3GPP TSG-RAN4 Meeting #85	R4-1712187

# Reno, Nevada, United States, 27th Nov 2017 - 1st Dec 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5308 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to Correct Pool Configuration and FRC for CBR test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Perf |  |  |  |  |  | Date: |  |  | 2017-11-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current pool configuration does not allow the test condition to be triggered. The FRC is not compatible with pool configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change pool configuration to 1 sub channel per subframe, each sub channel is 50RB Change FRC of PSSCH to 48 RB allocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current CBR measurement test is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Begin of Changes >

## A.3.24	V2X sidelink communication

### A.3.24.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for V2X sidelink communication.

### A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication

Table A.3.24.2-1: Pre-configuration for V2X Sidelink Communication (Configuration #1)

| Derivation Path: TS 36.331 [3] clause 9.3.2, SL-V2X-Preconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigCommPool-r14 ::= SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 | Indicates the offset of the first subframe of a resource pool within a SFN cycle. If absent, the resource pool starts from first subframe of SFN=0. |  |
| sl-Subframe-r14 included in SL-PreconfigV2X-TxPoolList | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |  |
| sl-Subframe-r14 included in SL-PreconfigV2X-RxPoolList | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the RX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |  |
| adjacencyPSCCH-PSSCH-r14 | True | Adjacent: TURE Non-adjacent: FALSE |  |
| sizeSubchannel-r14 | 5 | Minimum bandwidth of subchannel for adjacent transmission |  |
| startRB-Subchannel-r14 | 0 | Indicates the lowest RB index of the subchannel with the lowest index. |  |
| startRB-PSCCH-Pool-r14 | 0 | Indicates the lowest RB index of the PSCCH pool |  |
| } |  |  |  |

Table A.3.24.2-2: V2X sidelink Communication configuration for E-UTRAN (Configuration #2)

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  |  | Value (10MHz) | Comment |
| SL-V2X-InterFreqUE-SelectionConfigList-r14-DEFAULT ::= SEQUENCE (SIZE (1.. maxCellIntra)) OF SEQUENCE { |  |  |  |  |
|  | physCellIdList-r14 |  |  | Not present |
|  | typeTxSync-r14 |  | Set according to the specific test configuration | ENUMERATED {gnss, enb} |
|  | v2x-SyncConfig-r14 |  |  | Not present |
|  | v2x-CommRxPool-r14 | SL-CommResourcePoolV2X -r14 SEQUENCE { |  | RxPool A monitoring UE can receive on the resources of this pool when a transmitting UE uses the Tx Pool Normal or Tx Pool Exceptional |
|  |  | sl-OffsetIndicator-r14 | 0 | small-r12 Indicates the offset of the first subframe of a resource pool within a SFN cycle. If absent, the resource pool starts from first subframe of SFN=0. |
|  |  | sl-Subframe-r14 | 11111111 11111111 1111 | bs20-r14 for FDD Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |
|  |  | adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN Adjacent: TURE Non-adjacent: FALSE |
|  |  | sizeSubchannel-r14 | 50 | ENUMERATED {n50} Minimum bandwidth of subchannel for adjacent transmission |
|  |  | numSubchannel-r14 | 1 | ENUMERATED {n1} Number of subchannel for adjacent transmission |
|  |  | startRB-Subchannel-r14 | 0 | Indicates the lowest RB index of the subchannel with the lowest index. |
|  |  | startRB-PSCCH-Pool-r14 | 0 | Indicates the lowest RB index of the PSCCH pool |
|  |  | } |  |  |
|  | v2x-CommTxPoolNormal-r14 | SL-CommResourcePoolV2X -r14 SEQUENCE { |  | Tx Pool Normal |
|  |  | sl-OffsetIndicator-r14 | 0 | small-r12 |
|  |  | sl-Subframe-r14 | 11111111 11111111 0000 | bs20-r14 |
|  |  | adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN |
|  |  | sizeSubchannel-r14 | 50 | ENUMERATED {n50} |
|  |  | numSubchannel-r14 | 1 | ENUMERATED {n1} |
|  |  | startRB-Subchannel-r14 | 0 |  |
|  |  | startRB-PSCCH-Pool-r14 | 0 |  |
|  |  | dataTxParameters-r14 SEQUENCE { |  |  |
|  |  | alpha-r12 | al0 | Sidelink power control: al0 corresponds to 0 |
|  |  | p0-r12 | 31 | INTEGER (-126..31), unit dBm |
|  |  | } |  |  |
|  |  | } |  |  |
|  | v2x-CommTxPoolExceptional-r14 | SL-CommResourcePoolV2X -r14 SEQUENCE { |  | Tx Pool Exceptional |
|  |  | sl-OffsetIndicator-r14 | 0 | small-r12 |
|  |  | sl-Subframe-r14 | 00000000 00000000 1111 | bs20-r14 |
|  |  | adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN |
|  |  | sizeSubchannel-r14 | 50 | ENUMERATED {n50} |
|  |  | numSubchannel-r14 | 1 | ENUMERATED {n1} |
|  |  | startRB-Subchannel-r14 | 0 |  |
|  |  | startRB-PSCCH-Pool-r14 | 0 |  |
|  |  | dataTxParameters-r14 SEQUENCE { |  |  |
|  |  | alpha-r12 | al0 | Sidelink power control: al0 corresponds to 0 |
|  |  | p0-r12 | 31 | INTEGER (-126..31), unit dBm |
|  |  | } |  |  |
|  |  | } |  |  |
|  | p2x-CommTxPoolNormal-r14 |  |  | Not present |
|  | v2x-ResourceSelectionConfig-r14 | SL-CommTxPoolSensingConfig-r14-DEFAULT ::= SEQUENCE { |  |  |
|  |  | pssch-TxConfigList-r14 | SL-PSSCH-TxConfig-r14-DEFAULT |  |
|  |  | thresPSSCH-RSRP-List-r14 SEQUENCE (SIZE (64)) OF SL-ThresPSSCH-RSRP-r14 { |  |  |
|  |  | SL-ThresPSSCH-RSRP-r14[n] | 1 | For n=1,2,…,64, where n denotes the index for the threshold used for sensing based UE autonomous resource selection |
|  |  | } |  |  |
|  |  | restrictResourceReservationPeriod-r14 | 1111111111 | BIT STRING (SIZE (10)) |
|  |  | probResourceKeep-r14 | v0 |  |
|  |  | } |  |  |
|  | zoneConfig-r14 |  |  | Not present |
| } |  |  |  |  |

### A.3.24.3	Reference measurement channels for V2X Sidelink Communication

Table A.3.24.3-1: PSCCH Reference Measurement Channels

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | CC.1A HD |
| Channel bandwidth |  | MHz | 10 |
| Allocated PSCCH resource blocks |  |  | 2 |
| DFT-OFDM symbols per subframe (see Note 1) |  |  | 9 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 32 |
| Information Bit | SCI Format |  | 1 |
|  | Priority |  | As set by higher layers |
|  | Resource reservation |  | 0 |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | Retransmission index |  | Note 4 |
|  | Time gap between initial transmission and retransmission |  | 0 (Note 3) |
|  | Frequency resource location of the initial transmission and retransmission |  | Initial transmission: Set as per PSSCH RB allocation specific in the test Retransmission: Note 4 |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 16 |
| Binary Channel Bits (see Note 2) |  | Bits | 432 |
| Note 1: PSCCH transmissions are rate-matched for 10 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per TS 36.211. Note 2: Binary channel bits calculated under assumption of 9 DFT-OFDM symbols per subframe. Note 3: SFgap is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant, and SFgap =0 means no retransmission of the associated TB as per TS 36.213. Note 4: UE is allowed to autonomously select the un-used or redundant bits/code-points in SCI format 1 |  |  |  |

Table A.3.24.3-2: PSSCH Reference Measurement Channels

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CD.1A HD |
| Sidelink transmission mode |  | 4 |
| Channel bandwidth | MHz | 10 |
| Allocated PSSCH resource blocks |  | 48 |
| DFT-OFDM symbols per subframe (see Note 1) |  | 9 |
| Modulation |  | QPSK |
| Target Code Rate |  | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 3496 |
| Transport block CRC | Bits | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 |
| Binary Channel Bits (see Note 2) | Bits | 10368 |
| Note 1: PSSCH transmissions are rate-matched for 10 DFT-OFDM symbols per subframe, and the last symbol shall be punctured as per TS 36.211. Note 2: Binary channel bits calculated under assumption of 9 DFT-OFDM symbols per subframe. |  |  |

<End of Changes >