3GPP TSG-RAN WG4 Meeting #85                                      R4-1712239

Reno, Nevada, USA, 27th Nov – 1st Dec, 2017

Agenda item:			9.7.2.2

Source:			Samsung

Title:	System Simulation Results for mmWave Indoor Hotspot Scenario

Document for:			Discussion

- Introduction

In RAN4 NR#3 meeting, the updated assumptions for system level simulations are agreed in [1], while the initial discussion has begun in RAN4 #84, with the common understanding that the following discussion may be dependent on the system-level evaluation:

The following aspects are expected to be evaluated from system level simulation. - Number of SS block beams to be detected - Number of cells to be detected Note: A cell is called a detected cell to a UE if one of its beams is detected by that UE. - Number of SS block beams to be detected for a best RSRP cell - Number of SS block beams to be detected for a neighbour cell (any other detected cell than best RSRP cell) - Mobility performance - Interested companies are encouraged to provide results at 20th percentile, 50th percentile and 90th percentile of CDF for number of cells to be detected and number of beams to be detected

In RAN4 #84bis meeting, companies had been provided the simulation results and discussed on the detected beams/cells performance. However, no consensus was reached.

In this paper, we would like to provide our system level simulation results for 30GHz of FR2 under indoor hotspot scenario according to the updated simulation assumptions, and accordingly the analysis and views on the related topics.

- Simulation assumptions

- SS beam direction colliding model

  - SS blocks from different cells are colliding in the same time/frequency resource

- BS Tx beam sweeping (TDM scheme)

- Mapping between beam directions and SS blocks is random

- Each SS block within the SS burst set is transmitted in a different beam direction

- Each SS block within the SS burst is beamformed with a DFT codeword.

- UE cell measurements

  - RSRP of a cell is the RSRP of the best SS block

  - A beam is considered detected if it has an SINR≥-6dB (SINR is after UE RX beamforming when RX beamforming is used)

  - A cell is considered detected if any beam from the cell is detected

- UE Rx beamforming and antenna modelling

For above 6G, the UE Rx beamforming is considered in this paper. Based on the antenna configurations for below and above 6GHz in TR 36.802, some simplification is made. The simplification of Table A-2 is based on Table A.2.1-4 in TR 36.802.

BS antenna radiation pattern for above 6GHz and UE antenna radiation pattern can reuse Table A.2.1-6 and Table A.2.1-8 in TR 36.802. For UE RX beamforming direction, following Option 2 is used:

  - Option 1 : Isotropic antenna model is used for simulation purposes

  - Option 2 : Genie aided UE receive beamforming with codeword selection as specified in Table A-6

The detailed simulation assumptions are listed in the tables in the Annex.

- Simulation Results

In this section, we would provide the simulation results for Indoor Hotspot scenario for 30GHz carrier frequency.

In the system level simulation, multiple drops are evaluated. There are totally 12000 points (12000 UE) for drawing the CDF curves.

Figure 3.1~3.4 figures show the CDF of number of detected beams, number of detected cells, number of detected beams for the best cell, and the number of detected beams for neighbour cells, respectively. Table 3.1 shows the 20th percentile, 50th percentile and 90th percentile of the CDF curves.

| Fig. 3-1. CDF of detected beams | Fig. 3-2. CDF of detected cells |
| --- | --- |
| Fig. 3-3. CDF of detected beams for the best cell | Fig. 3-4. CDF of detected beams for neighbour cells |

Table 3.1: X%-tile of CDF of detected beams/cells

| Item | 20%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- |
| Detected beams | 28 | 36 | 48 |
| Detected cells | 4 | 6 | 8 |
| Detected beams for the best cell | 8 | 12 | 16 |
| Detected beams for neighbour cells | 2 | 3 | 10 |

Based on the above results, below observation can be made:

Observation 1: For 30GHz indoor hotspot scenario, the 20/50/90%-tile of CDF of detected beams/cells are shown as below table.

| Item | 20%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- |
| Detected beams | 28 | 36 | 48 |
| Detected cells | 4 | 6 | 8 |
| Detected beams for the best cell | 8 | 12 | 16 |
| Detected beams for neighbour cells | 2 | 3 | 10 |

- Conclusion

In this contribution, we provide our initial evaluation results for 30GHz carrier frequency Indoor Hotspot scenario according to the updated system level simulation assumptions. Based on the above evaluation results, the following observation could be obtained.

Observation 1: For 30GHz indoor hotspot scenario, the 20/50/90%-tile of CDF of detected beams/cells are shown as below table.

| Item | 20%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- |
| Detected beams | 28 | 36 | 48 |
| Detected cells | 4 | 6 | 8 |
| Detected beams for the best cell | 8 | 12 | 16 |
| Detected beams for neighbour cells | 2 | 3 | 10 |

- Reference

- R4-1709903, “System level simulation assumptions in NR RRM”, Ericsson.

- 3GPP TR 38.802, Study on New Radio Access Technology, Physical Layer Aspects

- 3GPP TR 38.900, Study on channel model for frequency spectrum above 6 GHz

- Annex A: Detailed Simulation Assumptions

Table A-1: System level evaluation assumptions for Indoor hotspot

| Parameters | Indoor hotspot |
| --- | --- |
| Layout | Single layer Indoor floor: (12BSs per 120m x 50m) |
| Inter-BS distance | 20m |
| Carrier frequency | 30GHz |
| Aggregated system bandwidth | No Carrier Aggregation |
| Simulation bandwidth | Downlink 80MHz for 30GHz |
| Channel model | 30GHz: 5GCM InH - Office |
| BS Tx power | Above 6GHz: 23dBm |
| UE Tx power | Not used in DL. |
| BS antenna configurations | See Table A.2.1-4. |
| BS antenna height | 3 m |
| BS antenna element gain + connector loss | See Table A.2.1-4 |
| BS receiver noise figure | Not used in DL. |
| UE antenna configuration | See Table A.2.1-4. |
| UE antenna height | 1 m |
| UE antenna gain | Follow the modeling of TR36.873 |
| UE receiver noise figure | Above 6GHz: 13dB |
| UE distribution | 100% Indoor, 3km/h, 100 users per BS for full buffer traffic |

Table A-2: Antenna configurations for above 6GHz

|  | Above 6GHz (30GHz) |
| --- | --- |
| TXRU mapping | Per panel, reuse models in TR 36.897. TXRU mapping to antenna elements for 30GHz: 2D DFT based beam per polarization BS TXRU mapping: all the elements for each polarization on one panel are mapped to a single antenna port, i.e. antenna elements with (M,N) = (4,8) per polarization is mapped to one TXRU; UE TXRU mapping: all the elements for each polarization on one panel are mapped to a single port, i.e. antenna elements with (M,N) = (2,2) per polarization is mapped to one TXRU. |
| TXRU mapping weights | 2D DFT based beam per polarization, TXRU to antenna mapping weights are adjustable and used to steer the panel beam direction in multi beam based approaches in time domain. |
| Number of BS antenna elements across all panels | 30GHz: 64 Tx antenna elements |
| Number of UE antenna elements | 30GHz: 8 Rx antenna elements |
| BS (M,N,P,Mg,Ng) | 30GHz: Indoor hotspot: - (M,N,P,Mg,Ng) = (4,8,2,1,1). |
| BS (dH,dV,dH,g,dV,g) | 30GHz: Indoor hotspot: - Baseline: (dH,dV) = (0.5, 0.5)λ. |
| UE antenna model parameters | Two panel: (M,N,P,Mg,Ng) = (2,2,2,1,2). dH= dV=0.5λ |
| BS antenna element gain pattern | See Table A-3 |
| UE antenna element gain pattern | See Table A-4 |

Table A-3: BS antenna radiation pattern for Indoor scenario for above 6GHz (Table A.2.1-7 in TR 38.802, Ceiling-mount)

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
| Note: Boresight direction is perpendicular to the ceiling. |  |

Table A-4: UE antenna radiation pattern model 1 (Table A.2.1-8 in TR 38.802)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

Note: are in local coordinate system.

Table A-5: Basestation Beam shaping for SS blocks for 30GHz (baseline)

| Scenario | Urban Macro | Indoor hotspot | Dense Urban |
| --- | --- | --- | --- |
| Antenna configuration (Mg,Ng,M,N,P) | (1,1,4,8,2) | (1,1,4,8,2) | (1,1,4,8,2) |
| Number of beams | 16 | 16 | TBD |
| Beam directions (θ,φ) | (102,-60) (102,-42.86) (102,-25.71) (102,-8.57) (102,8.57) (102,25.71) (102,42.86) (102, 60) (112,-60) (112,-42.86) (112,-25.71) (112,-8.57) (112,8.57) (112,25.71) (112,42.86) (112, 60) | (135,-180) (165,-180) (135, -135) (165,-135) (135,-90) (165,-90) (135, -45) (165,-45) (135,0) (165,0) (135, 45) (165, 45) (135,90) (165,90) (135, 135) (165, 135) | TBD |

Table A-6: UE RX Beam shaping for SS blocks for 30GHz (option 3)

| Antenna configuration (Mg,Ng,M,N,P) | (1,2,2,2,2) |
| --- | --- |
| UE panel orientation | For UE with (Mg, Ng) directional antenna panels. - Introduce (Ωmg,ng, Θmg,ng) for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng, where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873]. - Config 1: (Mg, Ng) = (1, 2); Θmg,ng=90; Ω0,1=Ω0,0+180; (dgH, dgV)=(0,0) - UE orientation for mobile device (Ω0,0, Θ0,0)=(U(0,360), 90); - Each antenna array has shape dH=dV=0.5λ |
| Number of beams | 4 |
| Beam directions (θ,φ) | (-45,-45) (45,-45) (-45,45) (45,45) |