3GPP TSG-RAN WG4 Meeting #85                                                             R4-1712296

Reno Nevada, USA, 27 Nov-1 Dec, 2017

Agenda Item:	9.7.11.1

Source:	Intel Corporation

Title:	SS block RSRP link level simulation result update

Document for:	Discussion

# Introduction

In this contribution, we updated our simulation results for NR-SSS based RSRP with 2Rx. Our previous results are based on 4Rx.

# Simulation results

The detailed simulation parameters can be found in [1]. The simulation results for RSRP based on NR-SSS are given with the CDF of measurement error below for the following simulation cases firstly.

- Subcarrier spacing: 15k/30K/120K/240K

- Carrier frequency: 4G Hz/30G Hz

- Number of SS blocks per SS burst set : K=1

- UE receive antennas: 2(no beamforming)

- SNR: Cell 1(1dB), cell 2(6dB)

- Cell1 ID: (2,224)  cell 2 ID (1, 112)

- Scaling of delays = 100ns for TDL A/B/C

Simulation results are in following figures.

Table 1 and Table 2 are simulation results for 15K Hz and 30K Hz respectively.

Table 3 and Table 4 are simulation results for 120K Hz and 240K Hz respectively.

Table 1:  simulation results for 15K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| EPA5 | 3 sample | -0.94 | 2.37 | 2.37 |
|  | 4 sample | -0.67 | 2.07 | 2.07 |
|  | 5 sample | -0.58 | 1.89 | 1.89 |
| ETU30 | 3 sample | -1.56 | 1.46 | 1.56 |
|  | 4 sample | -1.4 | 1.22 | 1.4 |
|  | 5 sample | -1.27 | 1.03 | 1.27 |

Table 2:  simulation results for 30K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| EPA5 | 3 sample | -0.83 | 2.19 | 2.19 |
|  | 4 sample | -0.76 | 1.99 | 1.99 |
|  | 5 sample | -0.72 | 1.97 | 1.97 |
| ETU30 | 3 sample | -2.12 | 1.23 | 2.12 |
|  | 4 sample | -1.96 | 1.04 | 1.96 |
|  | 5 sample | -1.84 | 0.94 | 1.84 |

Table 3:  simulation results for 120K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| TDL-B | 3 sample | -2.06 | 0.98 | 2.06 |
|  | 4 sample | -1.85 | 0.80 | 1.85 |
|  | 5 sample | -1.7 | 0.75 | 1.7 |

Table 4:  simulation results for 240K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| TDL-B | 3 sample | -2.18 | 1.35 | 2.18 |
|  | 4 sample | -1.96 | 1.06 | 1.96 |
|  | 5 sample | -1.84 | 0.98 | 1.84 |

From the simulation results, N=4 is considered for RSRP absolute estimation considering the margin.

Proposal 1: N=4 is suggested for absolute RSRP calculation.

# Conclusion

In this contribution some NR link level simulation results for NR SS RSRP was updated. The following proposal can be drawn:

Proposal 1: N=4 is suggested for absolute RSRP calculation.

# References

[1] R4-1706904, “N Link-level simulation assumptions for SS block measurements”, Ericsson