3GPP TSG-RAN WG4 Meeting #85	            R4-1712330

Reno, USA, 27 November - 1 December 2017

Agenda item:	9.4.3.3.2

Source: 	Intel Corporation

Title: 	MPR simulation results for FR2

Document for:	Discussion

# Introduction

In RAN4 #84 meeting, we provided our MPR simulations [1] for mmWave based on agreed WF [2] for a few predefined waveforms. In this contribution, our FR2 MPR evaluation results are presented with a complete coverage of different waveforms.

# Discussion

## Simulation assumptions

Based on our mmWave PA model, the MPR evaluation was performed by using 8 PAs, 4 for each polarizations within an antenna array panel.

Our simulation assumptions are captured below:

- EVM requirement option 1 in [3] was used, we only simulated modulation from Pi/2 BPSK up to 64QAM, Pulse shaped Pi/2 BPSK was not included.

| Modulation | EVM |
| --- | --- |
| Pi/2 BPSK | [35]% |
| QPSK | 17.5% |
| 16QAM | 12.5% |
| 64QAM | 8% |
| 256QAM | 3.5% |

- For IBE, the requirement defined in [4] was followed.

- ACLR = 16dBc was used, since we simulated 40G band. The measurement BW was determined by spectrum utilization in table 1 [3].

- SEM specified in [2] was used where ΔfOOB was up to 200% channel BW. In [5], the ΔfOOB was extended to 250% of channel BW. But the impact on MPR values is expected to be small.

- The reference waveform defined in [2] was chosen for FR2: BW=100MHz, SCS=60KHz, DFT-S-OFDM QPSK, 128RBs starting at RB0 location.

- For spectrum utilizations, we used table 1 for CP-OFDM, and table 2 [3] for DFT-S-OFDM.

Table 1. Spectrum utilizations for mmWave (CP-OFDM)

| SCS[kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

Table 2. Spectrum utilizations for mmWave (DFT-S-OFDM)

| SCS[kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 64 | 128 | 256 | N.A |
| 120 | 32 | 64 | 128 | 256 |

## Simulation results

The MPR values are provided in a similar way as in [6] in table 3, but inner and outer allocations are not differentiated. Only one MPR value is for all allocation types.

Table 3. Evaluated MPR values for mmWave

| WF type | modulation | MPR For all BW and SCS | Limiting Factors |
| --- | --- | --- | --- |
| DFT-s-OFDM | Pulse Shaped Pi/2 BPSK | -- |  |
|  | Pi/2 BPSK | 0.5 | SEM |
|  | QPSK | 0.5 | SEM |
|  | 16QAM | 1 | EVM |
|  | 64QAM | 4 | EVM |
|  | 256QAM | -- |  |
| CP-OFDM | QPSK | 2 | EVM |
|  | 16QAM | 3 | EVM |
|  | 64QAM | 6.5 | EVM |
|  | 256QAM | -- |  |

## Further discussion on reference waveform

In PA calibration, reference waveform with BW=100MHz, 60KHz SCS, DFT-S-OFDM QPSK 128RB full allocation was used to achieve peak EIRP with 0dB MPR. This aligns with our paper [7] regarding UE power class for FR2 and also follows the agreement in [2]. By using aforementioned reference waveform, a power boosting option by using pulse shaped Pi/2 BPSK is still not precluded.

It can be seen that two PAs calibrated to the same target power level with different reference waveforms, for example, one QPSK waveform and one pulse shaped Pi/2 BPSK waveform with only modulation difference, can show different non-linearity at target power level when they meet the same ACLR, SEM, spurious emission, and their corresponding EVM and IBE requirements. The one calibrated with QPSK waveform shows better linearity than the one calibrated with Pi/2 BPSK.  So it is recommended to have peak EIRP value for the UE power class is defined using above QPSK reference waveform.

Proposal 1: Use BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK waveform, 128RB0 as reference waveform with 0dB MPR to generate peak EIRP for UE power class in FR2.

We also observed that reference waveform represents a good coverage over other BW and SCS combinations with full allocations of DFT-S-OFDM QPSK signals given the spectrum utilizations are kept the same, see table 4. The highlighted in yellow is reference waveform.

Table 4. Relative Power backoff to reference waveform

| DFT-S-OFDM, QPSK |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BW | SCS | SU in RB | SU % | #RB | First RB location | Required PA backoff (dB) relative to reference waveform | Limit factor |
| 50 | 60 | 66 | 95.04% | 64 | 0 | 0 | IBE |
| 50 | 60 | 66 | 95.04% | 64 | 1 | 0 | IBE |
| 50 | 120 | 32 | 92.16% | 32 | 0 | -0.5 | SEM |
| 100 | 60 | 132 | 95.04% | 128 | 0 | 0 | IBE |
| 100 | 60 | 132 | 95.04% | 128 | 1 | 0 | IBE |
| 100 | 60 | 132 | 95.04% | 128 | 2 | 0 | IBE |
| 100 | 120 | 66 | 95.04% | 64 | 0 | 0 | IBE |
| 100 | 120 | 66 | 95.04% | 64 | 1 | 0 | IBE |
| 200 | 60 | 264 | 95.04% | 256 | 0 | 0 | IBE |
| 200 | 60 | 264 | 95.04% | 256 | 1 | 0 | IBE |
| 200 | 60 | 264 | 95.04% | 256 | 2 | 0 | IBE |
| 200 | 60 | 264 | 95.04% | 256 | 3 | 0 | IBE |
| 200 | 60 | 264 | 95.04% | 256 | 4 | 0 | IBE |
| 200 | 120 | 132 | 95.04% | 128 | 0 | 0 | IBE |
| 200 | 120 | 132 | 95.04% | 128 | 1 | 0 | IBE |
| 200 | 120 | 132 | 95.04% | 128 | 2 | 0 | IBE |
| 400 | 120 | 264 | 95.04% | 256 | 0 | 0 | IBE |
| 400 | 120 | 264 | 95.04% | 256 | 1 | 0 | IBE |
| 400 | 120 | 264 | 95.04% | 256 | 2 | 0 | IBE |
| 400 | 120 | 264 | 95.04% | 256 | 3 | 0 | IBE |
| 400 | 120 | 264 | 95.04% | 256 | 4 | 0 | IBE |

From the table, we can see the except BW50/SCS120 case which has 92.16% SU, all other full allocation waveforms have the same MPR with reference waveform.

# Conclusion

In this contribution, we provide our MPR evaluation results for mmWave, and also make the following proposal.

Proposal 1: Use BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK waveform, 128RB0 as reference waveform with 0dB MPR to generate peak EIRP for UE power class in FR2.

# References

[1] R4-1707396, ‘MPR evaluations for mmWave’, Intel Corp.

[2] R4-1706980, ‘WF on MPR evaluation assumption’, NTT DoCoMo, Inc.

[3] R4-1708814, ‘WF on MPR evaluation for mmW’, Qualcomm, Skyworks.

[4] R4-1711570, ‘WF on UE IBE for mmWave for MPR evaluation for NR’, Qualcomm.

[5] R4-17(11979) and R4-17(11510) TS 38.101-2 v0.1.0 v7 (2017-10).

[6] R4-1709944, ‘WF on MPR table and allocation definition for sub-6GHz’, Skyworks Solutions.

[7] R4-1712319, ‘UE power class for FR2’, Intel Corp.