3GPP TSG-RAN WG4 85	R4-1712335

Reno, Nevada, 27 November – 1 December, 2017

# Source:	Intel Corporation

# Title:	On difficult band combination for a single or dual UL Transmission in LTE-NR DC

# Agenda Item:	9.4.2

# Document for:	Discussion

# 1	Introduction

In RAN4-84bis meeting, the WF [1] was approved in which RAN4 agreed on a methodology how to identify difficult intra-band combination to support dual UL transmission in LTE-NR DC. In this contribution, we analyze which band combination could be categorized as difficult band or not based on the methodology.

# 2	Discussion

In WF [1], inter-band combinations that have IM2 component are categorized as difficult band and inter-band combinations between Low-Low (below 1 GHz) or High-High (1.695 – 2.695 GHz) band combinations that have IM3 are categorized as difficult band.

For this study, the band combinations approved TRs for dual connectivity (DC) band combination [2-5] are being used. In the TRs, there is no LTE re-farming band combination which RAN4 needs further study.

The following band combinations are categorized as difficult band combinations based on the methodology. The full list of analyzed band combinations are listed in Appendix.

- LTE 1DL/1UL + 1 NR case [2]

| DC combination | IMD order falling into Rx band |
| --- | --- |
| DC_1A-n77A | 2 |
| DC_3A-n77A | 2 |
| DC_3A-n78A | 2 |
| DC_20A-n28A | 3 |

- LTE 2DL/1UL + 1 NR case [3]

| DC combination | LTE 1st CC + NR Tx | LTE 2nd CC + NR Tx |
| --- | --- | --- |
|  | Fall in 1st LTE CC DL | Fall in 2nd LTE CC DL |
| DC_1A-28A-n50 | - | 2 |
| DC_3A-28A-n50 | - | 2 |
| DC_20A-28A-n50 | - | 2 |
| DC_1A-18A-n77 | 2 | - |
| DC_1A-28A-n77 | 2 | - |
| DC_1A-3A-n77 | 2 | 2 |
| DC_1A-19A-n77 | 2 | - |
| DC_1A-21A-n77 | 2 | - |
| DC_3A-19A-n77 | 2 | - |
| DC_3A-21A-n77 | 2 | - |
| DC_3A-7A-n78 | 2 | - |
| DC_3A-28A-n78 | 2 | - |
| DC_3A-19A-n78 | 2 | - |
| DC_3A-21A-n78 | 2 | - |
| DC_1A-3A-n78 | - | 2 |
| DC-3C-n78 | 2 | NA |
| DC_3A-5A-n78 | 2 | - |
| DC_3A-20A-n78 | 2 | - |
| DC_3A-38A-n78 | 2 | - |

- LTE 3DL/1UL + 1 NR case [4]

| DC combination | LTE 1st CC + NR Tx | LTE 2nd CC + NR Tx | LTE 3rd CC + NR Tx |
| --- | --- | --- | --- |
|  | Fall in 1st CC DL | Fall in 2nd CC DL | Fall in 3rd CC DL |
| DC_1A-3A-5A-n78A | - | 2 | - |
| DC_1A-3C-n78A | - | 2 | NA |
| DC_1A-3A-7A-n78A | - | 2 | - |
| DC_1A-3A-28A-n50A | - | - | 2 |
| DC_3A-7A-28A-n50A | - | - | 2 |
| DC_3A-7A-20A-n78A | 2 | - | 2 |
| DC_1A-3A-19A-n77A | 2 | 2 | - |
| DC_1A-3A-21A-n77A | 2 | 2 | - |
| DC_1A-19A-21A-n77A | 2 | 2 | - |
| DC_3A-19A-21A-n77A | 2 | - | - |
| DC_1A-3A-19A-n78A | - | 2 | - |
| DC_1A-3A-21A-n78A | - | 2 | - |
| DC_3A-19A-21A-n78A | 2 | - | - |
| DC_1A-41A-42A-n79A | - | 2 | - |
| DC_3A-7C-n78A | 2 | - | NA |
| DC_3A-5A-7A-n78A | 2 | - | - |
| DC_3A-7A-7A-n78A | 2 | - | - |
| DC_3C-20A-n78A | 2 | - | NA |
| DC_3C-7A-n78A | 2 | - | NA |
| DC_3A-7A-28A-n78A | 2 | - | - |
| DC_1A-18A-28A-n77A | 2 | - | - |

- LTE 4DL/1UL + 1 NR case [5]

| DC combination | LTE 1st CC + NR Tx | LTE 2nd CC + NR Tx | LTE 3rd CC + NR Tx | LTE 4th CC + NR Tx |
| --- | --- | --- | --- | --- |
|  | Fall in 1st CC DL | Fall in 2nd CC DL | Fall in 3rd CC DL | Fall in 4th CC DL |
| DC_1A-3A-7A-20A-n78A | - | 2 | - | - |
| DC_1A-3A-19A-21A-n77A | 2 | 2 | - | - |
| DC_1A-3A-19A-21A-n78A | - | 2 | - | - |
| DC_1A-3A-19A-21A-n79 | - | - | - | - |
| DC_1A-3A-19A-21A-n257A | - | - | - | - |
| DC_1A-3C-7A-n78A | - | 2 | - | NA |
| DC_3C-7C-n78A | 2 | - | NA | NA |
| DC_3A-7C-28A-n78A | 2 | - | - | NA |
| DC_41E-n41A | - | NA | NA | NA |
| DC_41A-41D-n41A | - | - | NA | NA |
| DC_41C-41C-n41A | - | - | NA | NA |
| DC_1A-41A-42C-n79A | - | - | - | NA |
| DC_1A-41A-42C-n257A | - | - | - | NA |
| DC_7A-46D-n78A | - | - | NA | NA |
| DC_46E-n78A | - | NA | NA | NA |
| DC_1A-3A-5A-7A-n78A | - | 2 | - | - |
| DC_3A-5A-7A-7A-n78A | - | - | - | - |
| DC_1A-3A-5A-7A-n257A | - | - | - | - |
| DC_3A-5A-7A-7A-n257A | - | - | - | - |
| DC_1A-3A-7A-20A-n28A |  | - | - | - |
| DC_1A-3A-7A-20A- n50A | - | - | - | - |
| DC_1A-3A-20A-28A- n50A | - | - | - | - |
| DC_1A-3A-7A-20A- n51A | - | - | - | - |
| DC_1A-3A-20A-28A- n51A | - | - | - | - |

# 3	Conclusions

In this contribution, we categorized difficult band combinations based on the agreed methodology.

# 4	Reference

- R4-1711618, “WF on single UL Tx switched UL”, Apple, RAN4-84bis, Dubrovnik, Croatia, Oct. 2017

- R4-1711037, “TR 37.863-01-01 V0.2 Rel-15 Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band”, RAN4-84bis, Dubrovnik, Croatia, Oct. 2017

- R4-1710236, “TR 37.863-02-1 V0.2.0 Dual connectivity (DC) band combinations of LTE 2DL/1UL + one NR band”, RAN4-84bis, Dubrovni, Croatia, Oct. 2017

- R4-1710831, “TR 37.863-03-01 v0.1.0 Rel-15 DC combinations LTE 3DL and one NR band”, RAN4-84bis, Dubrovnik, Croatia, Oct. 2017

- R4-1710950, “TR 37.863-04-01 V0.2 Rel-15 Dual connectivity (DC) band combinations of LTE 4DL/1UL + one NR band”, RAN4-84bis, Dubrovnik, Croatia, Oct. 2017

# 5	Appendix

The full list of analyzed band combinations:

- LTE 1DL/1UL + 1 NR case (full list)

| DC combination | IMD order falling into Rx band |
| --- | --- |
| DC_1A-n77A | 2 |
| DC_1A-n78A | - |
| DC_1A-n79A | - |
| DC_3A-n77A | 2 |
| DC_3A-n78A | 2 |
| DC_3A-n79A | - |
| DC_11A-n77A | - |
| DC_11A-n78A | - |
| DC_11A-n79A | - |
| DC_11A-n257A | - |
| DC_18A-n77A | - |
| DC_18A-n78A | - |
| DC_18A-n79A | - |
| DC_18A-n257A | - |
| DC_19A-n77A | - |
| DC_19A-n78A | - |
| DC_19A-n79A | - |
| DC_21A-n77A | - |
| DC_21A-n78A | - |
| DC_21A-n79A | - |
| DC_26A-n77A | - |
| DC_26A-n78A | - |
| DC_26A-n79A | - |
| DC_26A-n257A | - |
| DC_28A-n77A | - |
| DC_28A-n78A | - |
| DC_28A-n79A | - |
| DC_41A-n257A | - |
| DC_42A-n77A | - |
| DC_42A-n78A | - |
| DC_42A-n79A | - |
| DC_20A-n28A | 3 |
| DC_39A-n78A | - |
| DC_39A-n258A | - |
| DC_1A_n257 | - |
| DC_3A_n257 | - |
| DC_3A-n258A | - |
| DC_5A-n257A | - |
| DC_7A-n78 | - |
| DC_7A-n258A | - |
| DC_8A-n78A | - |
| DC_8A-n79A | - |
| DC_8A-n258A | - |
| DC_19A-n257 | - |
| DC_20A-n78A | - |
| DC_21A-n257 | - |
| DC_28A-n257 | - |
| DC_28A-n258A | - |
| DC_38A-n78A | - |
| DC_39A-n79A | - |
| DC_41A-n77A | - |
| DC_41A-n78A | - |
| DC_41A-n79A | - |
| DC_41A-n258A | - |
| DC_42A-n257 | - |
| DC_1A-n28 | - |
| DC_3A-n7 | - |
| DC_3A-n28 | - |
| DC_5A-n78A | - |
| DC_7A_n28 | - |
| DC_7A-n257A | - |
| DC_66A-n257A | - |

- LTE 2DL/1UL + 1 NR (full list)

| DC combination | LTE 1st CC + NR Tx | LTE 2nd CC + NR Tx |
| --- | --- | --- |
|  | Fall in 1st LTE CC DL | Fall in 2nd LTE CC DL |
| DC_1A-3A-n28 | - | - |
| DC_1A-7A-n28 | - | - |
| DC_1A-20A-n28 | - | - |
| DC_3A-7A-n28 | - | - |
| DC_3A-20A-n28 | - | - |
| DC_7A-20A-n28 | - | - |
| DC_41C-n41 | - | - |
| DC_1A-3A-n50 | - | - |
| DC_1A-20A-n50 | - | - |
| DC_1A-28A-n50 | - | 2 |
| DC_3A-20A-n50 | - | - |
| DC_3A-28A-n50 | - | 2 |
| DC_7C-n50 | - | NA |
| DC_1A-7A-n50 | - | - |
| DC_3A-7A-n50 | - | - |
| DC_20A-28A-n50 | - | 2 |
| DC_42C-n50 | - | NA |
| DC_1A-3A-n51 | - | - |
| DC_1A-20A-n51 | - | - |
| DC_1A-28A-n51 | - | - |
| DC_3A-20A-n51 | - | - |
| DC_3A-28A-n51 | - | - |
| DC_7C-n51 | - | NA |
| DC_1A-7A-n51 | - | - |
| DC_3A-7A-n51 | - | - |
| DC_20A-28A-n51 | - | - |
| DC_42C-n51 | - | - |
| DC_66A-71A-n71 | - | - |
| DC_2A-66A-n71 | - | - |
| DC_2A-71A-n71 | - | - |
| DC_1A-18A-n77 | 2 | - |
| DC_1A-28A-n77 | 2 | - |
| DC_18A-28A-n77 | - | - |
| DC_1A-3A-n77 | 2 | 2 |
| DC_1A-19A-n77 | 2 | - |
| DC_1A-21A-n77 | 2 | - |
| DC_3A-19A-n77 | 2 | - |
| DC_3A-21A-n77 | 2 | - |
| DC_19A-21A-n77 | - | - |
| DC_3A-7A-n78 | 2 | - |
| DC_3A-28A-n78 | 2 | - |
| DC_7A-28A-n78 | - | - |
| DC_7C-n78 | - | - |
| DC_1A-18A-n78 | - | - |
| DC_1A-28A-n78 | - | - |
| DC_18A-28A-n78 | - | - |
| DC_1A-5A-n78 | - | - |
| DC_5A-7A-n78 | - | - |
| DC_1A-19A-n78 | - | - |
| DC_1A-21A-n78 | - | - |
| DC_3A-19A-n78 | 2 | - |
| DC_3A-21A-n78 | 2 | - |
| DC_19A-21A-n78 | - | - |
| DC_1A-3A-n78 | - | 2 |
| DC_1A-5A-n78 | - | - |
| DC-3C-n78 | 2 | NA |
| DC_7C-n78 | - | - |
| DC_1A-7A-n78 | - | - |
| DC_3A-5A-n78 | 2 | - |
| DC_5A-7A-n78 | - | - |
| DC_7A-7A-n78 | - | NA |
| DC_1A-20A-n78 | - | - |
| DC_3A-20A-n78 | 2 | - |
| DC_7A-20A-n78 | - | - |
| DC_3C-n78 | - | - |
| DC_3A-38A-n78 | 2 | - |
| DC_1A-41A-n79 | - | - |
| DC_1A-42A-n79 | - | - |
| DC_41A-42A-n79 | - | - |
| DC_42C-n79 | - | - |
| DC_1A-3A-n79 | - | - |
| DC_1A-19A-n79 | - | - |
| DC_1A-21A-n79 | - | - |
| DC_3A-19A-n79 | - | - |
| DC_3A-21A-n79 | - | - |
| DC_19A-21A-n79 | - | - |
| DC_1A-41A-n257 | - | - |
| DC_1A-42A-n257 | - | - |
| DC_41A-42A-n257 | - | - |
| DC_42C-n257 | - | - |
| DC_1A-3A-n257 | - | - |
| DC_1A-19A-n257 | - | - |
| DC_1A-21A-n257 | - | - |
| DC_3A-19A-n257 | - | - |
| DC_3A-21A-n257 | - | - |
| DC_19A-21A-n257 | - | - |
| DC_1A-3A-n257 | - | - |
| DC_1A-5A-n257 | - | - |
| DC_1A-7A-n257 | - | - |
| DC_3A-5A-n257 | - | - |
| DC_3A-7A-n257 | - | - |
| DC_5A-7A-n257 | - | - |
| DC_7A-7A-n257 | - | - |

- LTE 3DL/1UL + 1 NR (full list)

| DC combination | LTE 1st CC + NR Tx | LTE 2nd CC + NR Tx | LTE 3rd CC + NR Tx |
| --- | --- | --- | --- |
|  | Fall in 1st CC DL | Fall in 2nd CC DL | Fall in 3rd CC DL |
| DC_1A-3A-5A-n78A | - | 2 | - |
| DC_1A-3A-5A-n257A | - | - | - |
| DC_1A-3C-n78A | - | 2 | NA |
| DC_1A-3C-n50A | - | - | - |
| DC_1A-3C-n51A | - | - | - |
| DC_1A-3A-7A-n78A | - | 2 | - |
| DC_1A-3A-7A-n257A | - | - | - |
| DC_1A-3A-7A- n50A | - | - | - |
| DC_1A-3A-7A- n51A | - | - | - |
| DC_1A-3A-7A-n28A | - | - | - |
| DC_1A-3A-20A-n78A | - | - | - |
| DC_1A-3A-20A-n28A | - | - | - |
| DC_1A-3A-20A-n50A | - | - | - |
| DC_1A-3A-28A-n50A | - | - | 2 |
| DC_1A-20A-28A-n50A | - | - | - |
| DC_1A-7A-20A-n50A | - | - | - |
| DC_3A-7A-20A-n50A | - | - | - |
| DC_3A-7A-28A-n50A | - | - | 2 |
| DC_1A-3A-20A-n51A | - | - | - |
| DC_1A-3A-28A-n51A | - | - | - |
| DC_1A-20A-28A-n51A | - | - | - |
| DC_1A-7A-20A-n51A | - | - | - |
| DC_3A-7A-20A-n51A | - | - | - |
| DC_3A-7A-28A-n51A | - | - | - |
| DC_1A-7A-20A-n78A | - | - | - |
| DC_1A-7A-20A-n28A | - | - | - |
| DC_3A-7A-20A-n78A | 2 | - | 2 |
| DC_3A-7A-20A-n28A | - | - | - |
| DC_1A-3A-19A-n77A | 2 | 2 | - |
| DC_1A-3A-21A-n77A | 2 | 2 | - |
| DC_1A-19A-21A-n77A | 2 | 2 | - |
| DC_3A-19A-21A-n77A | 2 | - | - |
| DC_1A-3A-19A-n78A | - | 2 | - |
| DC_1A-3A-21A-n78A | - | 2 | - |
| DC_1A-19A-21A-n78A | - | - | - |
| DC_3A-19A-21A-n78A | 2 | - | - |
| DC_1A-3A-19A-n79A | - | - | - |
| DC_1A-3A-21A-n79A | - | - | - |
| DC_1A-19A-21A-n79A | - | - | - |
| DC_1A-41A-42A-n79A | - | 2 | - |
| DC_1A-42C-n79A | - | - | NA |
| DC_41A-42C-n79A | - | - | NA |
| DC_1A-41A-42A-n257 | - | - | - |
| DC_1A-42C-n257A | - | - | - |
| DC_41A-42C-n257A | - | - | - |
| DC_3A-7C-n78A | 2 | - | - |
| DC_3A-19A-21A-n79A | - | - | - |
| DC_1A-3A-19A-n257A | - | - | - |
| DC_1A-3A-21A-n257A | - | - | - |
| DC_1A-19A-21A-n257A | - | - | - |
| DC_3A-19A-21A-n257A | - | - | - |
| DC_3A-5A-7A-n78A | 2 | - | - |
| DC_3A-5A-7A-n257A | - | - | - |
| DC_3A-7A-7A-n78A | 2 | - | - |
| DC_3A-7A-7A-n257A | - | - | - |
| DC_5A-7A-7A-n78A | - | - | - |
| DC_5A-7A-7A-n257A | - | - | - |
| DC_3C-20A-n78A | 2 | - | NA |
| DC_3C-7A-n78A | 2 | - | NA |
| DC_3A-7A-28A-n78A | 2 | - | - |
| DC_7C-28A-n78A | - | - | NA |
| DC_7A-46C-n78A | - | - | NA |
| DC_46D-n78A | - | - | - |
| DC_41D-n41A | - | - | - |
| DC_41C-41A-n41A | - | - | - |
| DC_1A-18A-28A-n78A | - | - | - |
| DC_1A-18A-28A-n77A | 2 | - | - |
| DC_1A-5A-7A-n78A | - | - | - |
| DC_1A-5A-7A-n257A | - | - | - |
| DC_2A-66A-71A-n71A | - | - | - |

- LTE 4DL/1UL + 1 NR (full list)

| DC combination | LTE 1st CC + NR Tx | LTE 2nd CC + NR Tx | LTE 3rd CC + NR Tx | LTE 4th CC + NR Tx |
| --- | --- | --- | --- | --- |
|  | Fall in 1st CC DL | Fall in 2nd CC DL | Fall in 3rd CC DL | Fall in 4th CC DL |
| DC_1A-3A-7A-20A-n78A | - | 2 | - | - |
| DC_1A-3A-19A-21A-n77A | 2 | 2 | - | - |
| DC_1A-3A-19A-21A-n78A | - | 2 | - | - |
| DC_1A-3A-19A-21A-n79 | - | - | - | - |
| DC_1A-3A-19A-21A-n257A | - | - | - | - |
| DC_1A-3C-7A-n78A | - | 2 | - | NA |
| DC_3C-7C-n78A | 2 | - | NA | NA |
| DC_3A-7C-28A-n78A | 2 | - | - | NA |
| DC_41E-n41A | - | NA | NA | NA |
| DC_41A-41D-n41A | - | - | NA | NA |
| DC_41C-41C-n41A | - | - | NA | NA |
| DC_1A-41A-42C-n79A | - | - | - | NA |
| DC_1A-41A-42C-n257A | - | - | - | NA |
| DC_7A-46D-n78A | - | - | NA | NA |
| DC_46E-n78A | - | NA | NA | NA |
| DC_1A-3A-5A-7A-n78A | - | 2 | - | - |
| DC_3A-5A-7A-7A-n78A | - | - | - | - |
| DC_1A-3A-5A-7A-n257A | - | - | - | - |
| DC_3A-5A-7A-7A-n257A | - | - | - | - |
| DC_1A-3A-7A-20A-n28A | - | - | - | - |
| DC_1A-3A-7A-20A- n50A | - | - | - | - |
| DC_1A-3A-20A-28A- n50A | - | - | - | - |
| DC_1A-3A-7A-20A- n51A | - | - | - | - |
| DC_1A-3A-20A-28A- n51A | - | - | - | - |