3GPP TSG-RAN WG4 Meeting #85	R4-1712343

Reno, U.S.A., 27 November - 1 December 2017

Agenda item:	10.1

Title: 	TP for discussion on available studies and simulation results in the TR of 8Rx SI

Source: 	Intel Corporation

Document for:	Discussion

- Introduction

The Release 15 Study Item “Study on LTE DL 8Rx antenna ports” has been approved and introduced in RAN#76 and the Study Item Description is given in [1] with an objective to study the performance gain with 8 Rx antenna configuration compared to a smaller number of receive antennas, such as 4 Rx.

In RAN4#84 and RAN4#84bis Meetings, there had been extensive discussions and simulation evaluations by different companies on the study of various 8Rx antenna ports scenarios and their corresponding performance, especially compared with 4Rx antenna ports. Based on the discussion in the 8Rx SI, two agreements among companies were captured as follows:

- In RAN4#84 Meeting:

Agreement: to use the correlation matrices in this way forward as the starting point with that the interested companies could have study to find out the limit where we do not get 8Rx gain, and do not preclude the other companies to provide the simulation results with their preferred correlation matrix for all the possible layers.

- In RAN4#84bis Meeting:

Agreement: all the available studies and simulation results from all the interested companies would be captured in the appendix of 8Rx final TR.

In this contribution, we provide a test proposal for 8Rx final TR (36.757), in particular, addressing the conclusion and appendix parts.

# Text Proposal

< Start of first change >

# 6	Conclusion

Based on all the available 8Rx studies and simulation results captured in Annex A from all the interested companies, we conclude that:

  - No new RRM and RLM requirement shall be defined for 8RX;

  - No new PDCCH/PCFICH requirements shall be defined for 8Rx;

  - PDSCH requirements for RI ≤ 4 (based on MMSE-IRC baseline receiver):

    - About 3 to 4 dB gains could be observed for noise limited scenarios;

    - Limited gains were observed for interference-limited scenarios (e.g. MU-MIMO);

    - Throughput performance is very sensitive to the Rx antenna correlation level, and gains decreases as the correlation level increases, especially for higher ranks;

  - PDSCH requirements for RI > 4 (based on MMSE-IRC baseline receiver):

    - The combination of 8-layer and 64QAM might be able to achieve the maximum configured downlink throughput within the practical operating SNR range, given zero antenna correlation and low MCS level;

    - The combination of 6/8-layer and 256QAM could not be feasible to achieve the maximum configured downlink throughput within the practical operating SNR range, even though antenna correlation is zero and MCS is low;

    - Throughput performance is considerably impacted by and extremely sensitive to Rx antenna correlation levels, and the throughput degrades dramatically if antenna correlation is not zero.

# 7	Annex A:

Annex A includes all the available 8Rx SI studies and simulation results from all the interested companies.

a. Intel simulation results [2] – [7]:

PDSCH TM2 Fading Channel Scenario:

PDSCH TM3 Fading Channel Scenario:

PDSCH TM9 Fading Channel Scenario:

## PDSCH TM9 Static Channel Scenario:

## PDSCH Miscellaneous Scenarios

Scenario (i): Single-layer Spatial Multiplexing

In Scenario (i), we consider an existing 4Rx testcase for TM9 Single-layer Spatial Multiplexing, which is an MU-MIMO testcase and specified in TS 36.101 Section 8.10.1.1.5A, and is also summarized and listed below:

Table 8.10.1.1.5A-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| PDSCH transmission mode |  |  | 9 |
| Number of MBSFN subframes |  | Subframes | NA |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. |  |  |  |

Table 8.10.1.1.5A-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.50 FDD | OP.1 FDD | EPA5 | 2x4 Low | 70 | 15.8 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

We then try to re-use the above 4Rx testcase for 8Rx configuration as a measure of comparison between 4Rx and 8Rx antenna configurations. Moreover, besides the “ULA low correlation” case, we also consider the “ULA medium correlation A” and “ULA medium correlation B” cases as specified in Table 1 below.

Table 1 The  and  parameters for ULA MIMO correlation matrices

| Correlation Model |  |  |
| --- | --- | --- |
| Low Correlation | 0 | 0 |
| Medium Correlation A | 0.3 | 0.3874 |
| Medium Correlation B | 0.3 | 0.005154 |

Fig.1 Normalized throughput performance for TM9, single-layer spatial multiplexing, EPA5

Observation 1: For the MU-MIMO Low Correlation (ULA-LowCorr) case, 2×8 antenna configuration achieves about 1.5dB gain against 2×4 antenna configuration at 70% maximum throughput.

Observation 2: For the MU-MIMO 2×4 and 2×8 both with Medium Correlation A (ULA-MedCorrA) cases, which suggests the same size/form factor for 4Rx and 8Rx, performance of 8Rx is even worse than 4Rx.

Observation 3: For the MU-MIMO 2×8 with Medium Correlation B (ULA-MedCorrB) case, we see about 4dB gain, compared with 2×4 ULA-MedCorrA case.

Scenario (ii): Dual-layer Spatial Multiplexing

In Scenario (ii), we consider cases for TM9 Dual-layer Spatial Multiplexing, and the simulation assumptions are listed in Table 2 below.

Table 2 Simulation Assumptions for Scenario 2 of TM 9 Dual-layer Spatial Multiplexing

| Duplex Mode | FDD |
| --- | --- |
| Transmission Mode (TM) | 9 |
| System Bandwidth (MHz) | 10 |
| Propagation Condition | EPA5 |
| MCS | 18 (64QAM) |
| Antenna Configuration | 4×4 / 4×8 |

Fig.2 Normalized throughput performance for TM9, dual-layer spatial multiplexing, EPA5

Observation 4: For TM9 dual-layer MCS18, 4×4 with Medium Correlation A cannot achieve the maximum throughput; while, by enlarging the size/form factor, 4×8 with Medium Correlation B can achieve maximum if SNR is sufficiently high.

Observation 5: For TM9 dual-layer MCS18, when SNR is greater than 30dB, 4×4 with Medium Correlation A obtains higher throughput than 4×8 with Medium Correlation A.

Scenario (iii): TM9 8×8 8-layer Spatial Multiplexing

In Scenario (iii), we consider cases for TM9 8×8 8-layer Spatial Multiplexing, and the simulation assumptions are listed in Table 3 below.

Table 3 Simulation Assumptions for Scenario 3 of TM 9 8×8 8-layer Spatial Multiplexing

| Duplex Mode | FDD |
| --- | --- |
| Transmission Mode (TM) | 9 |
| System Bandwidth (MHz) | 10 |
| Propagation Condition | EPA5 |
| MCS | 17 (64QAM) |
| Antenna Configuration | 8×8 |

Fig.3 Normalized throughput performance for TM9 8×8 8-layer Spatial Multiplexing

Observation 6: TM9 8×8 8-layer Spatial Multiplexing is very sensitive to the antenna correlation factor. In Fig.3, it shows that the maximum throughput for 64QAM MCS17 can be achieved when both correlation parameters α and β are zero. However, a very tiny increase in the correlation value, such as β = 10-6, would bring dramatic degradation in throughput vs. SNR performance.

## PDCCH Scenarios:

b. Company 2 simulation results:

c. Company 3 simulation results:

< End of first change >

# References

[1] RP-171491, “New SI proposal: Study on LTE DL 8Rx antenna ports”, Huawei, HiSilicon, 3GPP RAN #76, June 2017.

[2] R4-1710400, “PDSCH simulation results for 8Rx with transmit diversity”, Intel Corporation, October 2017.

[3] R4-1710401, “PDSCH simulation results for 8Rx with rank lower than or equal to 4”, Intel Corporation, October 2017.

[4] R4-1710402, “PDSCH simulation results for 8Rx with rank higher than 4”, Intel Corporation, October 2017.

[5] R4-1710403, “PDSCH simulation results for 8Rx in static propagation channels”, Intel Corporation, October 2017.

[6] R4-1710399, “Discussion on the antenna correlation impact on 8Rx scenarios”, Intel Corporation, October 2017.

[7] R4-1710406, “PDCCH Performance Evaluation with 8Rx”, Intel Corporation, October 2017.