3GPP TSG-RAN WG4 Meeting #84-bis	R4-1712371

# Dubrovnik, Croatia, 9 - 13 October, 2017

Source:	KDDI Corporation

Title:	TP for TR36.715-04-04: UE RF requirements for UL CA more than 2CCs

Agenda item:	8.15.2 – UE RF

Document for:	Approval

Introduction

During TSG RAN#77, new work item for up to 4UL CA with Band 41 and Band 42 was approved [1].  TR number for this work item was assigned as TR36.715-04-04 and ver 0.0.1 of draft TR skeleton was approved in RAN4#84-bis [2].  This contribution intends to capture UE RF requirements for UL CA more than 2CCs into TR.

Proposal

It is proposed to approve TP described in section 4.

Reference

- [1]	RP-172030, “New WID on LTE Advanced Inter-band Carrier Aggregation (4DL/4UL) of Band 41C and Band 42C”, KDDI, et al.

- [2] R4-1710274, “Draft skeleton: TR36.715-04-04”, KDDI

TP for TR36.715-04-04

<Start of TP>

6.3	UE characteristic

## 6.3.1	Co-existence studies

As listed in sub-clause 5.1, the scope of this technical report is as following three combinations;

- 2 contiguous CCs in Band 41 + 2 contiguous CCs in Band 42 (CA_41C-42C)

- 1 CC in Band 41 + 2 contiguous CCs in Band 42 (CA_41A-42C)

- 2 contiguous CCs in Band 41 + 1 CC in Band 42 (CA_41C-42C)

Table 6.3.1-1 lists co-existence studies of inter-band up to 4 uplink carrier aggregation with Band 41 and Band 42.  It should be noted that co-existence studies for 3 uplink carrier aggregation combinations are omitted in this technical report because difference between three component carriers and four component carriers is minimal.  Only difference could be found in some of 3rd IMD components, which are calculated by equations of “(fx_low – max BW fy) - (fx_high + max BW fy)” and “(fy_low – max BW fx) - (fy_high – max BW fx)”.  Even if results themselves are different between 3UL and 4UL, 3 components’ results are comprehensive in studies of 4 UL.  Therefore, there is no need to capture co-existence studies with 3 component carriers in this technical report.

# Table 6.3.1-1: Band 41 and Band 42 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |  | 3400 | 3600 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 4992 – 5380 |  |  | 6800 – 7200 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 – 8070 |  |  | 10200 – 10800 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 710 – 1104 |  |  | 5896 – 6290 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1392 – 1980 |  |  | 4110 – 4704 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8392 – 8980 |  |  | 9296 – 9890 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2456 – 2730 |  |  | 3360 – 3640 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3888 – 4670 |  |  | 7510 – 8304 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1420 – 2208 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10888 – 11670 |  |  | 12696 – 13490 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11792 – 12580 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10904 – 11910 |  |  | 6384 – 7360 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4820 – 5808 |  |  | 288 – 1270 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 – 17090 |  |  | 13384 – 14360 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 – 16180 |  |  | 14288 – 15270 |  |  |

Based on Table above, 3rd order IMD may fall into own Rx of band 41 and band 42 but as long as self-desense is concerned, there would be no problem since features in this technical report assumes non simultaneous Rx/Tx, i.e. Rx is not operated while Tx is active.

In addition, there are many possible fall-down into UE receive frequencies as below:

- 2nd harmonics: band 46,

- 2nd IMD: bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 46, 67 and 68,

- 3rd IMD: bands 2, 3, 7, 9, 11, 21, 22, 24, 25, 30, 32, 33, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45, 48, 50, 51, 69, 71, 72, 74, 75 and 76,

- 4th IMD: bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 48, 50, 51, 65, 66 and 70, 74, 75 and 76,

- 5th IMD: bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 67, 68, 71, and 72.

When more than 2UL inter-band carrier aggregation UE in Band 41 and Band 42 is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.3.1-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

# Table 6.3.1-2: more than 2UL B41+B42 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3, IMD4 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3, IMD4 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3, IMD4 |
| GPS | 1563 | - | 1587 | YES |  | IMD3, IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | IMD3 |
|  | 2400 | - | 2494 | YES | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 2nd harmonic, IMD2, IMD5 |
|  | 5150 | - | 5350 | YES | Europe | 2nd harmonic, IMD5 |
|  | 5470 | - | 5725 | YES |  | IMD5 |
|  | 5150 | - | 5825 | YES | Asia | 2nd harmonic, IMD5 |

## 6.3.2	∆TIB and ∆RIB values

Based on working assumption on UE perspective in sub-clause 5.3, same UE architecture which supports dual uplink carrier aggregation in Band 41 and Band 42 is applicable for features described in sub-clause 5.1.  Hence, completely same values for ∆TIB and ∆RIB shall apply for CA_41A-42C, CA_41C-42A and CA_41C-42C as well.

# Table 6.3.2-1: TIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_41A-42C | 41 | 0.3 |
|  | 42 | 0.8 |
| CA_41C-42A | 41 | 0.3 |
|  | 42 | 0.8 |
| CA_41C-42C | 41 | 0.3 |
|  | 42 | 0.8 |
| Note: Only applicable for UE supporting inter-band carrier aggregation with uplink in two E-UTRA bands and without simultaneous Rx/Tx. |  |  |

# Table 6.3.2-2: RIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_41A-42C | 41 | 0 |
|  | 42 | 0.5 |
| CA_41C-42A | 41 | 0 |
|  | 42 | 0.5 |
| CA_41C-42C | 41 | 0 |
|  | 42 | 0.5 |
| Note: Only applicable for UE supporting inter-band carrier aggregation with uplink in two E-UTRA bands and without simultaneous Rx/Tx. |  |  |

## 6.3.3	Maximum Sensitivity Degradation (MSD)

As a general working assumption for features in this technical report, sub-clause 5.1 describes that synchronized operation between Band 41 and Band 42 shall be assumed.  Since both bands are operating bands with unpaired spectra, degradation caused by inter-modulation or harmonic distortions never occurs.  Hence, there is no need to specify MSD requirements for CA_41A-42C, CA_41C-42A or CA_41C-42C.

## 6.3.4	Spurious emission band UE co-existence for CA_41A-42C, CA_41C-42A or CA_41C-42C

This clause summarizes the requirements for the carrier aggregation configurations in this technical report for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

For inter-band carrier aggregation with the uplink assigned to E-UTRA bands 41 and 42 with 3 or 4 component carriers, the requirements in Table 6.3.4-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two E-UTRA bands 41 and 42 the requirements in Table 6.3.4-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.6.3.2A-0 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.3.4-1: Requirements for uplink inter-band carrier aggregation in Band 41 and Band 42 (two bands with three or four carriers)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_41A-42C | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 3 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 2, 3 |
| CA_41C-42A | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 3 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 2, 3 |
| CA_41C-42C | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 3 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 2, 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 in TS36.101. NOTE 2: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 3: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. |  |  |  |  |  |  |  |

<End of TP>