3GPP TSG-RAN WG4 NR #85	R4-1712397

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda item:			9.7.11.1

Source:	MediaTek Inc.

Title:	Discussion on RSRP Measurement requirement of NR

Document for:			Discussion

1	Introduction

In [1], it was agreed to conduct link-level simulation studies for measurements. Besides, the simulation assumptions for SS block measurements were agreed [2]. In this contribution, the multi-cell RSRP accuracy for both FR1 and FR2 are provided. The required sample number and measurement period are also discussed.

2	RSRP simulation assumptions

The simulation assumptions are given in Table 1:

| Parameters | Cell 1 | Cell 2 |
| --- | --- | --- |
| Carrier frequency | 4 GHz /30 GHz | 4 GHz /30 GHz |
| System bandwidth | 20 MHz | 20 MHz |
| Subcarrier spacing | 15kHz/30kHz/120kHz/240kHz | 15kHz/30kHz/120kHz/240kHz |
| SS burst periodicity | 5ms | 5ms |
| Number of SS blocks per SS burst set, K | 1 | 1 |
| L3 filtering | Disabled | N/A |
| SNR | 1dB | 6dB |
| CP length | Normal | Normal |
| Channel type | Static EPA 5Hz, ETU 30Hz, ETU 70Hz, TDL-A 87Hz, TDL-B 87Hz, TDL-C 87Hz. | Static EPA 5Hz, ETU 30Hz, ETU 70Hz, TDL-A 87Hz, TDL-B 87Hz, TDL-C 87Hz. |

Table 1: RSRP simulation assumptions.

3	RSRP simulation results

In this section, we compare the measurement accuracy of NR-SSS based RSRP on different sub-carrier spacing (SCS), sample number (N) and channel type, with detail setting parameters shown in Table 1. Here, Table 2 and Table 3 summarizes the absolute accuracies and values of 5%, 50%,95% points on the CDF curves of Delta RSRP in FR1 and FR2, respectively. The CDF curves of SCS 15kHz, SCS 30kHz, SCS 120kHz, and SCS 240kHz are compared in Figure 1 to Figure 4.

Table 2. NR-SSS based RSRP measurement accuracy in FR1

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  |  | SCS = 30 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |
| AWGN | 1 | -0.6588 | 0.4840 | 1.4990 | 1.2984 | -0.6588 | 0.4840 | 1.4990 | 1.2984 |
|  | 3 | -0.4475 | 0.3545 | 1.0230 | 0.9163 | -0.4475 | 0.3545 | 1.0230 | 0.9163 |
|  | 5 | -0.3995 | 0.2997 | 0.8949 | 0.7714 | -0.3995 | 0.2997 | 0.8949 | 0.7714 |
| EPA5 | 1 | -1.5605 | 0.0223 | 1.6321 | 1.6195 | -1.5105 | 0.0173 | 1.2965 | 1.3955 |
|  | 3 | -1.0027 | 0.0159 | 1.1447 | 1.0558 | -0.9542 | 0.0001 | 0.8257 | 0.8956 |
|  | 5 | -0.8152 | -0.0028 | 0.9577 | 0.8847 | -0.7934 | -0.0110 | 0.6836 | 0.7327 |
| ETU30 | 1 | -1.4903 | -0.0389 | 1.1745 | 1.3550 | N/A | N/A | N/A | N/A |
|  | 3 | -1.0113 | -0.0954 | 0.7692 | 0.8726 | N/A | N/A | N/A | N/A |
|  | 5 | -0.8687 | -0.0781 | 0.6274 | 0.7820 | N/A | N/A | N/A | N/A |
| ETU70 | 1 | -1.5649 | -0.0596 | 1.1420 | 1.3915 | N/A | N/A | N/A | N/A |
|  | 3 | -1.0935 | -0.0584 | 0.7311 | 0.9398 | N/A | N/A | N/A | N/A |
|  | 5 | -0.9187 | -0.0662 | 0.6047 | 0.7912 | N/A | N/A | N/A | N/A |

Figure 1(a)                                  Figure 1(b)

Figure 1(c)                                  Figure 1(d)

Figure 2(a)                                  Figure 2(b)

Table 3. NR-SSS based RSRP measurement accuracy in FR2

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  |  | SCS = 240 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |
| AWGN | 1 | -0.6588 | 0.4840 | 1.4990 | 1.2984 | -0.6588 | 0.4840 | 1.4990 | 1.2984 |
|  | 3 | -0.4475 | 0.3545 | 1.0230 | 0.9163 | -0.4475 | 0.3545 | 1.0230 | 0.9163 |
|  | 5 | -0.3995 | 0.2997 | 0.8949 | 0.7714 | -0.3995 | 0.2997 | 0.8949 | 0.7714 |
| TDL-A 87 | 1 | -1.6492 | -0.1513 | 1.2141 | 1.4401 | -1.8643 | -0.3072 | 0.9888 | 1.5146 |
|  | 3 | -1.1914 | -0.1981 | 0.7030 | 0.9951 | -1.4213 | -0.3211 | 0.5099 | 1.1621 |
|  | 5 | -1.0756 | -0.2035 | 0.5682 | 0.8678 | -1.3311 | -0.3438 | 0.4087 | 1.0954 |
| TDL-B 87 | 1 | -1.5541 | 0.0062 | 1.2143 | 1.3682 | -1.6907 | -0.2461 | 0.9608 | 1.3591 |
|  | 3 | -1.0685 | -0.0289 | 0.7944 | 0.9086 | -1.1453 | -0.2848 | 0.5053 | 0.9580 |
|  | 5 | -0.8998 | -0.0322 | 0.6784 | 0.7725 | -1.0512 | -0.3085 | 0.3678 | 0.8718 |
| TDL-C 87 | 1 | -1.5590 | -0.0212 | 1.1915 | 1.3774 | -1.8057 | -0.2335 | 1.1148 | 1.5086 |
|  | 3 | -1.1054 | -0.0469 | 0.7320 | 0.8810 | -1.3294 | -0.2934 | 0.6417 | 1.1242 |
|  | 5 | -0.9942 | -0.0445 | 0.6074 | 0.8075 | -1.2105 | -0.3115 | 0.5265 | 0.9930 |

Figure 3(a)                                  Figure 3(b)

Figure 3(c)                                  Figure 3(d)

Figure 4(a)                                  Figure 4(b)

Figure 4(c)                                  Figure 4(d)

In Table 2, considering that in SCS 30kHz, the delay profile length of ETU channel exceeds the normal CP length 2.34μs, so results in ETU channel are omitted. According to the summarized results in Table 2 and Table 3, we have following observation: in SCS = 240 kHz, channel type TDL-A, the absolute accuracy of 5 samples is 1.0954. Therefore, 5 samples are required to achieve absolute accuracy around 1dB

Observation 1: In some cases, 5 samples are required to achieve absolute accuracy around 1dB.

4	Discussion on Required Sample Number

In this section, the required sample numbers for intra-frequency and inter-frequency measurement are discussed. Our simulation results only model the base band system, but do not consider the effect of DFE chain, and the inter carrier interference due to the new PBCH format (working assumption in last RAN1 meeting) shown in Figure 5 are not considered.

Figure 5 PCBH format

In order to leave enough margin, we suggest that sample number for intra-frequency measurement without gap should be 5.

Proposal 1: RSRP intra-frequency measurement without gap requirement is specified based on 5 samples.

In slow fading channel, e.g. Doppler frequency 5Hz, to guarantee independent samples for L3 input (i.e. L1 output), we need at least 1/5 =0.5s = 200ms separation between two consecutive samples. That indicates the minimum requirement of measurement period is 200ms (i.e., TSSB_measurement_period = 200ms). Suppose that the sample number is 5, which also provides the lower-bound of sampling interval for UE is 200/5 = 40ms. Considering that the SS block measurement times is now also dependent SMTC periodicity, without the DRX cycle, the requirement should be specified based on max{SMTC periodicity, 40ms}

Proposal 2: RSRP intra-frequency measurement without gap requirement is specified based on max{SMTC periodicity, 40ms}

The measurement period of intra-frequency measurement without gap in both non-DRX and DRX cases are listed in Table4.

Table 4. Measurement period of intra-frequency measurement without gap

| DRX cycle | T SSB_measurement_period |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | [5] x Note 1 |
| DRX1 < DRX cycle≤ DRX2 | [5] x DRX period |
| … | … |
| … | … |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Similarly, considering that the SS block measurement times in intra-frequency with gap is now also dependent MGRP periodicity. Without the DRX cycle, the requirement should be specified based on max{SMTC period, MGRP, 40ms}

Proposal 3: RSRP intra-frequency measurement with gap requirement is specified based on max{SMTC periodicity, MGRP, 40ms}.

Table 5. Measurement period of intra-frequency measurement without gap

| DRX cycle | T SSB_measurement_period |
| --- | --- |
| No DRX, or DRX cycle < MGRP, SMTC periodicity | [5] x |
| DRX1 < DRX cycle≤ DRX2 | [5] x DRX period |
| … | … |
| … | … |

In order to leave enough margin for AGC gain compensation, we suggest that sample number for inter-frequency measurement should be 6.

Proposal 4: RSRP inter-frequency measurement requirement is specified based on 6 samples.

5	Summary

In this contribution, the simulation results for NR measurement accuracy are presented. It is observed that

Observation 1: In some cases, 5 samples are required to achieve absolute accuracy around 1dB.

And we propose

Proposal 1: RSRP intra-frequency measurement without gap requirement is specified based on 5 samples.

Proposal 2: RSRP intra-frequency measurement without gap requirement is specified based max{SMTC periodicity, 40ms}.

Proposal 3: RSRP intra-frequency measurement with gap requirement is specified based on max{SMTC periodicity, MGRP, 40ms}

Proposal 4: RSRP inter-frequency measurement requirement is specified based on 6 samples.

4	Reference

[1] R4-1704806, NR RRM way forward, Ericsson, May 2017.

[2] R4-1706904, Link-level simulation assumptions for SS block measurements, Ericsson, June 2017.