3GPP TSG-WG RAN4 Meeting #85	R4-1712540

# Reno, Nevada, USA 27th Nov – 1st Dec, 2017

Source: 	Nokia, Nokia Shanghai Bell, AT&T

Title: 					TP to TR 36.715-04-01: CA_4DL_ 14A-30A-66A-66A_1UL_BCS0

Agenda Item: 			8.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_14A-30A-66A-66A in to the TR 36.715-04-01

**************************** Start of changes ********************************************

## 5.X	CA_14A-30A-66A-66A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_14A-30A-66A-66A | 14 |  |  | Yes | Yes |  |  | 60 | 0 |
|  | 30 |  |  | Yes | Yes |  |  |  |  |
|  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 14, Band 30 and Band 66 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 14 | 788 | 798 | 1576 | 1596 | 2364 | 2394 |
| 30 | 2305 | 2315 | 4610 | 4630 | 6915 | 6945 |
| 66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 |

It can be seen from Table 5.X.2-1 that there is no harmonic relation.

### 5.X.3	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band 14, Band 30 and Band 66 for 1UL/4DL CA_14A-30A-66A-66A, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively, as derived from CA_14A-30A-66A.

Table 5.X.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_14A-30A-66A-66A | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |

Table 5.X.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_14A-30A-66A-66A | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |

**************************** End of changes ********************************************