3GPP TSG-RAN WG4 Meeting #85		R4-1712589

Reno, USA, 27th November – 1st December, 2017

Agenda item:	8.19.4

Source:	CATT

Title:	Impact on PSSCH-RSRP measurement accuracy of Rel-14 UEs due to eV2X Transmit Diversity

Document for:	Discussion

Introduction

RAN1 has discussed the transmit diversity scheme for several meetings, and two-port non-transparent diversity was assumed as working assumption. And RAN1 asked RAN4 to give feedback on the aspects described in [1] due to two-port non-transparent transmit diversity.

1. Overall Description: RAN1 has taken the following working assumption: - For designing PSSCH, RAN1 assumes the use of two-port non-transparent transmit diversity - The use of non-transparent transmit diversity is configured. - Details, including diversity scheme, are FFS - Support of transmission and/or reception up to UE capability - Note: It is RAN1 understanding that requirements on capabilities can be set at regional level and are outside 3GPP scope - Send LS to RAN4 to ask their opinion about when non-transparent scheme for transmit diversity is used by Rel-15 UEs: - Impact on Rel-14 UEs of PSSCH-RSRP measurement accuracy - MPR for Rel-15 UEs - Non-transparent Transmit diversity is not used in the following cases: - When communicating with Rel-14 UEs - When there is a high probability of resource collision with Rel-14 UEs Note: Some companies observe that the performance of MMSE-IRC receiver degrades when a non-transparent Transmit diversity scheme is used in interference limited scenarios with a dominant interferer. RAN1 has the following list of candidate two-port non-transparent diversity schemes: - STBC (including half symbol STBC proposal in R1-1705002) - SFBC - PVS in time domain Note: other schemes are not precluded RAN1 would like to get feedback on the following aspects due to two-port non-transparent transmit diversity: - Impact on PSSCH-RSRP measurement accuracy of Rel-14 UEs - MPR for Rel-15 UEs Impact on MMSE MRC receivers and advanced receivers in the a) presence of one interferer (single-port transmission and two-port diversity) b) presence of multiple interferers (single-port transmission and two-port diversity)

In last meeting, RAN4 had some initial discussion on impact on PSSCH-RSRP measurement accuracy of Rel-14 UEs due to two-port non-transparent transmit diversity, however, RAN4 had not any consensus.  In this contribution, we further discuss the impact on PSSCH-RSRP measurement accuracy of Rel-14 UEs.

Discussion

For Rel-14 UEs, the PSSCH-RSRP absolute accuracy is defined as [5] dB under normal condition [2]. The requirement is defined based on one-port transmitted DMRS and AWGN channel. If Rel-14 UEs receive DMRS with two-port non-transparent transmit diversity, the straightforward impact is the measured RSRP power, PSSCH-RSRP signal strength will have 3dB bias due to two-port transmit diversity. So, the side condition of PSSCH-RSRP measurement will be changed. Therefore, the evaluation of PSSCH-RSRP accuracy impact shall be based on the same side condition.

In order to evaluate the impact on PSSCH-RSRP accuracy for Rel-14 UEs due to two-port non-transparent transmit diversity, we performed the related link level simulations. We used the following simulation assumption [3] which was agreed to conduct PSSCH-RSRP simulation in Rel-14 V2X as baseline.

Table 1: Simulation assumptions for PSSCH-RSRP measurement accuracy

| Parameters | Value | Comments |
| --- | --- | --- |
| Frame structure type | 1 | HD |
| Measurement bandwidth for PSSCH-RSRP | 3 resource blocks | Adjacent transmission |
|  | 4 resource blocks | Non-adjacent transmission |
| System bandwidth | 50 resource blocks |  |
| L1 measurement | 1 shot measurement |  |
| L3 filtering | Disabled |  |
| Transmit antenna | 1/2 |  |
| Receive antennas | 2 | Note that the measured value shall not be lower than the corresponding value of any of the individual diversity branches. |
| Propagation conditions | AWGN, EVA |  |
| Doppler Frequency: EVA and EPA | 1530Hz(EVA) |  |
| CP length | Normal |  |
| Carrier frequency | 5.9GHz |  |
| Center Frequency Offset | 0 |  |
| PSSCH_Ec/Iot | 0dB, 5dB, 10dB | AWGN noise |

The simulation results in table 2 imply that compared to one-port DMRS, the degradation in PSSCH-RSRP accuracy measured on two-port DMRS is 1.5dB at SNR = 0dB. And the degradation is decreased and limited with increased input SNR.

In TS.36.133, Intra-frequency PSSCH-RSRP absolute accuracy is defined in following table, the PSSCH-RSRP absolute accuracy is defined as [5] dB under normal condition assuming the RF margin is 1.5dB. With the degradation in PSSCH-RSRP accuracy measured on two-port DMRS, Rel-14 UEs can meet the accuracy requirements defined in table Table 9.10.3.1-1 in TS36.133.

Table 9.10.3.1-1: Intra-frequency PSSCH-RSRP absolute accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 4 | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA V2X operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| [5] | [9.5] | [0]dB | TDD_G | -118 | N/A | -70 |
| [8.5] | [11.5] | [0] dB | TDD_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA V2X operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |  |  |  |  |

Table 2: PSSCH-RSRP accuracy with one-port and two-port DMRS

| Channel model | Scenario | SNR, dB | Delta PSSCH-RSRP, dB |  | Absolute RSRP accuracy, dB |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 5% CDF | 95% CDF |  |
| AWGN | Single port | 0 | 0.545 | 1.534 | 1.534 |
|  |  | 5 | 0.053 | 0.667 | 0.667 |
|  |  | 10 | -0.06 | 0.29 | 0.29 |
|  | Two port | 0 | -0.101 | 1.604 | 1.604 |
|  |  | 5 | -0.345 | 0.691 | 0.691 |
|  |  | 10 | -0.296 | 0.331 | 0.331 |
| EVA-1530Hz | Single port | 0 | 0.394 | 1.984 | 1.984 |
|  |  | 5 | -0.054 | 0.776 | 0.776 |
|  |  | 10 | -0.169 | 0.262 | 0.262 |
|  | Two port | 0 | 1.035 | 3.343 | 3.343 |
|  |  | 5 | 0.246 | 1.429 | 1.429 |
|  |  | 10 | -0.028 | 0.574 | 0.574 |

RSRP accuracy CDF

Observation: Compared to one-port DMRS, the degradation in PSSCH-RSRP accuracy measured on two-port DMRS is 1.5dB at SNR = 0dB, the degradation is decreased and limited with increased input SNR.

Proposal: The impact on Rel-14 UEs PSSCH-RSRP measurement accuracy would be expected to be limited due to introduction of two-port non-transparent transmits diversity schemes in Rel-15.

Conclusion

In this contribution, we discuss and provide the simulation results to evaluate the impact of two-port non-transparent transmit diversity on PSSCH-RSRP accuracy, our observation and proposal is as follows:

Observation: Compared to one-port DMRS, the degradation in PSSCH-RSRP accuracy measured on two-port DMRS is 1.5dB at SNR = 0dB, the degradation is decreased and limited with increased input SNR.

Proposal: The impact on Rel-14 UEs PSSCH-RSRP measurement accuracy would be expected to be limited due to introduction of two-port non-transparent transmits diversity schemes in Rel-15.

Reference

- R1-715308, LS on transmit diversity for PC5, RAN1

- TS36.133

- R4-168695, Simulation assumption of PSSCH-RSRP measurement, LG Electronics, Huawei, CATT, Qualcomm