3GPP TSG-RAN WG4 Meeting #85				R4-1712592

Reno, USA, 27th November – 1st December, 2017

Title:	Further discussion on UE measurement capabilities

Source:	CATT

Agenda Item:	9.7.3.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 had extensive discussion on UE measurement capabilities. The UE SSB based measurement capabilities for number of cells and beams to be monitored should be specified as frequency ranges dependent, e.g. Sub-6GHz and mmWave. And RAN4 did not reach the consensus on how to define the capability on number of cells and beams.

In this contribution, we further discuss the UE measurement capability requirements and provide our consideration and proposals.

- Discussion

2.1    Number of beams to be monitored

In last meeting, RAN4 discussed whether the number of beams should be defined per cell or per frequency layer. From our consideration, the number of beams should be defined on considering UE mobility performance aspect and UE power consumption aspect. Thus, we propose to define capability on number of cells per frequency layer and the total number of beams per frequency layer. For each cell to be monitored, at least one beam should be measured, and the total number of beams per frequency layer should be larger than the multiple of minimum beam numbers per cell and cell numbers. The total number of beams could provide more useful information for UE mobility, e.g. more beams can be monitored for good cell. For sub-6GHz scenario, if UE is required to be capable of performing measurement for at least 8 intra-frequency cells, we propose UE shall be capable of monitoring at least [1] beams per cell and at least total of [20] beams per frequency layer.

Proposal 1: Define capability on number of cells per frequency layer and number of beams per frequency layer.

2.2    Number of cells to be monitored

Generally, the LTE UE shall be capable of performing measurement for at least 8 cells per frequency layer for intra-frequency measurements and at least 4 cells per frequency layer for inter-frequency measurements. The number of cells per frequency layer that UEs should perform measurement depends on the network deployment and SINR level that UE can measure. Compared the deployment assumptions in TR38.802 with the LTE deployment, there are not big difference from network topology, the neighbour cells which need to be monitored for mobility might be similar as legacy LTE. So, we propose to reuse the 8 identified intra-frequency cells and at least 4 inter-frequency cells per inter-frequency for NR UE measurement capability for frequency range sub 6GHz.

For above 6GHz, base on the simulation results from several companies, the number of detectable cells can increase in NR compared to LTE. However, the larger number of cells to be monitored would increase significantly the complexity, measurement delay and power consumption. Take these aspects into account, it is proposed to reuse the same number of monitored cells as defined for sub 6GHz.

Proposal 2: For sub 6GHz, the NR UE shall be capable of performing measurements for 8 indentified-intra-frequency cells and at least 4 inter-frequency cells per frequency layer.

Proposal 3: For mmWave, the NR UE shall be capable of performing measurements for [8] indentified-intra-frequency cells and at least [4] inter-frequency cells per frequency layer.

2.3   Number of frequency layers to be monitored

In WF [1], the NR UE capability of frequency layers is defined separately according whether the NR carrier is added or not. When NR carrier is not added, the Rel-15 LTE UE should be capable of monitoring the a total number of frequency layers comprising of any combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 carriers), cdma2000 1x and HRPD layers. Considering current number of LTE bands and band combinations, the number of frequency layers for LTE IncMon UE could be reused, thus, we propose to Rel-15 LTE UE shall be capable of monitoring at least 8 FDD EUTRA frequency layers and at least 8 TDD EUTRA frequency layers. And the effective total number of frequency layers to be monitored could be 16.

Release 15 LTE UE

The UE shall be capable of monitoring at least per RAT group - Depending on UE capability, [8] TDD E-UTRA inter-frequency carriers, and - Depending on UE capability, [8] FDD E-UTRA inter-frequency carriers, and - Depending on UE capability, [8] NR inter-RAT carriers

The UE shall be capable of monitoring a total of at least [16] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 carriers), cdma2000 1x and HRPD layers

When NR carrier is added, the Rel-15 LTE-NR NSA UE can only be able to monitor E-UTRA FDD, E-UTRA TDD and NR carriers. The number of frequency layers for E-UTRA FDD/TDD inter-frequency carriers and NR inter-RAT carriers could be reused for Rel-15 LTE-NR NSA UE.

Release 15 LTE-NR NSA UE

The UE shall be capable of monitoring at least per RAT group - Depending on UE capability, [7] NR inter-frequency carriers, and - Depending on UE capability, [8] TDD E-UTRA inter-RAT carriers, and - Depending on UE capability, [8] FDD E-UTRA inter-RAT carriers

The UE shall be capable of monitoring a total of at least [16] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD layers.

Proposal 4: The NR UE capability of frequency layers for NSA scenario is defined in tables as follows:

Release 15 LTE UE

The UE shall be capable of monitoring at least per RAT group - Depending on UE capability, [8] TDD E-UTRA inter-frequency carriers, and - Depending on UE capability, [8] FDD E-UTRA inter-frequency carriers, and - Depending on UE capability, [8] NR inter-RAT carriers

The UE shall be capable of monitoring a total of at least [16] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 carriers), cdma2000 1x and HRPD layers

Release 15 LTE-NR NSA UE

The UE shall be capable of monitoring at least per RAT group - Depending on UE capability, [7] NR inter-frequency carriers, and - Depending on UE capability, [8] TDD E-UTRA inter-RAT carriers, and - Depending on UE capability, [8] FDD E-UTRA inter-RAT carriers

The UE shall be capable of monitoring a total of at least [16] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD layers.

- Conclusion

In this paper, we further discuss the NR UE measurement capability requirements and make our proposals as follows:

Proposal 1: Define capability on number of cells per frequency layer and number of beams per frequency layer.

Proposal 2: For sub 6GHz, the NR UE shall be capable of performing measurements for 8 indentified-intra-frequency cells and at least 4 inter-frequency cells per frequency layer.

Proposal 3: For mmWave, the NR UE shall be capable of performing measurements for [8] indentified-intra-frequency cells and at least [4] inter-frequency cells per frequency layer.

Proposal 4: The NR UE capability of frequency layers for NSA scenario is defined in tables as follows:

Release 15 LTE UE

The UE shall be capable of monitoring at least per RAT group - Depending on UE capability, [8] TDD E-UTRA inter-frequency carriers, and - Depending on UE capability, [8] FDD E-UTRA inter-frequency carriers, and - Depending on UE capability, [8] NR inter-RAT carriers

The UE shall be capable of monitoring a total of at least [16] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 carriers), cdma2000 1x and HRPD layers

Release 15 LTE-NR NSA UE

The UE shall be capable of monitoring at least per RAT group - Depending on UE capability, [7] NR inter-frequency carriers, and - Depending on UE capability, [8] TDD E-UTRA inter-RAT carriers, and - Depending on UE capability, [8] FDD E-UTRA inter-RAT carriers

The UE shall be capable of monitoring a total of at least [16] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD layers.

- Reference

[1]		R4-1709909	Way Forward on UE measurement definition and capability, Intel