3GPP TSG-RAN WG4 Meeting #85	R4-1712684

Reno, Nevada, USA, 27 November – 1 December 2017

Agenda Item:	9.3.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Coordinated Urban Macro Simulation Results with Total Fading Correlation at 30GHz for 55dBm EIRP Transportable Stations

Document for:	Discussion

# 1.	Introduction

Requirement for 55dBm EIRP transportable stations was discussed in RAN4#84Bis, and summary of simulation results with the way forward was approved in [1].

Simulation results in coordinated (0% grid shift) urban macro scenario with 0.5 fading correlation at 30GHz for 55dBm EIRP transportable stations were provided in [2]. This contribution provides the coordinated (0% grid shift) urban macro simulation results with total fading correlation (i.e. path loss from aggressor UE to victim BS is equal to that from aggressor UE to aggressor BS) at 30GHz for 55dBm EIRP transportable stations, according to the agreed simulation parameters in [3] as well as those revised in [4].

# 2.	Discussion

The coordinated urban macro simulation results with total fading correlation at 30GHz for 55dBm EIRP (35dBm conducted transmit power) transportable stations are provided in Figures 1 to 5 below. The coordinated urban macro simulation results with total fading correlation at 30GHz for 23dBm conducted transmit power UE are also provided for purpose of comparison. Here the currently agreed 17dB UE ACLR and 24dB BS ACS are used [5].

(a) 23dBm									(b) 35dBm

Figure 1: CDF of UE conducted transmit power

(a) 23dBm									(b) 35dBm

Figure 2: CDF of UL SINR of victim UE

It can be seen from the results in Figures 1 and 2 that around 30% of the 23dBm UE are transmitting with maximum power and cannot meet the UL SINR of 15dB, while the percentage is reduced to around 10% for the 35dBm transportable stations. Therefore, 55dBm EIRP transportable stations can be used effectively to extend the coverage of coordinated urban macro network at 30GHz.

(a) 23dBm									(b) 35dBm

Figure 3: CDF of UL interference-over-thermal-noise of victim BS

It can be seen from the results in Figure 3 that the interference-over-thermal-noise of the victim BS are similar between the two cases (around 5dB at 90%-tile). Note that it has already been agreed that there is no need to specify the mmWave NR BS receiver dynamic range requirement in the RAN4 specifications.

(a) 23dBm									(b) 35dBm

Figure 4: CDF of received blocking signal power of victim BS

It can be seen from the results in Figure 4 that the received blocking signal power of the victim BS are similar between the two cases (around -65dBm at 99%-tile).

(a) 23dBm									(b) 35dBm

Figure 5: CDF of wanted to blocking signal power ratio of victim BS

It can be seen from the results in Figure 5 that the wanted to blocking signal power ratio of the victim BS are similar between the two cases until the high end <1%-tile) where the 35dBm transportable stations increase the wanted to blocking signal power ratio for around 20dB.

The results in Figures 3 to 5 have shown that the 35dBm transportable stations (with the assumed UL power control parameter) would have minor impacts on the required BS dynamic range and in-band blocking performances.

On the other hand, the simulation results on average and 5%-tile throughput loss using different UE ACLR and BS ACS are summarized in Table 1 below. The results with 23dBm UE using the currently agreed UE ACLR and BS ACS are also provided for purpose of comparison.

Table 1: UL throughput loss

|  | 23dBm | 35dBm |
| --- | --- | --- |
| NR UE ACLR (dBc) | 17 | 17 |
| NR BS ACS (dBc) | 24 | 24 |
| Average throughput loss (%) | 0.73 | 0.81 |
| 5%-tile throughput loss (%) | NA | 2.29 |

It can be seen from the results in Table 1 that the 35dBm transportable stations have a bit higher but still sufficiently low average throughput loss using the currently agreed UE ACLR and BS ACS comparing to the 23dBm UE, while 5%-tile throughput can only be achieved with the 35dBm transportable stations, because more than 5% of the 23dBm UE are out-of-coverage in the simulated scenario. Moreover, the 5%-tile throughput loss is sufficiently low using the currently agreed UE ACLR and BS ACS. Therefore, applying the currently agreed UE ACLR and BS ACS will provide sufficient UL coexistence performance for coordinated urban macro network at 30GHz with the 35dBm transportable stations.

# 3.	Conclusion

This contribution has provided the coordinated urban macro simulation results with total fading correlation at 30GHz for 55dBm EIRP transportable stations, according to the agreed simulation parameters in the way forward as well as those agreed in the revised simulation assumptions.

The simulation results have shown that 55dBm EIRP transportable stations can be used effectively to extend the coverage of coordinated urban macro network at 30GHz, with minor impacts on the required BS dynamic range and in-band blocking performances, while applying the currently agreed UE ACLR and BS ACS will provide sufficient UL coexistence performance.

# References

[1]		R4-1711861, “Summary of simulation results for 55dBm CPE”, Ericsson, Qualcomm, Samsung, Nokia, Verizon.

[2]	R4-1709701, “Urban Macro Simulation Results at 30GHz for 55dBm EIRP Transportable Stations”, Nokia, Nokia Shanghai Bell.

[3]		R4-1708853, “WF on Requirement for 55dBm EIRP Transportable Stations”, Verizon, Ericsson, Qualcomm, Qorvo, Huawei, ZTE, Nokia.

[4]	R4-1709987, “Simulation assumptions of Co-existence study for 55dBm CPE in mmWave spectrum”, Ericsson, Nokia.

[5]	R4-1706063, “WF on mmW ACLR and ACS”, Qualcomm Incorporated, Huawei.