3GPP TSG-RAN4 Meeting #85	R4-1713579

# Reno, Nevada, USA, 27th November – 1st December 2017

Source: 	Qualcomm Incorporated

Title: 	MPR for power class 6 CAT M1 and M2

Agenda item:	8.21.3

Document for:	Approval

# Introduction

In [1], a 0 dB MPR has been proposed for power class 6 for both CAT M1 and M2. The primary assumption here seems to be that SEM is the main factor in determining MPR. Since SEM is in dBm and max power for power class 6 is 14 dBm, which is heavily backed off, no MPR is needed.

In this paper, we highlight all the non-linearities that determine MPR and make a proposal for MPR for CAT M1 and CAT M2.

# Discussion

In [2], MPR needed due to EUTRA, UTRA1, UTRA 2 and SEM is shown in a plot. A copy of that plot is shown in figure 1 below.

Figure 1: MPR required to meet SEM, UTRA1, UTRA2, EUTRA and spurious emissions for 16QAM 10 MHz system BW and RB start of 1

As can be seen though SEM is a limiting factor when fewer number of RBs are allocated, when the RB assignment increases ACLR determines MPR. Since ACLR is measured in dBc and not dBM unlike that of SEM, if the PA is sized correctly to maximize efficiency then MPR required doesn’t change significantly though Max output power is reduced.

Proposal: For power class 6, maintain the current MPR defined for power class 5 for both CAT M1 and CAT M2. The tables are defined below.

Table 6.2.3E-5: Maximum Power Reduction (MPR) for category M1 for Power Class 6

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | - | - | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | - | - | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | - | - | ≤ 2 |

Table 6.2.3E-6: Maximum Power Reduction (MPR) for category M2 UE for Power Class 6

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | > 16 | > 18 | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | > 16 | > 18 | ≤ 2 |

# Conclusion

In conclusion, we propose that the MPR tables for CAT M1 and M2 for power class 6 be maintained for power class 6 as well.

# References

- R4-1711104, CR for adding new power class for CAT-M, 3GPP TSG-RAN WG4 Meeting #84bis, Dubrovnik, Croatian, 9 – 13 Oct 2017

- R4-1703975, MPR for FeMTC, 3GPP TSG-WG RAN4 Meeting #82-Bis, Spokane, US, Date 3rd –7th April