3GPP TSG-RAN4 WG Meeting #85	R4-1713850
Reno, Nevada, USA 27 November - 1 December, 2017

Agenda item:	9.4.3.1.3

Source:	Apple Inc., Intel Corporation

Title:	Consideration of EIRP spherical coverage requirement

Document for:	Discussion

# 1	Introduction

During the RAN #75 meeting, the study item on New Radio access technology [1] was finalized with its outcome captured in TR38.803 [3]. During RAN #75, the New Radio Work Item was initiated [2]. Along with the agreement to define and test RF requirements for NR mm-wave over the air (OTA), the discussion of UE power class definition has evolved over a number of agreements. Currently, power class definition will focus on peak EIRP, with spherical coverage addressed as a separate topic.

This paper contains observations from the author companies with respect to the impact of form factor on the spherical coverage for mm-wave. Simulation results from 3D EM simulations of 28GHz antenna arrays in two very different phone form factor models are presented. These simulations illustrate some of the challenges of integrating NR into modern phone form factors, and showcase the continued need to factor in integration impacts of modern form factors to any discussion or specification of spherical coverage requirements. Our recommendation is to follow the simulation and measurement work plan suggested by UE and Semiconductor vendors at RAN4 #84 before any consideration of specification of spherical CDF requirements [10] and [11].

Spherical EIRP simulations show that the modern phone form factor designs with a full LCD/OLED display and a metal bezel have significant challenges in spherical coverage.  The impact of this on network performance needs to be factored into deployment models and basestation requirements.

# 2	Discussion

## 2.1	Background

The existing RAN4 agreements on UE power class definition include the NR SI outcome in TR38.803 [3], the WF from RAN4 #82bis [4], the WF from RAN4 #83 [5] and the WF from RAN4 #84 [6].

From RAN4 #83 [5]:

- Max allowed EIRP for all UEs is 43 dBm for regulatory reasons

- Companies are encouraged to provide input

  - For discussed issues mentioned in previous slides and their relevance for power class definition

  - Feasible UE output power values, both TRP and EIRP considering different implementations

- Max allowed EIRP for all UEs is 43 dBm for regulatory reasons

- Companies are encouraged to provide input

  - For discussed issues mentioned in previous slides and their relevance for power class definition

  - Feasible UE output power values, both TRP and EIRP considering different implementations

From RAN4 #84 [6]:

Agreements:

- For NR mm-wave, power class definition and CDF have been discussed and it was agreed to define power class based on EIRP

- Table format for specific budget loss parameters has been defined

  - Table now includes more design details (antenna type, number of arrays)

  - CDF EIRP studies will be provided based on the agreed table format to determine final EIRP requirement

Agreements:

- For NR mm-wave, power class definition and CDF have been discussed and it was agreed to define power class based on EIRP

- Table format for specific budget loss parameters has been defined

  - Table now includes more design details (antenna type, number of arrays)

  - CDF EIRP studies will be provided based on the agreed table format to determine final EIRP requirement

The work plan for spherical coverage agreed to RAN4 #84 is copied here.  The author group encourages all RAN4 participants to follow the plan, resulting in a measurement-based quantification of spherical requirements contained in both [10] and [11].

## 2.2	Discussion

To further the discussions related to spherical coverage for mobile platforms supporting 5GNR FR2, this section contains simulations of two different phone form factor models. The first one has a plastic frame and a conventional LCD/OLED display, which is smaller in area than the front glass. The second phone model, instead, is representative of future mobile phone industrial designs, and has a full display and a metal bezel that present challenges for omnidirectional spherical radiation. It should be noted that both of these models are impractical for implementation of mm-wave radiation. Although they have the general size of a modern phone form factor, they both employ volume for mm-wave antenna arrays that is not currently allocated for that purpose. The benign plastic model is worse in this regard than the modern second model since the plastic model has 6 arrays while the modern model has 2 arrays. Furthermore, no electrical and thermal/mechanical design requirements are factored in.

Model 1

A description of the first model is reported in Fig.1. Four arrays of dipoles are placed around the edges: two vertical polarized (L and R) and two horizontal polarized (T and B). Two arrays of dual polarized patches are placed at the front (FR) and back (BA) of the phone. The FR array is located at the top right corner just below the glass and in between the plastic bezel and the display, while the BA array is at the center of the metal back plate.

The CDFs have been processed.

Figure 1: Description of the plastic phone form factor model with antenna arrays location and relevant dimensions where PEC is perfect electrical conductor.

Model 2

A description of the second model is reported in Fig. 2. Two patch arrays are used in this model, both radiating towards the back since radiation toward the front is severely attenuated by the multiple layers of metal in the display. One of the arrays is placed to top the right (Array B) while the second array is placed on the side of the phone (Array A).  Dipole arrays are severely impacted due to the presence of the metal bezel and thus are not employed.

Figure 2: Description of the second phone form factor model with antenna arrays location and relevant dimensions.

In Fig. 3 the cumulative distribution for the EIRP is shown for model 1 and 2, respectively.  It can be noticed that model 1 and 2 present a drastic difference in performance, highlighting challenges for spherical radiation for the modern UE design having a full OLED/LCD display, and metal bezel.  These results are consistent with those in [9]. However, the CDF performance for Model 2 is degraded additionally due to the additional modeling of the metal bezel which restricts radiation further.

Figure 3: Combined EIRP CDF for model 1 and 2.

As stated earlier, both of these designs are not practical as they employ volume for mm-wave radiation that is currently used for something else (battery, board, sensors, cameras, etc.).

In addition, for spherical coverage, the impact of additional factors including case materials, user hand holds, body blockage, will be significant and subsequently will impact the link.  We suggest that the link requirement be quantified with realistic downlink and uplink network simulations emulating realistic deployment scenarios in combination with realistic UE spherical implementations.

Observation 1: Based on simulation analysis the EIRP spherical coverage CDF for modern phone form factors where there is a full OLED/LCD display and metal bezel, the EIRP spherical coverage CDF is challenged.

Observation 2:  The benefit of multiple arrays is largely dependent on the form factor design. For designs where each array adds significantly new coverage, then each additional array will improve the CDF.  For form factors, where each additional array largely sees the same radiation space the benefit of additional arrays is minimal.

Observation 3: The 5GNR FR2 NSA and SA deployment scenarios should be considered from a link level where we start with the use case definitions and develop the realistic UE and BTS requirements for a successful 5GNR service deployment.

# 3	Conclusions

This paper has shared our views of the EIRP CDF for a modern form factor and compared it to a form factor in which the radiating elements see largely plastic. Results show that when the phone model employs features common to the modern cellular phone, including a full display OLED/LCD and a metal bezel, spherical CDF performance is significantly compromised relative to that of a generally plastic and benign form factor.  It is the author company opinion that setting spherical radiation requirements based on benign form-factor design, will only impede adoption of the technology.  Our suggestion is to work on uplink and downlink network simulations and carrier deployment scenarios to further define the use cases and how to monetize it.

The following observations and conclusions have been made:

Observation 1: Based on simulation analysis the EIRP spherical coverage CDF for modern phone form factors where there is a full OLED/LCD display and metal bezel, the EIRP spherical coverage CDF is challenged.

Observation 2:  The benefit of multiple arrays is largely dependent on the form factor design. For designs where each array adds significantly new coverage, then each additional array will improve the CDF.  For form factors, where each additional array largely sees the same radiation space the benefit of additional arrays is minimal.

Observation 3: The 5GNR FR2 NSA and SA deployment scenarios should be considered from a link level where we start with the use case definitions and develop the realistic UE and BTS requirements for a successful 5GNR service deployment.

# 4	References

- RP-161596, “Study on New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #73, September 2016

- RP-170855, “Work Item on New Radio (NR) Access Technology,” NTT DOCOMO, INC., 3GPP RAN #75, March 2017

- TR 38.803, “Study on New Radio Access Technology RF and co-existence aspects,” Version 2.0.0, 3GPP RAN #75, March 2017

- R4-1704362, “WF on mmW UE power class,” Qualcomm, Ericsson, Sony, Skyworks, 3GPP RAN4 #82bis, April 2017

- R4-1706322, “Way Forward on mmW power class,” Qualcomm, Ericsson, Sony, 3GPP RAN4 #83, May 2017

- R4-1708914, “WF on Power Class and CDF,” NTT DOCOMO, 3GPP RAN #84, August 2017

- R4-1710081, “WF on Power class for mm Wave,” Qualcomm, 3GPP RAN4 NR AH #3, September 2017

- R4-1710102, “RAN4-NR#3 Meeting report,” 3GPP RAN4 NR AH #3, September 2017

- R4-1711306, “EIRP for Spherical Coverage, Samsung, Apple, Intel, 3GPP RAN4 #84, October 2017

- R4-1711609, “ WF power class FR2,” Intel, Apple, Samsung, LGE, Huawei, MediaTek, Xiaomi, vivo,OPPO, Motorola, 3GPP RAN #84, October 2017

- R R4-1711895, “WF on power class in FR2,” ,3GPP RAN #84bis, October 2017