3GPP TSG-RAN WG4 Meeting #85	R4-1714052

Reno, NV, USA, 27th Nov – 1st Dec 2017

Agenda item:	9.4.1.1

Source:	T-Mobile USA

Title:	TP for TS 38.101-1 introduction of band n71 for transmitter characteristics

Document for:	Approval

# 1	Background

NR band n71 was approved and captured in the attachment of the Revised WID on New Radio Access Technology. [1] This document includes a text proposal that is an introduction of n71 to TS 38.101-1 for transmitter characteristics.  The text proposal is in Annex A.

# 2	Proposal

It is proposed that the attached text proposal is included in TS 38. 101-1.

# References

# [1]   RP-171485 “Revised WID on New Radio Access Technology”, NTT DOCOMO

# Annex A  		Text Proposal for TS 38.101-1

# <Begin Changes>

### 6.2.1 UE maximum output power

<< Unchanged sections omitted >>

Table 6.2.1-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n71 |  |  |  |  | 23 | +2/-2.5 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |  |  |  |  |

<< Unchanged sections omitted >>

### 6.2.3	UE additional maximum output power reduction

<< Unchanged sections omitted >>

Table 6.2.3-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) | Value of additionalSpectrumEmission |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 |  |  |  |  | N/A | 1 |
| NS_02 | 6.5.2.1.2 | NOTE 1 |  |  | TBD | 2 |
| [NS_35] | 6.5.2.1.2.x | n71 | 5, 10, 15, 20 | N/A |  | [35] |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed |  |  |  |  |  |  |

<< Unchanged sections omitted >>

##### 6.5.2.1.2.x	Minimum requirement (network signalled value "[NS_35]")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "[NS_35]" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.1.x-1.

Table 6.5.2.1.x-1: Additional requirements

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<< Unchanged sections omitted >>

#### 6.5.3.2	Spurious emissions for UE co-existence

<< Unchanged sections omitted >>

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
|  |  | FDL_low | - | FDL_high | -50 | 1 |  |
| n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66, 71, n71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71, n71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| Note 2 : As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and [Table x.x.x.x-x from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

<End Changes>