3GPP TSG RAN WG4 Meeting #85	R4-1714293

Reno, US, 27 November - 1 December 2017

3GPP TSG RAN WG1 Meeting #91			R1-1721560

Reno, NV, USA, November 26th – December 1st 2017

Title:	LS on NR TDD UL/DL configurations and support of HPUE

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG1

To:	TSG RAN WG4

Cc:	TSG RAN WG2

Contact Person:

#### Name:	Yosuke Akimoto

E-mail Address:	yosuke.akimoto at g.softbank.co.jp

#### Name:	George Cummings

E-mail Address:	george.cummings at sprint.com

1. Overall Description:

So far RAN1 has agreed the mechanisms regarding NR TDD UL/DL configurations as in Annex.

RAN1 understanding is that the discussion on HPUE for Rel-15 NR is ongoing in RAN4. Meanwhile, due to the presence of flexible slot(s) in cell-specific configuration, which can be overwritten to DL and/or UL by UE-specific and/or dynamic signaling, the same mechanism as LTE would not be applicable when a UE determines the actual maximum transmit power. RAN1 is wondering whether or not the RAN1 agreements in Annex have already offered sufficient tools to define an appropriate HPUE behavior specified in RAN4.

2. Actions:

ACTION: 	RAN1 respectfully asks RAN4 to take this information into account when specifying a UE behavior regarding HPUE, and send a feedback to RAN1 whether or not RAN1 needs to define additional functionality. Furthermore, RAN1 respectfully asks RAN4 to provide information whether the potential new RAN1 functionality needs to be introduced in Rel-15 December release, or can be June release.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting AH-1801 	22 - 26 January 2018 			Vancouver, CA

TSG RAN WG1 Meeting #92	26 February - 2 March 2018  		Athens, GR

# Annex

RAN1 #88bis:

Agreements:

- From UE signaling perspective,

  - The higher layer signalling for the semi-static assignment of DL/UL transmission direction for NR can achieve at least the followings

    - A periodicity where the configuration applies;

      - FFS: Detailed periodicity set;

      - FFS: how to achieve the signaling of periodicity

    - A subset of resources with fixed DL transmission;

      - FFS: The subset of resources can be assigned in granularity of slot and/or symbol;

    - A subset of resources with fixed UL transmission;

      - Resources with fixed UL transmission happens in the ending part of the periodicity is supported;

      - FFS: The subset of resources can be assigned in granularity of slot and/or symbol;

  - FFS: Other resources not indicated as “fixed UL” or “fixed DL” or “reserved/blank” can be considered as “flexible resource”, where transmission direction can be changed dynamically.

Agreements:

- Strive for unified design regardless of whether the DL/UL resource partition is dynamic or semi-static

- UE behaviors at least the following are common regardless of whether the DL/UL resource partition is dynamic or semi-static:

  - Scheduling timing between control to the scheduled data

  - HARQ-ACK feedback including timing

- Strive for a limited number of semi-static DL/UL resource partition.

- NR may include tools motivated by either dynamic or semi-static.

- FFS: UE behavior if there is a conflict between dynamic and semi-static signaling.

RAN1 #89:

Agreements:

- The SFI transmitted in a group-common PDCCH can indicate the slot format related information for one or more slots

  - The slot format related information informs the UEs of the number of slots and the slot format(s) related information of those slots

  - FFS: how to interpret the SFI when the UE is configured with multiple bandwidth parts

  - FFS: details for UE behaviour

- FFS: A UE may be configured to monitor for at most one group-common PDCCH

Agreements:

- Regarding to the periodicity that included in the higher layer signalling for the semi-static assignment of DL/UL transmission direction for NR, at least the following values are supported:

  - [Roughly 0.125ms, roughly 0.25ms,] 0.5ms, 1ms, 2ms, 5ms, 10ms;

    - Each periodicity is supported for particular SCS(s)/slot duration(s)

  - FFS: details

Agreements:

- In ‘Slot format related information’, ‘other’ is at least:

  - ‘Unknown’

    - UE shall not assume anything for the symbol with ‘Unknown’ by this information

    - FFS: UE behavior when the UE receives the information for the symbol from SFI and broadcast DCI and/or UE-specific DCI and/or semi-static signaling/configuration

  - FFS: ‘Empty’

    - UEs can use this resource for interference measurement

    - UE may assume there is no transmission

NR Adhoc #2:

Agreements:

- In ‘Slot format related information’, ‘Empty’ is not indicated explicitly.

- Note: RAN1 specification ensures that UE(s) is/are aware of which resources can be for ‘gap for DL-UL switching’ and/or ‘gap’

- Note: RAN1 specification ensures that UE(s) is/are aware of which resources are for ‘CSI/interference measurement’.

Agreements:

- UE is configured with a CORESET to monitor group-common PDCCH.

  - When configured, the group-common PDCCH follows the same CORESET configuration (e.g., REG-to-CCE mapping) of the CORESET.

    - A group-common PDCCH is formed by an integer number of CCEs.

- The CORESET for the monitored group-common PDCCH carrying SFI can be the same or different from the CORESET for the monitored PDCCH for other types of control signalling.

Agreements:

- Prioritize discussion of SFI functionality of a group-common PDCCH.

  - Further work will be on group-common PDCCH carrying the SFI at least in August meeting.

RAN1 #90:

Working assumption: (Confirmed in NR Adhoc #3)

- ‘Unknown’ resource is ‘flexible’ and can be overridden by at least by DCI indication; ‘Unknown’ is used to achieve the (FFS: exactly/approximately) the same as ‘Reserved’ if not overridden.

  - ‘Unknown’ is signalled at least by SFI in a group-common PDCCH

  - FFS: Possibility of overridden by some types of RRC (e.g., measurement configuration)

- ‘Reserved’ resource is ‘not transmit’ and ‘not receive’ but cannot be overridden by DCI/SFI indication.

  - ‘Reserved’ is signalled at least by RRC

- FFS: handling of ‘gap’

- For semi-static DL/UL transmission direction, ‘Unknown’ can be informed as part of the semi-static configuration.

NR Adhoc #3

Agreements:

- Regarding dynamic SFI content definition

  - The SFI carries an index to a table that is UE-specifically configured via RRC

    - FFS how to manage the table for future proof

    - FFS how to define entries in the table

  - FFS whether to have separate or joint management of slot based SFI (SFI indicates the slot format of the corresponding slot) vs. multi-slot SFI (SFI indicates the slot format of more than one corresponding slots)

Agreements:

- Confirm the following WA

  - ‘Unknown’ resource is ‘flexible’ and can be overridden by at least by DCI indication; ‘Unknown’ is used to achieve the (FFS: exactly/approximately) the same as ‘Reserved’ if not overridden.

    - ‘Unknown’ is signalled at least by SFI in a group-common PDCCH

    - FFS: Possibility of overridden by some types of RRC (e.g., measurement configuration)

  - ‘Reserved’ resource is ‘not transmit’ and ‘not receive’ but cannot be overridden by DCI/SFI indication.

    - ‘Reserved’ is signalled at least by RRC

  - FFS: handling of ‘gap’

  - For semi-static DL/UL transmission direction, ‘Unknown’ can be informed as part of the semi-static configuration.

Agreements:

- For semi-static DL/UL assignment

  - Cell-specific RRC configuration (SIB) + additionally UE-specific RRC configuration

    - UE-specific RRC configuration only overwrites the “unknown” state of the cell-specific RRC configuration

RAN1 #90bis

Agreements:

- For GC-PDCCH monitoring, confirm the working assumption

  - UE can be configured to monitor SFI in group common PDCCH for a Scell on a different cell

Agreements:

- For cross cell GC-PDCCH monitoring, support by RRC configuration for a UE the following:

  - The same SFI can be applicable to more than one cell

  - Different SFI fields in one GC-PDCCH can be applied to different cells

  - FFS interaction with multiple BWP configuration per cell

Agreements:

The UE is not expected to have conflict on link (DL or UL) direction between that of dynamic SFI and that of UE specific data (UE specific DCI triggered PDSCH, PUSCH (grant-based), and PUCCH with A/N for a PDSCH) in Rel-15

  - Note: a link direction denoted as “unknown” in dynamic SFI is not deemed as in conflict with DL or UL

Agreements:

- The single slot format table supports up to two D/U switching points per slot

  - Zero switching point: 14 DL symbols, or 14 unknown symbols, or 14 UL symbols.

  - One D/U switching point of all combinations: Start with zero or more DL symbols, end with zero or more UL symbols, and with unknown symbols in between, where there is at least one unknown symbol and one DL or UL symbol.

  - Two D/U switching points within a slot: The first 7 symbols start with zero or more DL symbols, ends with at least one UL symbol at symbol #6 with zero or more unknown symbols in between. The second 7 symbols starts with one or more DL symbols and ends with zero or more UL symbols with zero or more unknown symbols in the middle.

  - Note: This single slot slot format table will be captured in RAN1 spec. In Rel.15, RAN1will specify up to X<[256] entries, but the RRC signaling need to consider future compatibility with more entries and from RAN1 perspective, a total of [256] entries in the RRC signalling is necessary (with only X entries specified in Rel-15 in RAN1)

Agreements:

- gNB configures a per serving cell GC-PDCCH (for dynamic SFI) monitoring periodicity of K slots (based on GC-PDCCH numerology), up to 8 choices

  - K = 1, 2, 5, 10, 20

    - FFS other values

Agreements:

For the UE specific single-slot/multi-slotset SFI table configuration

  - Each entry of the table indicates a sequence of configured single-slot slot formats

    - Note if the sequence length is 1, the entry is a single-slot slot format

    - Note if the sequence length is more than one, the entry is a multi-slot slot format

    - Note that it is possible all the slots in a multi-slot slot-format can have the same slot format

    - Note The entries in the table can be of different length including a mix of single slot SFI and multi-slot SFI

      - The length of each entry in the table is FFS, e.g., multiple of configured GC-PDCCH monitoring period, a fraction of the configuration GC-PDCCH monitoring period, etc.

Agreements:

- GC-PDCCH for dynamic SFI monitoring

  - For same cell GC-PDCCH monitoring: UE is required to monitor at most one GC-PDCCH per spatial QCL per configuration period carrying dynamic SFI in the active BWP in the cell

    - The coreset(s) is located in the first 1/2/3 symbols in a slot

    - Configuration of GC-PDCCH for UE to monitor is FFS especially considering interaction with BWP configuration

    - Note: This is not intended to address the case of multi-TRP which is deprioritized before Dec.

  - When configuring the GC-PDCCH monitoring for dynamic SFI, the gNB will configure the payload length

  - When configuring the GC PDCCH monitoring for dynamic SFI for a serving cell, the gNB will configure the location of the bits used for the dynamic SFI in the payload

Agreements:

- For the cell-specific higher layer signalling on semi-static DL/UL assignment, the transmission indication is in pattern of DL-unknown-UL. The signaling include:

  - For DL resources indication, the signaling include:

    - Number of full DL slot(s) (x1) at the beginning of the period. Values for x1 include {0,1,…, (Number of slots in a UL-DL switching periodicity)}

    - Number of DL symbol(s) follow the full DL slots (x2). Values for x2 include {0,1,…, 13}

  - For UL resource indication, the signaling include:

    - Number of full UL slot(s) (y1) at the end of the period. Values for y1 include {0,1,…, (Number of slots in a UL-DL switching periodicity)}

    - Number of UL symbol(s) (y2) preceeds full UL slots. Values for y2 include {0,1,…, 13}

  - The resource(s) in a period between DL and UL segments are unknown resources.

  - FFS The UE does not receive and not transmit on ‘Unknown’ resources in cell-specific higher layer signalling if not otherwise indicated.

Agreements:

- For the UE-specific higher layer signalling on semi-static DL/UL assignment,

  - The signaling includes the indication as per slot basis, the signalling includes:

    - Number of DL symbol(s) (y3) in the beginning of slot No.x3

      - Values for x3 include {1,…, (Number of slots in a UL-DL switching periodicity)}

      - Values for y3 include {0,1,…,13,14}

    - Number of UL symbol(s) (y4) in end of slot No.x4

      - Values for x4 include {1,…, (Number of slots in a UL-DL switching periodicity)}

      - Values for y4 include {0,1,…,13,14}

    - The resource(s) in a slot without DL/UL indication are unknown resource(s).

  - FFS the UE does not receive and not transmit on ‘Unknown’ resources in UE-specific higher layer signalling if not otherwise indicated.

  - FFS At most single DL/UL switching point exists in a UL-DL switching periodicity.

Agreements:

- GC-PDCCH for SFI is associated with a SFI RNTI by configuration

Agreements:

- For the blind decoding of GC-PDCCH carrying SFI, the GC-PDCCH blind decoding is configured with up to two decoding candidates  with a configured aggregation level in a CSS or group-CSS in a configured corset

Agreements:

On overwriting rules across semi-static DL/UL assignment, dynamic SFI, DCI, etc

- Need to decide the overwriting rules between

  - Semi-static DL/UL assignment (including states DL, UL, unknown)

  - Dynamic SFI (indicated in GC-PDCCH with states DL, UL, and unknown)

  - UE specific data transmission (UE specific DCI triggered PDSCH, PUSCH, and PUCCH with A/N for a PDSCH)

    - Also include DCI triggered aperiodic measurement related signals, such as aperiodic CSI-RS, aperiodic SRS, etc

    - FFS: Broadcast transmission, sync, PRACH, RAR, UL data transmission without UL grant, …

  - Measurement: Measurement related signals semi-statically configured by UE-specific RRC (eg. periodic/semi-persistent CSI-RS for CSI report, periodic CSI report, periodic/semi-persistent SRS) where a DL or UL direction will be assumed

    - FFS: CSI-RS for RRM, TRS, etc

  - FFS: Other signals, measurements, and monitoring (including configured coreset monitoring)

- Consider the following directions of potential overwriting in Rel. 15:

  - States from semi-static DL/UL assignment overwritten by measurement, dynamic SFI, or UE specific data

  - State from measurement overwritten by dynamic SFI or UE specific data

  - Dynamic SFI overwritten by UE specific data

  - FFS: UE-specific data and measurement related signals not semi-statically configured by RRC overwritten by “unknown” in dynamic SFI

    - The timing requirement for overwriting

    - UE behavior will be the cancellation of the measurement/data reception or measurement/data related transmission

- For the states from semi-static DL/UL assignment

  - “Unknown” in semi-static DL/UL assignment can be overwritten by measurement, dynamic SFI, and UE specific data

  - DL/UL in semi-static DL/UL assignment cannot be overwritten to the other direction (DL to UL or UL to DL) by measurement, dynamic SFI and UE specific data

  - DL/UL in semi-static DL/UL assignment cannot be overwritten by “unknown” by dynamic SFI

- For the states from measurement in symbols not under DL/UL from semi-static DL/UL assignment:

  - DL/UL direction implied by measurement can be overwritten by unknown in dynamic SFI

    - UE behavior will be the cancellation of the measurement or measurement related transmission

  - DL/UL direction implied by measurement can be overwritten by UL/DL from dynamic SFI

    - UE behavior will be the cancellation of the measurement or measurement related transmission

  - DL/UL direction implied by measurement can be overwritten by UE’s own UE-specific data if the UE specific data imples the other direction

    - UE behavior will be the cancellation of the measurement or measurement related transmission

    - UE will follow the DCI for UE-specific data transmission and reception

- For the states in dynamic SFI in symbols not under DL/UL from semi-static DL/UL assignment (Already agreed in other agreements and only include for completeness)

  - UL/DL in dynamic SFI cannot be overwritten by UE specific data

    - UE will treat it as an error case when UE specific data and dynamic SFI imply different transmit directions

  - Unknown in dynamic SFI can be overwritten by UE specific data (change to DL or UL)

    - UE will follow the DCI for UE-specific data transmission and reception