3GPP TSG RAN WG4 Meeting #85	R4-1714324

Reno, US, 27 November - 1 December 2017

3GPP TSG RAN WG1 Meeting 91						         R1-1721241

Reno, USA, November 27 – December 1, 2017

Title:	LS on wake-up signal

Response to:	-

Release:	Rel-15

Work Item:	NB_IOTenh2-Core

Source:	TSG RAN WG1

To:	TSG RAN WG2, TSG RAN WG4

Cc:

Contact Person:

#### Name:	Xiaolei Tie

E-mail Address:	tiexiaolei@hisilicon.com

1. Overall Description:

RAN1 has discussed wake-up signal design in NB-IoT, and has reached the following agreements:

- The [maximum] duration of WUS is configured per NB-IoT carrier

  - FFS: WUS actual transmission duration can be shorter than the configured maximum duration of WUS.

    - Alt 1: The actual WUS duration is transmitted aligning to the start of the configured maximum duration of WUS.

    - Alt 2: The actual WUS duration is transmitted aligning to the end of the configured maximum duration of WUS.

- There is a non-zero gap from the end of configured [maximum] WUS duration to the associated PO

  - FFS: exact value of non-zero-gap

  - FFS if it is fixed in spec or configurable explicitly, or known implicitly from other configured parameters

- The network can enable or disable use of the WUS

  - How UE acquires information on WUS enabling/disabling is up to RAN2 decision

- UE can assume all the REs for transmission of WUS in a given subframe use the same antenna port.

- The UE shall not assume the transmission of WUS in more than X consecutive subframes use same antenna port.

  - FFS: value of X

- WUS sequence is based on ZC-sequence

  - When designing WUS sequence, negative impact on legacy NSSS detection should be avoided.

2. Actions:

To RAN2

RAN1 respectfully ask RAN2 and RAN4 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #92	26 February – 2 March 2018 	             Athens, GR

TSG RAN WG1 Meeting #92bis	16-20 April 2018 	             	CN