3GPP TSG RAN WG4 Meeting #85	R4-1714379

Reno, US, 27 November - 1 December 2017

3GPP TSG RAN WG1 Meeting 91	 		R1-1721606

Reno, USA, November 27th – December 1st, 2017

Title:	LS on RAN1 agreement on UL power sharing for LTE/NR NSA operation

Response to:	   	-

Release:		Rel-15

Work Item:		NR_newRAT-Core

Source:		TSG RAN WG1

To:		TSG RAN WG2, TSG RAN WG4

Cc:		-

Contact Person:

#### Name:	Jeongho Jeon

E-mail Address:	jeongho.jeon@intel.com

Attachments:		-

1. Overall Description:

RAN1 would like to inform the following agreement made in RAN1#91 related to UL power sharing for LTE/NR NSA operation.

Agreement: For LTE/NR NSA operation, - If this UE supports dual UL operation and also supports single UL operation with Case 1 HARQ timing, RRC signaling can configure a UE to operate in one of the following modes: - Dual UL operation - Single UL operation with Case 1 HARQ timing - Single UL operation with Case 2 HARQ timing - For UE supporting single UL operation and with Case 1 HARQ timing if UE does not support power scaling for LTE-NR DC with P_LTE+P_NR>Pcmax, UE shall support the following two operations: - Operation A with Case1: P_LTE + P_NR > Pcmax, in which case the UE assumes that no NR UL transmission takes place in an UL subframe/slot that is designated as LTE UL in the Case 1 reference TDD configuration - Operation B with Case1: P_LTE + P_NR <= Pcmax, in which case NR UL can be scheduled in any UL subframe/slot (while the UE behaviour in case of being simultaneously scheduled on LTE and NR uplinks is not specified) - The operation A vs operation B configuration is implicitly determined based on P_LTE and P_NR - Note that the above agreement does not affect the current status on the optional/mandatory support of power scaling for LTE-NR DC with P_LTE + P_NR > Pcmax - Note that the above agreement can become obsolete if power scaling for LTE-NR DC is mandated to all UEs

2. Actions:

RAN1 would like to kindly ask RAN2 and RAN4 to take the above agreements into account.

Reference:  -

3. Date of Next TSG RAN WG1 Meetings:

TSG-RAN WG1 AH-1801                                                   22nd Jan. – 26th Jan. 2018, Vancouver, CA

TSG-RAN WG1 Meeting #92	                       26th Feb. – 2th Mar. 2018, Athens, GR

TSG-RAN WG1 Meeting #92-Bis	16th Apr. – 20th Apr. 2018, TBD, CN