3GPP TSG-RAN WG4 Meeting 85	    R4-1712246

Reno, Nevada, USA, 27 Nov. – 1 Dec., 2017

Agenda Item:	9.7.11.1

Source: 	Samsung

Title: 	Updated simulation results for SSB based RSRP measurement

Document for:	Discussion

# Introduction

In RAN4 84bis meeting, link level simulation assumptions for SS block measurement were summarized in [1]. In RAN1 90bis, the PBCH bandwidth is shrinked from 24 RBs to 20 RBs, and 8 RBs of PBCH are padded in the SSS symbol [2]. Besides, the PBCH DMRS sequence is also fixed in this meeting.

In this contribution, we update the simulation results of SSB based RSRP measurement, and also make a comparison between the cases with and without DMRS in RSRP calculation.

# Simulation Assumptions

The detailed simulation assumptions are listed in Table A.1, and A.2 in Annex A, with some downselections highlighted by red. In addition, L1 sampling interval is 40 ms.

Figure 2.1 New PBCH format.

PBCH DMRS sequence follows the agreements in [2], and the new PBCH format is shown in Figure 2.1.

The metric is Delta RSRP=Measured RSRP-Ideal RSRP, where Measured RSRP assumes real channel estimation.

# Simulation results

## SCS 15 kHz

Figure 3.1-1~Figure 3.1-6 are the CDF curves of Delta RSRP under AWGN, EPA, ETU, TDL-A, TDL-B, and TDL-C channels, respectively, for SCS 15 kHz. For each channel, only synchronous network is considered since asysnchronous network generates similar results. In each figure, RSRP based on SSS and SSS plus DMRS are shown.

| Figure 3.1-1 AWGN, sync, 15 kHz, -6 dB | Figure 3.1-2 EPA5, sync, 15kHz, -6 dB |
| --- | --- |
| Figure 3.1-3 ETU30, sync, 15kHz, -6 dB | Figure 3.1-4 TDL-A, sync, 15kHz, -6 dB |
| Figure 3.1-5 TDL-B, sync, 15kHz, -6 dB | Figure 3.1-6 TDL-C, sync, 15kHz, -6 dB |

It can be seen from the above figures that the accuracy improves significantly when the number of samples increases from 1 to 3, but the improvement of accuracies from 3 samples to 5 samples is limited. Besides, when PBCH DMRS is used in deriving RSRP, the accuracy improves significantly for 1 sample, however, for 3 or 5 samples, the improvement becomes less. Note that the total number of REs for PBCH DMRS is 144 in a 4 symbol SSB, and that number for SSS is 127, so the achieved RSRP accuracy of 1 sample with both SSS and PBCH DMRS is like that of 2 samples with only SSS.

Table 3.1-1 summarizes the 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions. From the table, it is seen that with only 1 sample, RSRP accuracy is around ±2.5 dB, while with 3 or more samples, RSRP accuracy is to around ±1 dB. To guarantee that the RSRP accuracy is not worse than that of LTE, at least 3 samples should be considered.

Observation 1: To gurantee that the RSRP accuracy is not worse than that of LTE, at least 3 samples are necessary.

Table 3.1-1: Delta RSRP, SINR -6 dB, 15 kHz

| CDF Channel | Samples | SSS |  |  | SSS+DMRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.58 | -0.94 | 0.35 | -0.83 | 0.08 | 0.89 |
|  | 3 | -0.86 | -0.32 | 0.12 | -0.28 | 0.03 | 0.30 |
|  | 5 | -0.51 | -0.19 | 0.07 | -0.17 | 0.02 | 0.18 |
| EPA5 | 1 | -2.45 | -0.03 | 1.64 | -1.53 | -0.12 | 0.66 |
|  | 3 | -0.82 | -0.01 | 0.54 | -0.51 | -0.04 | 0.22 |
|  | 5 | -0.48 | -0.01 | 0.32 | -0.30 | -0.02 | 0.13 |
| ETU30 | 1 | -2.63 | 0.20 | 2.54 | -2.04 | -0.23 | 1.10 |
|  | 3 | -0.87 | 0.06 | 0.85 | -0.68 | -0.07 | 0.37 |
|  | 5 | -0.54 | 0.04 | 0.51 | -0.42 | -0.05 | 0.23 |
| TDL-A | 1 | -2.55 | 0.04 | 1.92 | -1.63 | -0.11 | 0.92 |
|  | 3 | -0.86 | 0.01 | 0.64 | -0.54 | -0.04 | 0.31 |
|  | 5 | -0.51 | 0.01 | 0.38 | -0.33 | -0.02 | 0.18 |
| TDL-B | 1 | -2.65 | 0.02 | 2.00 | -1.68 | -.012 | 0.95 |
|  | 3 | -0.89 | 0.01 | 0.67 | -0.56 | -0.04 | 0.31 |
|  | 5 | -0.54 | 0 | 0.40 | -0.35 | -0.02 | 0.19 |
| TDL-C | 1 | -2.65 | 0 | 1.77 | -1.56 | -0.12 | 0.83 |
|  | 3 | -0.89 | 0 | 0.59 | -0.51 | -0.04 | 0.28 |
|  | 5 | -0.52 | 0 | 0.36 | -0.32 | -0.02 | 0.17 |

## SCS 30 kHz

Table 3.2-1 summarizes the values of 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions with SCS 30 kHz.

Table 3.2-1: Delta RSRP, SINR -6 dB, 30 kHz

| CDF Channel | Samples | SSS |  |  | SSS+DMRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.57 | -0.94 | 0.33 | -0.82 | 0.09 | 0.88 |
|  | 3 | -0.86 | -0.31 | 0.11 | -0.27 | 0.03 | 0.29 |
|  | 5 | -0.52 | -0.19 | 0.06 | -0.17 | 0.02 | 0.18 |
| EPA5 | 1 | -2.56 | 0.04 | 2.15 | -1.70 | -0.10 | 1.04 |
|  | 3 | -0.85 | 0.01 | 0.70 | -0.58 | -0.03 | 0.35 |
|  | 5 | -0.50 | 0.01 | 0.43 | -0.35 | -0.02 | 0.21 |
| ETU30 | 1 | -2.27 | 0.34 | 2.77 | -2.04 | -0.25 | 1.27 |
|  | 3 | -0.77 | 0.11 | 0.93 | -0.69 | -0.08 | 0.43 |
|  | 5 | -0.45 | 0.07 | 0.55 | -0.41 | -0.05 | 0.26 |
| TDL-A | 1 | -2.59 | 0.04 | 2.06 | -1.64 | -0.13 | 1.00 |
|  | 3 | -0.88 | 0.02 | 0.69 | -0.55 | -0.04 | 0.34 |
|  | 5 | -0.53 | 0.01 | 0.41 | -0.33 | -0.03 | 0.20 |
| TDL-B | 1 | -2.61 | 0.04 | 2.08 | -1.68 | -0.11 | 1.01 |
|  | 3 | -0.90 | 0.01 | 0.69 | -0.56 | -0.04 | 0.33 |
|  | 5 | -0.52 | 0.01 | 0.42 | -0.33 | -0.02 | 0.21 |
| TDL-C | 1 | -2.54 | 0.03 | 2.07 | -1.69 | -0.12 | 1.01 |
|  | 3 | -0.85 | 0.01 | 0.69 | -0.55 | -0.04 | 0.33 |
|  | 5 | -0.49 | 0.01 | 0.42 | -0.33 | -0.03 | 0.21 |

## SCS 120 kHz

Table 3.3-1 summarizes the values of 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions with SCS 120 kHz.

Table 3.3-1: Delta RSRP, SINR -6 dB, 120 kHz

| CDF Channel | Samples | SSS |  |  | SSS+DMRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.58 | -0.94 | 0.35 | -0.82 | 0.09 | 0.89 |
|  | 3 | -0.86 | -0.31 | 0.12 | -0.27 | 0.03 | 0.29 |
|  | 5 | -0.52 | -0.19 | 0.07 | -0.17 | 0.02 | 0.18 |
| EPA5 | 1 | -2.68 | 0.15 | 2.47 | -1.95 | -0.19 | 1.13 |
|  | 3 | -0.90 | 0.05 | 0.82 | -0.65 | -0.06 | 0.38 |
|  | 5 | -0.54 | 0.03 | 0.50 | -0.39 | -0.04 | 0.23 |
| TDL-A | 1 | -2.63 | 0.05 | 2.13 | -1.70 | -0.14 | 1.10 |
|  | 3 | -0.88 | 0.02 | 0.71 | -0.56 | -0.04 | 0.37 |
|  | 5 | -0.52 | 0.01 | 0.42 | -0.34 | -0.03 | 0.22 |
| TDL-B | 1 | -2.57 | 0.03 | 1.96 | -1.67 | -0.13 | 1.02 |
|  | 3 | -0.86 | 0.01 | 0.65 | -0.56 | -0.04 | 0.34 |
|  | 5 | -0.52 | 0.01 | 0.39 | -0.33 | -0.02 | 0.20 |
| TDL-C | 1 | -2.49 | 0.03 | 2.21 | -1.68 | -0.11 | 1.08 |
|  | 3 | -0.81 | 0.01 | 0.73 | -0.57 | -0.04 | 0.36 |
|  | 5 | -0.50 | 0.01 | 0.43 | -0.34 | -0.02 | 0.22 |

## SCS 240 kHz

Table 3.4-1 summarizes the values of 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions with SCS 240 kHz.

Table 3.4-1: Delta RSRP, SINR -6 dB, 240 kHz

| CDF Channel | Samples | SSS |  |  | SSS+DMRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.59 | -0.94 | 0.36 | -0.83 | 0.09 | 0.89 |
|  | 3 | -0.87 | -0.31 | 0.12 | -0.28 | 0.03 | 0.30 |
|  | 5 | -0.53 | -0.19 | 0.07 | -0.17 | 0.02 | 0.18 |
| EPA5 | 1 | -2.29 | 0.19 | 2.27 | -1.83 | -0.28 | 1.00 |
|  | 3 | -0.76 | 0.07 | 0.76 | -0.61 | -0.09 | 0.33 |
|  | 5 | -0.46 | 0.03 | 0.46 | -0.37 | -0.06 | 0.21 |
| TDL-A | 1 | -2.50 | -0.03 | 1.74 | -1.64 | -0.16 | 0.82 |
|  | 3 | -0.84 | -0.01 | 0.58 | -0.55 | -0.05 | 0.27 |
|  | 5 | -0.51 | 0 | 0.35 | -0.34 | -0.03 | 0.17 |
| TDL-B | 1 | -2.62 | 0.04 | 2.17 | -1.75 | -0.15 | 1.14 |
|  | 3 | -0.87 | 0.01 | 0.73 | -0.59 | -0.05 | 0.38 |
|  | 5 | -0.53 | 0.01 | 0.44 | -0.35 | -0.03 | 0.23 |
| TDL-C | 1 | -2.61 | 0.04 | 2.19 | -1.76 | -0.14 | 1.11 |
|  | 3 | -0.87 | 0.01 | 0.73 | -0.58 | -0.05 | 0.37 |
|  | 5 | -0.54 | 0.01 | 0.45 | -0.36 | -0.03 | 0.22 |

## Summary across different SCSs

Based on previous simulation results, the maximum absolute value of 5% and 95% points on the CDF curve of Delta RSRP across different SCSs is summarized in Table 3.5-1.

Table 3.5-1 Maximum absolute value of Delta RSRP (5%~95% on CDF), SINR -6 dB

| RS type | SSS | SSS+DMRS |
| --- | --- | --- |
| 1 sample | 2.68 | 2.04 |
| 3 sample | 0.93 | 0.69 |
| 5 sample | 0.55 | 0.42 |

Comparing the results between different reference signal types, it is observed that PBCH DMRS can bring an improvement on the measurement accuracy by providing additional REs, however, as the number of samples increases to 3 or 5, the improvement becomes less. On the other hand, DMRS involves blind detection of the SSB index, which induces additional complexity and delay, hence, it is suggested to consider only SSS in defining the RSRP requirement and leave the use of DMRS for UE implementation.

Observation 2: PBCH DMRS can bring an improvement on RSRP measurement accuracy, but the improvement becomes less as the number of samples increases.

# Conclusion

In this paper, we provided the SSB based RSRP measurement accuracy simulation results. The results covered different SCSs, channel conditions, network synchronization conditions, and sample numbers. From the simulation results, we obtain the following observations:

Observation 1: To gurantee that the RSRP accuracy is not worse than that of LTE, at least 3 samples are necessary.

Observation 2: PBCH DMRS can bring an improvement on RSRP measurement accuracy, but the improvement becomes less as the number of samples increases.

# Reference

- R4-1711722, Simulation results summary for SS block based RSRP, Ericsson

- Chairman’s notes RAN1 90bis final

# Annex A: Simulation Assumptions

Table A.1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | Case A Note2: 2 rx and 4 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx and 4 rx with rx beamforming Note1 | Case A Note2: 2 rx and 4 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx and 4 rx with rx beamforming Note1 |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 1, 3, 5; other number of samples may also be studied upon a need | 1, 3, 5; other numbers of samples may also be studied upon a need |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set, K | 1 Note2 and 4 | 1 Note2 and 4 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - SS block BW | 127 subcarriers for SS, 240 subcarriers for PBCH | 127 subcarriers for SS, 240 subcarriers for PBCH |
| - Actual SS block transmissions | always transmitted | always transmitted |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results NOTE2: prioritized for RAN4#84 |  |  |

Table A.2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | (,)=(112, 1) | (,)=(224, 1) |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR for Case A | dB | K=1: 6 dB K=4: (2, 6,4, -2) dB | K=1: 1 dB K=4: (2, 1, 1, 1) dB and (-1, 5, -3, 2) dB |
| Es/IoT for Case A (calculated from SNR) | dB | N/A | K=1: -5.97 dB K=4: (-2.12, -5.97, -4.45, -1.12) dB and (-5.12, 1.46, -8.45, -0.12) |
| SNR for Case B, including beamforming gain | dB | K=1: 6 dB K=4: (2, 6,4, -2) dB | K=1: 1 dB K=4: (2, 1, 1, 1) dB and (-1, 5, -3, 2) dB |
| Es/IoT for Case B (calculated from SNR), including beamforming gain | dB | N/A | K=1: -5.97 dB K=4: (-2.12, -5.97, -4.45, -1.12) dB and (-5.12, 1.46, -8.45, -0.12) |
| Propagation conditions | - | AWGN, EPA5, ETU30, ETU70, CDL-A/B/C Note, TDL-A/B/C Note |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |