3GPP TSG RAN WG4 Meeting #85	R4- 1712300

Reno, US, 27 November - 1 December 2017

Agenda item:	9.7.10.2

Title: 	On Cell Identification requirements in NR

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4 AH NR#3 it was agreed to specify 2 sets of requirements for cell identification [1].

RAN4 is to specify two sets of cell identification requirements for SSB based measurement: - Cell identification without SSB index reading: Tidentify_w/o_index = TPSS/SSS_sync + T SSB_measurement_period - Cell identification with SSB index reading: Tidentify_w_index = TPSS/SSS_sync + T SSB_measurement_period + TSSB_time_index where Tindex is the delay allowed for UE to acquire the index of the SSB being measured: for sub 6GHz: TSSB_time_index is total time for PBCH_DMRS detection for above 24GHz: TSSB_time_index is total time for PBCH decoding

In this contribution we present our views on cell identification requirements.

# Discussion

In RAN4#84bis companies submitted simulation results for PSS/SSS detection based on simulation assumptions in [2]. However, the simulation assumption for DMRS time index detection and PBCH decoding in [3] are different from that in [2].  Technically, the cell identification is a procedure including both PSS/SSS detection and SSB time index detection. We believe that the delay requirement from the two detections – PSS/SSS and SSB time index should be derived from the same simulation assumption. So, in our paper [4] we present simulation results for PSS/SSS detection and SSB index detection under the same simulation assumptions.

Proposal #1: The simulation assumption for deriving TSSB_time_index should be the same as that used to derive TPSS/SSS_sync so that they can be added together to reflect the total cell identification delay

Based on the simulation results presented in [4] we have the following observations and proposals for sub-6GHz operation.

Observation #1: For sub-6GHz for 90% successful detection of PSS/SSS is up to 3 SS-Blocks are required at SINR of -6dB.

Taking into account UE implementation margin and the baseline cell identification time in LTE of 600ms as specified in TS36.133, we propose that TPSS/SSS_sync be defined using max{600, 5*SMTC_period}ms. At least 600ms is an acceptable value to the mobility performance from system perspective (which might be similar to LTE) and power consumption from UE perspective.

Proposal #2: Define TPSS/SSS_sync using max{600, 5*SMTC_period}ms for sub-6GHz

Observation #2: For sub-6GHz the minimum number of SS-Blocks for 99% successful detection of SSB Index is 2 at SINR of -6dB.

Considering UE implementation margin we propose that for sub-6GHz TSSB_time_index be defined using 4*SMTC_period as requirement

Proposal #3: Define TSSB_time_index using 4*SMTC_period for sub-6GHz

Based on the RSRP measurement time in [5], 4 SS-Blocks are required for SSS based measurement in NR. In TS 36.133 the measurement time of 200ms is based on sampling interval of 40ms and considering the trade-off between UE power consumption and mobility performance. Hence, for SSS based measurement we may also use 200ms as a baseline for the small SMTC case to avoid huge power assumption at UE. So, we propose that the TSSB_measurement_period be defined using max{200, 4*SMTC period}ms

Proposal #4: Define T SSB_measurement_period using max{200, 4*SMTC_period}ms for sub-6GHz

# Conclusion

Based on the discussion presented above, we have the following proposals:

Proposal #1: The simulation assumption for deriving TSSB_time_index should be the same as that used to derive TPSS/SSS_sync so that they can be added together to reflect the total cell identification delay Proposal #2: Define TPSS/SSS_sync using max{600, 5*SMTC_period}ms for sub-6GHz Proposal #3: Define TSSB_time_index using 4*SMTC_period for sub-6GHz Proposal #4: Define T SSB_measurement_period using max{200, 4*SMTC_period}ms for sub-6GHz

# References

- R4-1709885, “Way forward on cell identification”. Huawei, HiSilicon, NTT DOCOMO, MediaTek, Samsung.

- R4-1708698, “Updated link level simulation assumption of cell detection in NR”, Huawei, HiSilicon, NTT DOCOMO, INC.

- R4-1709886, “Updated PBCH Simulation assumption”, Huawei, HiSilicon

- R4- 1712298, “Simulation results for Cell Detection in NR”, Intel Corporation

- R4-1712296, “SS block RSRP link level simulation result update”, Intel Corporation