3GPP TSG-RAN WG4 meeting #85	R4-1712390
Reno, NV, USA, 27 Nov – 1 Dec, 2017

Source:	T-Mobile USA

Title:	TP to 38.101-1: N71 specific changes to section 5

Agenda item:	9.4.1.1

Document for:	Approval

## Background

This contribution contains the N71 specific changes for 38.101-1 section 5.  The N71 changes in this section mirror the band 71 requirements found in TS36.101.

Transmission bandwidth in section 5.3 was divided between TDD and FDD bands because there are FDD bands where the UL and DL frequencies are closely spaced.  In this case the IMD’s from the larger channel BW’s impact the REFSENS values.  The increased utilization makes the REFSENS values for these bands worse than E-UTRA.  With the exception of band 71 the values in Table 5.3-1 and 5.3-1 are taken from [2].

## 2. Proposal

It is proposed that the attached text proposal is included in TS 38.101-1.

## References

- RP-172115 Revision of WI: New Radio Access Technology

- R4-1709075, WF on NR spectrum utilization

TEXT PROPOSAL:

< start of text proposal >

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 450 MHz – 6000 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The TS 38.101-1 specification covers FR1 operating bands shown in Table 5.1-1.

## 5.2	Operating bands

New Radio is designed to operate in the operating bands defined in Table 5.2-1

Table 5.2-1 NR operating bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

## 5.3	Channel bandwidth

Requirements in present document are specified for the channel bandwidths listed in Table 5.3-1 and 5.3-2.

Table 5.3-1: Transmission bandwidth configuration NRB in NR channel bandwidths for TDD bands

| Channel bandwidth BWChannel [MHz] | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission bandwidth configuration NRB for 15 KHz SCS | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N/A | N/A | N/A |
| Transmission bandwidth configuration NRB for 30 KHz SCS | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
| Transmission bandwidth configuration NRB for 60 KHz SCS | N/A | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |

Table 5.3-2: Transmission bandwidth configuration NRB in NR channel bandwidths for FDD bands

| NR FDD Band | Channel bandwidth BWChannel [MHz] | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Transmission bandwidth configuration NRB for 15 KHz SCS | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N/A | N/A | N/A |
| 71 | Transmission bandwidth configuration NRB for 15 KHz SCS | 25 | 52 | 79 | 100 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Transmission bandwidth configuration NRB for 30 KHz SCS | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
|  | Transmission bandwidth configuration NRB for 60 KHz SCS | N/A | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |

### 5.3.1	Channel bandwidths per operating band

The requirements in this specification apply to the combination of channel bandwidths and operating bands shown in Table 5.3.1-1. The transmission bandwidth configuration in Table 5.3-1 and Table 5.3-2 shall be supported for each of the specified channel bandwidths.

Table 5.3.1-1: NR channel bandwidth

| NR Band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 71 | 15 KHz | Yes | Yes1 | Yes1 | Yes1,6 |  |  |  |  |  |  |
| NOTE 1: Refers to the bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (subclause 7.3) is allowed. NOTE 6: For the 20MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 673-678 MHz or 683-688MHz. |  |  |  |  |  |  |  |  |  |  |  |

## 5.4	Channel Arrangement

### 5.4.1	Channel spacing

## The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths.

### 5.4.2	Channel raster

## The channel raster is 100 kHz for all FR1 bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

### 5.4.3	Carrier frequency and EARFCN

The carrier frequency in the uplink and downlink is designated by the New Radio Absolute Radio Frequency Channel Number (NARFCN) in the range 0 – 262143. The relation between NRARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.4.3-1 and NDL is the downlink NARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between NARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.7.3-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

Table 5.4.3-1: E-UTRA channel numbers

| NR Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 71 | 617 | 68586 | 68586 - 68935 | 663 | 133122 | 133122 - 133471 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 5, 10, 15 and 20 MHz respectively. |  |  |  |  |  |  |

### 5.4.4	TX–RX frequency separation

a)	The default NR TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.4.4-1 for the TX and RX channel bandwidths defined in Table 5.3.1-1

Table 5.7.4-1: Default UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 71 | -46 MHz |
| NOTE 1: Default TX-RX carrier centre frequency separation. |  |

< end of text proposal >