3GPP TSG-RAN WG4 Meeting #85	R4-1712687

Reno, Nevada, USA, 27 November – 1 December 2017

Agenda Item:	9.5.2.5.2.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposal on mmWave NR BS Absolute ACLR

Document for:	Approval

# 1.	Introduction

mmWave NR BS absolute ACLR limits were discussed in RAN4#84Bis, and a way forward was agreed [1] to decide on one of the two options in this meeting.

This contribution provides a proposal to specify the mmWave NR BS absolute ACLR limits in the RAN4 specifications per the agreed way forward.

# 2.	Discussion

The two options listed in [1] are:

- Option 1:

  - To adopt -13 dBm/MHz as the absolute ACLR value for Macro BS and [-20 dBm/MHz] for other BS classes.

- Option 2:

  - To adopt -13 dBm/MHz for BS with maximum output power PTx ≥ 35 dBm in the frequency range 24.24 – 33.4 GHz and PTx ≥ 33 dBm in the frequency range 37 – 52.6 GHz;

  - To adopt [-20 dBm/MHz] for BS with maximum output power PTx < 35 dBm in the frequency range 24.24 – 33.4 GHz and PTx < 33 dBm in the frequency range 37 – 52.6 GHz.

It has been agreed in RAN4 [2] that NR BS classes for BS without antenna connectors or TAB connectors are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

Note that the maximum output power for each NR BS class is yet to be decided.

On the other hand, it has been agreed in RAN4 [2] that NR Spectrum Emission Masks are defined based on the same principles as for the WP5D LS response [3], applicable to all BS classes:

-	A fixed mask applicable to the highest BS power levels.

-	Variable masks scaled with power level and applicable to BS with power level below certain thresholds:

- One mask for 24.24 – 33.4 GHz, based on ACLR = 28 dBc, applicable for BS maximum output power < 35 dBm.

- One mask for 37 – 52.6 GHz, based on ACLR = 26 dBc, applicable for BS maximum output power < 33 dBm.

-	Masks are limited to an absolute lowest emission limit of -20 dBm as in the WP5D LS response [3].

It can be seen from the above that if Option 1 is adopted, then the impacts on BS implementation complexity would not be clear until the maximum output power for each NR BS class is decided. On the other hand, adopting Option 2 would better align the specification of absolute ACLR limits with that of the Spectrum Emission Masks. Therefore, it is proposed to adopt Option 2 to specify the mmWave NR BS absolute ACLR limits in the RAN4 specifications.

# 3.	Conclusion and proposal

This contribution has provided a proposal to specify the mmWave NR BS absolute ACLR limits in the RAN4 specifications per the agreed way forward.

Proposal:

To adopt Option 2 to specify the mmWave NR BS absolute ACLR limits in the RAN4 specifications:

  - To adopt -13 dBm/MHz for BS with maximum output power PTx ≥ 35 dBm in the frequency range 24.24 – 33.4 GHz and PTx ≥ 33 dBm in the frequency range 37 – 52.6 GHz;

  - To adopt [-20 dBm/MHz] for BS with maximum output power PTx < 35 dBm in the frequency range 24.24 – 33.4 GHz and PTx < 33 dBm in the frequency range 37 – 52.6 GHz.

# References

[1]	R4-1711927, “WF on absolute ACLR for FR2”, Huawei, Hisilicon, NTT DOCOMO, INC., Ericsson.

[2]	R4-1712124, “TR 38.817-02 v0.4.0: General aspects for BS RF for NR”, Ericsson.

[3]	R4-1700305, "LS on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz", Ericsson, Nokia.