# 3GPP TSG-RAN WG4 Meeting #85                                       R4- 1712747

# Reno, Nevada, 27 Nov - 1 Dec, 2017

Source: 	Qorvo

Title: 	Discussion of UE Architectures for DC_71A-n71

Agenda Item:	9.4.4.2.1

Document for:	Discussion

# Introduction

In RAN-84bis, the DC_71A-n71 combination was first discussed [1]. Additional considerations on LTE-NR uplinks were also raised in the discussion [2]. The way forward was agreed indicating that different RF architectures should be studied for difficult combinations [3]. Depending on the frequency plan chosen, this L-L combination can have low order IM products that could pose a self-desense concern. This submission discusses possible UE RF Front End (RFFE) architectures to support this combination.

# Discussion

## 2.1 Overview

Bands 71 and n71 are the 600 MHz spectrum authorized currently in the United States and to be auctioned next year in Canada.  The spectrum consists of 7 paired 5 MHz blocks with an eleven MHz duplex gap with the blocks designated A thru G.  The band layout is shown in Figure 1 [4]

Figure 1: Band Plan for Band 71 / n71

The potential for a NSA deployment of DC_71A-n71 has been discussed.  This submission looks at the question of self-desense resulting from the mixing of the UL of the blocks above under simultaneous transmit conditions.  In this analysis, we first observe what order of IM products would cause a self-desense problem for various choices of blocks and then consider the performance of two possible UE RFFE architectures.

## 2.2 IM product order and block combination

Block combinations were analysed for potential self-desense issues by finding the frequency location of IM products results from mixing of the UL signals and comparing these locations to the associated DL frequencies.  An example of this procedure for A block (UL 663 – 668 MHz) and B block ( 668 – 673 MHz is shown below in Table 1.  Results for the selected block combinations are shown below in Figure 1 showing the order of the IM product that would cause a self-desense issue and the total bandwidth of the configuration.

Table 1 IMD Product Analysis for A & B block case

Figure 2 Block combinations and relevant IM products

Observation 1: The order of IM product that would cause a self-desense issue is strongly correlated to the BW of the emission.

Observation 2: IM3 components are not a problem for cases with a bandwidth of 20 MHz or less

## 2.3 Analysis assumptions

Key assumptions for the architecture analysis are listed in Table 1 below. In general, our effort here is to show the key differences between the approaches.

Table 2: Assumptions for architecture analysis

| Parameter | Assumption |
| --- | --- |
| Antenna to antenna coupling | 10 dB |
| Diplexer loss | 0.5 dB |
| Switch loss (all) | 0.5 dB |
| Switch OIP3 | >70 dBm |
| Duplexer Passband Rx and Tx loss | 2.5 dB |
| Duplexer TX – RX isolation | 55 dB |
| Duplexer Ant – RX selectivity at TX frequency | 55 dB |
| Duplexer Ant – TX selectivity at RX frequency | 55 dB |
| Loss PA to antenna | 4 dB (Sum of two switch losses, diplexer and duplexer loss) |
| LNA IIP3 | -5 dBm |
| PA gain | 30 dB |
| PA OIP3 | 40 dBm |
| PA OIP5 | 40 dBm |
| NR and LTE infrastructure separation | Collocated |
| Isolation between power amplifiers in dual TX approach | >60 dB |

## 2.4 Single TX Architecture

The single TX approach is shown in Figure 3 with the powers at Band 71 or n71 shown for the assumptions in Table 1. The antenna power is assumed to be +23 dBm with equal NR and LTE powers of +20 dBm. The fundamental power for NR are shown in purple while the ones for LTE are shown in orange.

Fundamental power at the primary receiver LNA input resulting from the PA output power is calculated as:

PA Po – switch loss – duplexer isolation – switch loss = 24 – 0.5 – 55 – 0.5 = -32 dBm

The fundamental power at the secondary receiver LNA input is calculated as:

Antenna Po – antenna coupling – diplexer loss – switch loss – duplexer loss – switch loss

20 – 10 – 0.5 – 0.5 – 55 – 0.5 = -46.5 dBm

Figure 3: Single TX Architecture with powers at fundamental frequency

The IM3 power falling in the receive channel for the fundamental powers shown in Figure 3 are shown in Figure 4 and the IM5 power is shown in Figure 5

Figure 4: Single TX Architecture with IM3 powers

Figure 5: Single TX Architecture with IM5 powers

## 2.5 Dual TX Architecture

The dual TX approach is shown in Figure 6 with the powers at Band 71 or n71 shown for the assumptions in Table 1. The total emission power is assumed to be +23 dBm with +20 dBm NR on one antenna and +20 dBm LTE on the other antenna. The powers of the lower frequency block are shown in blue while those of the upper frequency block are shown in green.

One item to note in this architecture is the need for good isolation between the PA chains. For a 30 dB gain PA, a leakage signal of -24 dBm at the PA input would create a signal equal in strength to the 6 dBm coupled back thru the antenna.  The suggests isolation of 60 dB would be needed between one output and the input of the other PA chain.

The notable difference in the dual TX architecture is unequal levels of the fundamental signals at the PA output versus the equal levels seen in the Single TX case.  The relevant IM3 product is the lower tone that falls into the DL band and its amplitude moves 2 dB for dB of the lower frequency and dB for dB with amplitude of the upper frequency. For example, to calculate the IM3 power for the case with lower frequency power of 24 dBm and upper frequency power of 6 dBm as in the Antenna 1 case below in Figure 6, the calculation would be

OIP3 – 2 * (OIP3 - Lower freq power ) – (OIP3 - upper frequency power)

= 40 – 2*(40-24) – (40 – 6)

= 40 – 32 – 34

= -26 dBm

Figure 6: Dual TX architecture with powers at fundamental frequencies

The IM3 power falling in the DL channel for the fundamental powers shown in Figure 7. The IM5 powers are falling into the DL channel are neglectable.

Figure 7: Dual TX architecture with IM3 powers

The results are summarized in Table 2 below.

Table 3 Summary of Results

|  | Single TX |  | Dual TX |  |
| --- | --- | --- | --- | --- |
|  | Receiver 1 | Receiver 2 | Receiver 1 | Receiver 2 |
| IM3 | -64 dBm | -78.5 dBm | -82 dBm | -100 dBm |
| IM5 | -96 dBm | neglectable | neglectable | neglectable |

Observation 3: The single TX architecture has IM3 products at the input of the LNAs that will cause significant desense on both receivers.

Observation 4: The single TX architecture IM5 products are much less significant than the IM3 products

Observation 5: The dual TX architecture has significantly lower IM3 and IM5 products at the LNA inputs than the single TX architecture.

Observation 6: The dual TX architecture should be considered as a way forward for addressing IM issues in the CA_71A-n71 combination

# Conclusion

This contribution discusses self-desense for DC_71A-n71 and considers two different architectures.

Observation 1: The order of IM product that would cause a self-desense issue is strongly correlated the BW of the emission.

Observation 2: IM3 components are not a problem for cases with a combined bandwidth of 20 MHz or less

Observation 3: The single TX architecture has IM3 products at the input of the LNAs that will cause significant desense on both receivers.

Observation 4: The single TX architecture IM5 products are much less significant than the IM3 products

Observation 5: The dual TX architecture has significantly lower IM3 and IM5 products at the LNA inputs than the single TX architecture.

Observation 6: The dual TX architecture should be considered as a way forward for addressing IM issues in the CA_71A-n71 combination.

# References

[1] R4-1710849, “TP for TR 37.863-01-01 DC_71A-n71A”, Ericsson, T-Mobile USA

[2] R4-1711448, “LTE-NR Dual Uplink Issues – Other Considerations beyond IMD Order”, Skyworks

[3] R4-1711618, “WF on Single Tx Switched UL”

[4] DA 17-314, Incentive Auction Closing and Channel Reassignment Public Notice, April 13, 2017, FCC