3GPP TSG-RAN WG4 Meeting #85	R4-1712917

Reno, USA, 27th November – 1st December 2017

Title:                        Consideration on SUL band on TDD frequency

Source: 	CMCC

Agenda Item:	9.2.2.9

Document for:	Approval

1   Introduction

SUL is NR supplementary uplink, which is used for improving NR uplink coverage and throughput. So far, several NR SUL band has been defined as follows:

| Band Number | UL | DL | Duplexing mode |
| --- | --- | --- | --- |
| n80 | 1710 - 1785 MHz | N/A | SUL |
| n81 | 880 - 915 MHz | N/A | SUL |
| n82 | 832 - 862 MHz | N/A | SUL |
| n83 | 703 - 748 MHz | N/A | SUL |
| n84 | 1920-1980 MHz | N/A | SUL |
| n85 | 2496-2690MHz | N/A | SUL |

Among the SUL bands, n85 is the SUL band on TDD frequency band 41. So far there is no corresponding SUL band combination approved. In this contribution, we provide our considerations on SUL band on TDD frequency.

2. Discussion

According to RAN1 LS [1], SUL refers to the case when there is only UL resource for a carrier from NR perspective. In NR, TDD frame structure does not support all uplink configuration (0:10). So if operators would like to deploy SUL on TDD frequency, new SUL band needs to be defined [2].

Considering the SUL operation on TDD frequency, there may be some restrictions due to the co-existence issue between operators. When there are multiple operators in one TDD band in the same country, co-existence issue between operators need to be carefully considered. Otherwise, the uplink reception on SUL band may be blocked by the downlink transmission of adjacent operators.

One approach to solve the co-existence issue is to synchronize and apply the unified TDD configurations between operators. This approach will restrict the usage of uplink resources on SUL bands.  For example, in China, the existing TDD configuration on LTE band 41 is 3（DL）:1（UL）, only two uplink slots can be used in every 10ms. In this case, the performance of SUL will be degraded significantly compared to the SUL on FDD frequency at the cell edge, which is of course not preferable.

If operators would like to use all the slots as uplink resource, one possible approach is to reserve guard band to avoid the interference. And how much guard band is required in order to mitigate the interference need to be carefully studied. This approach is also not preferable since the spectrum resource is very valuable, especially for the low frequency range.

In Rel-15, there are already over 350 band combinations including DC, CA and SUL proposed by operators. Considering the high workload in RAN4 and restrictions on TDD SUL operation due to co-existence issue, we suggest that SUL band on TDD frequency that belongs to multiple operators should be deprioritized and not considered in Rel-15.

For the TDD band that belongs to single operator, there is no co-existence issue between operators. It is feasible to use all the slots as uplink resource and achieve the same performance as SUL on FDD frequency.  Hence, it can be treated the same as SUL band on FDD frequency and could be specified in Rel-15.

Proposal: Considering the high workload of RAN4, SUL band on TDD frequency that has co-existence issue between multiple operators should not be considered in Rel-15.

2   Conclusions

In this contribution, we provide our considerations on the SUL band on TDD frequency and provide our views. Considering the co-existence issue on TDD band, we propose that:

Proposal: Considering the high workload of RAN4, SUL band on TDD frequency that has co-existence issue between multiple operators should not be considered in Rel-15.

3   References

R1-1709843, LS on the support of supplementary uplink in NR, CMCC R4-1706966, Way forward on SUL band definition, CMCC