3GPP TSG-RAN WG4 meeting #85	R4-1713048
Reno, Nevada, USA, 27th November – 1st December, 2017

Source:	ZTE Corporation

Title:	TP to TS 38.113 ESD test level

Agenda item:	9.6.1

Document for:	Approval

Introduction

In this contribution we provide TP to the NR EMC specification in TS 38.113[1] for ESD test levels which should be concerned as core part of the TS.

References

- R4-1711984, Draft TS 38.113 v0.0.2, ZTE

--------------Start of text proposal-------------

9.3	Electrostatic discharge

The test shall be performed on a representative configuration of the radio equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

9.3.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the event of an electrostatic discharge.

9.3.2	Test method and level

-	for contact discharge, the equipment shall pass at ±2 kV and ±4 kV;

-	for air discharge shall pass at ±2 kV, ±4 kV and ±8 kV;

The test method is FFS.

-------------End of text proposal-------------