3GPP TSG-RAN WG4 Meeting #85						R4-1713060

Reno, USA, 27 Nov. - 1 Dec., 2017

Source:		NTT DOCOMO, INC., SGS Wireless

Title:	Additional measurement results for MIMO OTA

Agenda Item:	6.2

Document for:	Discussion

Introduction

This contribution provides additional measurement result of LTE band 3, 5, and 19 for defining the FDD Band 3 and low band requirement of MIMO OTA. 4-samples are tested. Based on the work sheet, potential requirements are investigated.

Test Conditions

The measurement is done at SGS Taiwan which is CATL and performance lab of this work. Test condition aligns with [1] and same chamber is used in [2]. Information of measurement equipment is shown in Table 1. 4-samples are measured in this contribution.

Table 1.  Measurement equipment

| Equipment | Manufacturer |
| --- | --- |
| Anechoic chamber | MVG Star MIMO |
| Channel emulator | Keysight Technology Anite F32-16C |
| BS emulator | Anritsu MT8820C |
| Network Analyzer | Agilent E5071C |

Test results

Measurement results

4-test results for Band 3, 5, 19, are provided in this contribution. Over 15 results are available by providing these data at each band. Table 2 and 3 are summary of measurement results of TRMS for band 3, Table 4 and 5 are for band 5, and Table 6 and 7 are for band 19.

.

Table 2.  Summary for band 3 data for 70% of TRMS.

| DUT (dummy) | RE EPRE [dBm//15kHz]@70% TP |  |  | TRMS (average 70%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_16 | -101.44 | -102.05 | -99.05 | -101.03 | 0 | 17 | 9H 8L |
| Sample_17 | -101.22 | -101.61 | -99.47 | -100.86 | 0 | 19 | 10H 9L |
| Sample_18 | -103.93 | -104.69 | -101.80 | -103.64 | 0 | 22 | 10H 12L |
| Sample_19 | -103.92 | -103.82 | -100.88 | -103.08 | 0 | 23 | 12H 11L |

Table 3.  Summary for band 3 data for 95% of TRMS.

| DUT (dummy) | RE EPRE [dBm//15kHz]@95% TP |  |  | TRMS (average 95%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_16 | -99.50 | -100.13 | -97.19 | -99.11 | 0 | 17 | 9H 8L |
| Sample_17 | -99.35 | -99.62 | -97.73 | -98.98 | 0 | 19 | 10H 9L |
| Sample_18 | -101.90 | -102.64 | -99.93 | -101.63 | 0 | 22 | 10H 12L |
| Sample_19 | -101.87 | -101.80 | -98.79 | -101.04 | 0 | 23 | 12H 11L |

Table 4.  Summary for band 5 data for 70% of TRMS.

| DUT (dummy) | RE EPRE [dBm//15kHz]@70% TP |  |  | TRMS (average 70%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_16 | -95.44 | -95.01 | -92.85 | -94.57 | 0 | 17 | 9H 8L |
| Sample_17 | -95.85 | -95.03 | -93.25 | -94.84 | 0 | 19 | 10H 9L |
| Sample_18 | -97.65 | -96.10 | -94.62 | -96.30 | 0 | 22 | 10H 12L |
| Sample_19 | -100.60 | -98.43 | -96.64 | -98.86 | 0 | 23 | 12H 11L |

Table 5.  Summary for band 5 data for 95% of TRMS.

| DUT (dummy) | RE EPRE [dBm//15kHz]@95% TP |  |  | TRMS (average 95%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_16 | -93.57 | -93.08 | -90.89 | -92.66 | 0 | 17 | 9H 8L |
| Sample_17 | -93.97 | -93.06 | -91.35 | -92.92 | 0 | 19 | 10H 9L |
| Sample_18 | -95.25 | -93.75 | -92.34 | -93.94 | 0 | 22 | 10H 12L |
| Sample_19 | -98.38 | -96.30 | -94.55 | -96.69 | 0 | 23 | 12H 11L |

Table 6.  Summary for band 19 data for 70% of TRMS.

| DUT (dummy) | RE EPRE [dBm//15kHz]@70% TP |  |  | TRMS (average 70%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_16 | -95.32 | -94.84 | -92.28 | -94.34 | 0 | 17 | 9H 8L |
| Sample_17 | -95.77 | -94.50 | -92.39 | -94.44 | 0 | 19 | 10H 9L |
| Sample_18 | -97.76 | -96.27 | -94.61 | -96.40 | 0 | 22 | 10H 12L |
| Sample_19 | -100.11 | -98.10 | -96.08 | -98.40 | 0 | 23 | 12H 11L |

Table 7.  Summary for band 19 data for 95% of TRMS.

| DUT (dummy) | RE EPRE [dBm//15kHz]@95% TP |  |  | TRMS (average 95%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_16 | -93.43 | -92.92 | -90.42 | -92.44 | 0 | 17 | 9H 8L |
| Sample_17 | -93.90 | -92.62 | -91.08 | -92.68 | 0 | 19 | 10H 9L |
| Sample_18 | -95.29 | -93.92 | -92.63 | -94.08 | 0 | 22 | 10H 12L |
| Sample_19 | -97.86 | -95.93 | -94.02 | -96.22 | 0 | 23 | 12H 11L |

Analysis of results

With using the worksheet [4], CDF value is calculated. Table 4 shows the summary of the CDF value. Band 19 data is also listed for comparison.

| DUT (dummy) | Band 3 |  |  | Band 5 |  |  | Band 19 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 80% value of CDF | 85% value of CDF | 90% value of CDF | 80% value of CDF | 85% value of CDF | 90% value of CDF | 80% value of CDF | 85% value of CDF | 90% value of CDF |
| 70% limit | -96.0 | -94.9 | -94.7 | -94.7 | -94.6 | -94.3 | -95.6 | -95.3 | -95.1 |
| 95% limit | -94.1 | -92.8 | -92.5 | -92.7 | -92.7 | -92.6 | -93.5 | -93.3 | -93.2 |

Conclusion

This contribution provides additional measurement result of LTE band 3, 5, and 19 for defining the FDD Band 3 and low band requirement of MIMO OTA. 4-samples are tested. Based on the work sheet, requirements are proposed in [5].

References

[1]    TR 37.977, V14.3.0, “Verification of radiated multi-antenna reception performance of UEs,” 3GPP RAN #75, March 2017

[2]    R4-1706401, Measurement results for lab alignment test activity – Re-tested at SGS, NTT DOCOMO INC., SGS Wireless, MVG Industries, RAN4#MIMO OTA ad-hoc

[3]    R4-1704529, MIMO OTA measurement results for band 1 and 19, NTT DOCOMO, SGS Wireless, RAN4#83

[4]     R4-1709061, TRMS joint band passing rate worksheet, Intel,

[5]   R4-1713075, Proposal for MIMO OTA TRMS requirements, NTT DOCOMO, SGS Wireless, RAN$#85