3GPP TSG-RAN WG4 #85	R4-1713109

Reno, USA, November 27th ‒ December 1st, 2017

Agenda item:	9.3.1

Source: 	Qualcomm Incorporated

Title: 	TP for TS 38.101-1: Channel Bandwidth

Document for:	Approval

# Introduction

In this paper we present the text proposal for the channel bandwidth definition for FR1.

# Reference

<<< Start of TP >>>

## 5.3	Channel Bandwidth

### 5.3.1	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 5.3.1.-1

Table 5.3.1-1: Maximum transmission bandwidth configuration NRB

| SCS [kHz] | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N.A | N.A | N.A |
| 30 | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
| 60 | N.A | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |

### 5.3.2	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1 The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.2-1.

Table 5.3.2-1: Minimum Guradband

| SCS [kHz] | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 |  | 242.5 |  | 312.5 |  | 382.5 |  | 452.5 |  | 522.5 |  | 552.5 |  | 692.5 |  | N.A |  | N.A |  | N.A |  |
| 30 |  | 505 |  | 665 |  | 645 |  | 805 |  | 785 |  | 905 |  | 1045 |  | 825 |  | 925 |  | 845 |  |
| 60 |  | N.A |  | 1010 |  | 990 |  | 1330 |  | 1310 |  | 1610 |  | 1570 |  | 1530 |  | 1450 |  | 1370 |  |

Figure 5.3.2-1 Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

The number of RBs configured in any channel bandwidth shall ensure that the minimum guardband specified in this clause is met. In the case that multiple numerologies are multiplexed in the same symbol, the minimum guardband is dictated by the subcarrier spacing of the respective edge RB.

### 5.4.3	RB alignment with Different Numerologies

The reference position on which the resource element 0 aligns among supported numerologies is provided in TBD

### 5.4.4	Channel bandwidth per Operating Band

The requirements in this specification apply to the combination of channel bandwidths, subcarrier spacing and operating bands shown in Table 5.4.4-1. The transmission bandwidth configuration in Table 5.6.1-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the TX and RX path.