3GPP TSG-RAN4 Meeting #85 	R4-1713722

# Reno, United States, 27 November – 1 December 2018

Title: 	TP to TS 38.133: Number of carriers being monitored

Source: 	Nokia, Nokia Shanghai Bell

Agenda item:	9.7.3.1

Document for:	Approval

# 1 Introduction

A text proposal to capture the number of carriers being monitored in 38.133 version 0.3.0.

# 2 TP for TS 38.133 v0.3.0

##### 9.1.3.1	Monitoring of multiple layers using gaps for NSA

The requirements in this section are applicable for an E-UTRA-NR dual connectivity capable UE.

When monitoring of multiple inter-frequency E-UTRAN, inter-frequency NR and inter-RAT NR carriers using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (SS-RSRP, SS-RSRQ, SS-SINR, E-UTRAN RSRP, E-UTRAN RSRQ, E-UTRAN RS-SINR etc.) of detected cells on all the layers.

The effective total number of frequencies excluding the frequencies of the LTE PCell, LTE SCells, PSCell and SCells being monitored is Nfreq, NSA, which is defined in section 8.1.2.1.1b in [TS 36.133].

##### 9.1.3.2	Maximum allowed layers for multiple monitoring

The UE shall be capable of monitoring at least:

-	Depending on UE capability, [12] NR inter-frequency carriers

Additionally, the UE shall be capable of monitoring a total number of effective carrier frequency layers as defined in Section 8.1.2.1.1b.1 in [TS 36.133].