3GPP TSG-RAN WG4 Meeting 85	R4-1714292

Reno, USA, November 27th – December 1st, 2017

Title:	Acquisition of NTA_Offset for Uplink Transmission

Release:	Release 15

Work Item:	NR_newRAT

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

#### Name:	Magnus Larsson

E-mail Address:	magnus.k.larsson@ericsson.com

Attachments:	None

- Overall Description:

RAN4 has discussed the values of NTA_Offset defined in TS 38.211. RAN4 has agreed the following values of NTA_Offset expressed in Tc except the one for uplink transmission on a TDD carrier in frequency range#1 (FR1) when the NR co-exist with LTE.

Table 1: The Value of  [Tc]

| Duplex Mode of cell used for uplink transmission |  |
| --- | --- |
| FDD in FR1 or FR2 | 0 (Note) |
| TDD in FR1 with LTE-NR coexistence | TBD |
| TDD in FR1 without LTE-NR coexistence | 25560 (correspond to 13 µs) |
| TDD in FR2 | 13763 (correspond to 7 µs) |
| Note: Regardless of whether LTE-NR coexistence is configured or not in FR1 |  |

In LTE the NTA_Offset for uplink transmission on a TDD carrier is 624Ts ( 20 µs). RAN4 would like to ask RAN1 to evaluate if there is any impact on the performance if the same NTA_Offset is used regardless of whether or not the NR co-exist with LTE on a TDD carrier in FR1. RAN4 would also like to seek guidance from RAN1 regarding NTA_Offset for uplink transmission on a TDD carrier in FR1 when the NR co-exist with LTE.

The UE shall apply a certain NTA_Offset value for any uplink transmission including a random access during an initial access procedure.

2. Actions:

To RAN1: RAN4 kindly requests RAN1 to:

- evaluate if there is any impact on the performance if NTA_Offset is the same regardless of whether or not the NR co-exist with LTE on a TDD carrier in FR1 and

- provide guidance regarding NTA_Offset for uplink transmission on a TDD carrier in FR1 when the NR co-exist with LTE.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting AH-1801 				22 - 26 Jan 2018, San Diego, USA

TSG RAN WG4 Meeting #86 					26 Feb - 2 Mar 2018, Athens, Greece