3GPP RAN4 Meeting #85	R4-1712320
Reno, USA, 27 November – 1 December, 2017

Agenda item:	9.4.4.1.2

Source:	Intel Corporation

Title:	TP on UE power class for FR2

Document for:	Approval

# 1	Introduction

Now that mm-wave power class definition is to be agreed on the upcoming meeting [1], it is pertinent to establish how this data will be incorporated and presented in a technical specification document (TS 38.101). This paper focuses on our text proposal for transmitter characteristics and UE max output power.

# 2	Discussion

As power class for FR2 discussions evolved [2-4], focus shifted from simply stating the UE must produce a certain EIRP expressed as nominal with a given tolerance, to defining a peak minimum EIRP. Due to the more sensitive nature of this frequency range, all in-package losses must be carefully attributed to the link budget calculation. A detailed survey of several companies’ reported peak EIRP values helps highlight how to properly choose the right EIRP value for definition [5].

# 3	References

- R4-1711895, “WF on power class in FR2,” Intel Corporation, Apple, Samsung, LGE, Huawei, MediaTek, Xiaomi, vivo, OPPO, Motorola, 3GPP RAN4 #84bis, October 2017

- TR 38.803, “Study on New Radio Access Technology RF and co-existence aspects,” Version 2.0.0, 3GPP RAN #75, March 2017

- R4-1704362, “WF on mmW UE power class,” Qualcomm, Ericsson, Sony, Skyworks, 3GPP RAN4 #82bis, April 2017

- R4-1708914, “WF on Power Class and CDF,” NTT DOCOMO Inc., 3GPP RAN #84, August 2017

- R4-1712319, “UE power class for FR2,” Intel Corporation, Apple Inc., OPPO, Samsung, LGE, Xiaomi, MediaTek, Motorola, Huawei, 3GPP RAN4 #85, November 2017

# 4	Text proposal

<start of text proposal>

# 6	Transmitter characteristics

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA) with a single or multiple transmit chains.

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The values listed on the table below are for handheld UE, defined as minimum peak EIRP.

Table 6.2.2-1: NR FR2 UE Power Class

| NR band |  | Handheld Power Class Min Peak EIRP (dBm) |
| --- | --- | --- |
| n257 |  | [21.2-25.2] |
| n258 |  | [21.2.25.2] |
| n260 |  | [19.4-23.7] |
| NOTE 1: minimum peak EIRP is defined as the lower limit without tolerance |  |  |

The maximum output power values for TRP and EIRP are found on the table below. The max allowed EIRP is derived from regulatory requirements [3].

Table 6.2.2-2: NR UE Maximum Output Power Limits

| NR band | TRP Handheld (dBm) | EIRP (dBm) Handheld |
| --- | --- | --- |
| n257 | TBD | 43 |
| n258 | TBD | 43 |
| n260 | TBD | 43 |

<end of text proposal>