3GPP TSG-RAN WG4 Meeting #86	R4-1801428

Athens, Greece, 26 Feb – 2 March 2018

Agenda item:	7.2.1.2

Source: 	CMCC

Title: 	TP for TR 38.813 HPUE of n78 and n77

Document for:	Approval

# 1. Introduction

RAN4 has approved TR 38.813 v0.2.0_New frequency range for NR 3.3GHz - 4.2GHz [1]. This paper is a text proposal to capture the agreements of Band n77 and n78 HPUE requirements for TR 38.813 based on [2].

# 2.	References

[1]	R4-1800511, “TR 38.813 v0.2.0_New frequency range for NR 3.3GHz - 4.2 GHz ”, CMCC

[2]	R4-1801430, “Proposal on NR HPUE definition for PC2”, CMCC

# Text proposal for TR 38.813

----- Start of TP -----

<Unchanged sections omitted>

7	NR band 3.3GHz - 3.8GHz specific issues

7.1	UE specific

7.1.1	Transmitter characteristics

7.1.1.1	UE maximum output power

The MOP requirements for Band 42 and 43 single-carrier were specified as 23 dBm +2/-3 dB for power class 3 based on simulation results shown in Table 7.1.1.1-1 in August 2011.

Table 7.1.1.1-1 Simulation results for combined Band 42 and Band 43 filter [R4-114656]

|  | Bandwidth | Max IL (corner) | 2f0 rejection | 2.7 GHz rejection |
| --- | --- | --- | --- | --- |
| Design 7 | 400 MHz | 1.9 | 20 dB | 30 dB |
| Design 8 | 400 MHz | 2.3 | 15 dB | 10 dB |
| Design 9 | 400 MHz | 2.0 | 20 dB | 15 dB |

When specifying MOP requirements of Band n77 and n78, the delta to be checked is IL impact of BPF due to the extension of pass-bandwidth. It is shown in Table 7.1.1.1-2.

Table 7.1.1.1-2.  Simulation results for Band n77 and n78

| Parameter | Frequency range | Vendor 1 |  | Vendor 2 |  | Vendor 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF |
| Insertion loss (ETC) | 3300-3400 MHz | 1.05 dB | 1.05 dB | 2.0 dB | 2.0 dB | 1.9 dB | 1.9 dB |
|  | 3400-3800 MHz | 1.0 dB | 1.0 dB | 1.5 dB | 1.5 dB |  |  |
|  | 3800-4200 MHz | - | 1.25 dB | - | 2.0 dB | - |  |
| Attenuation (Typ) | 698-2690 MHz | 41.0 dB | 41.0 dB | 45 dB | 35 dB (ETC) | 46.3 dB | 47.8 dB |
|  | 5150-5925 MHz | 40.2 dB | 40.2 dB | 35 dB | 35 dB (ETC) | 40.8 dB | 35.6 dB |

With comparison between Table 7.1.1.1-1 and Table 7.1.1.1-2, it was concluded that there is no degradation of IL at ETC even with the extension and agreed to specify the same MOP as that of Band 42 and 43 (i.e., 23 dBm +2/-3 dB) for Band n77 and n78 of power class 3.

Agreement: MOP is to be specified as 23 dBm +2/-3 dB for Band n77 and n78 of power class 3

The following NR UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non-CA configuration and UL-MIMO unless otherwise stated.

Table 7.1.1.1-3: NR UE power classes

| NR Operating band | Class 2 | Class 3 | Comments |
| --- | --- | --- | --- |
| n77 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 4.2 GHz |
| n78 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 3.8 GHz |
| n79 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 4.4 - 5 GHz |

For a power class 2 UE, applicability to certain restriction of uplink/downlink configuration is TBD.

If UE is configured for power class 2 UE, the requirements in Table 7.1.1.1-4.

Table 7.1.1.1-4: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

7.1.1.5	Spectrum emission mask

The power of any UE emission shall not exceed the levels specified in Table 7.1.1.5-1 for the specified channel bandwidth.

Table 7.1.1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  |  |  |  |  |  |  |  |  |
| ± 10-15 | -25 |  |  |  |  |  |  |  |  |
| ± 15-20 |  | -25 |  |  |  |  |  |  |  |
| ± 20-25 |  |  | -25 |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  | -25 |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  | -25 |  |

No changes to Table 7.1.1.5-1 are needed as a result of Band n78 power class 2 operation.

7.1.1.6	Spurious emissions

Since Band n77 and/or n78 will be used at least in Japan, bands operated in Japan need to be protected. In addition, protected bands required in Europe, Korea and China regions (i.e., operating bands specified for Band n78 of 3.3-3.8 GHz) should also be added for potential use of these bands. As a consequence, required protected bands for Band n77 and n78 will be the same. Note that it was already agreed not to specify the co-existence requirement between Band n77 and Band n79, which means that the co-existence will be guaranteed by the NR general emission requirements.

Table 7.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

No changes to Table 7.1.1.6-1 are needed as a result of Band n78 power class 2 operation.

7.1.2	Receiver characteristics

No changes to receiver characteristics are needed as a result of Band n78 power class 2 operation.

8	NR band 3.3GHz - 4.2GHz specific issues

8.1	UE specific

For Power Class 2 UE in Band n77 supporting UL MIMO, the requirements in Clause 7.1.1.7 and 7.1.2.4 apply.

# For UE in Band n77 supporting 4Rx, the requirements in Clause 7.1.2.5 apply.

8.1.1	Transmitter characteristics

8.1.1.1	UE maximum output power

Based on the same discussion as UE maximum output power of Band n78 described in clause 7.1.1.1, the following was agreed.

Agreement: MOP is to be specified as 23 dBm +2/-3 dB for Band n77 and n78 of power class 3

The following NR UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non-CA configuration and UL-MIMO unless otherwise stated.

Table 8.1.1.1-1: NR UE power classes

| NR Operating band | Class 2 | Class 3 | Comments |
| --- | --- | --- | --- |
| n77 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 4.2 GHz |
| n78 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 3.8 GHz |
| n79 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 4.4 - 5 GHz |

For a power class 2 UE, applicability to certain restriction of uplink/downlink configuration is TBD.

If UE is configured for power class 2 UE, the requirements in Table 8.1.1.1-2.

Table 8.1.1.1-2: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

8.1.1.5	Spectrum emission mask

The power of any UE emission shall not exceed the levels specified in Table 8.1.1.5-1 for the specified channel bandwidth.

Table 8.1.1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  |  |  |  |  |  |  |  |  |
| ± 10-15 | -25 |  |  |  |  |  |  |  |  |
| ± 15-20 |  | -25 |  |  |  |  |  |  |  |
| ± 20-25 |  |  | -25 |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  | -25 |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  | -25 |  |

No changes to Table 8.1.1.5-1 are needed as a result of Band n77 power class 2 operation.

8.1.1.6	Spurious emissions

Based on the same discussion as Spurious emissions of Band n78 described in clause 8.1.1.6, the following was agreed.

Table 8.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

No changes to Table 8.1.1.6-1 are needed as a result of Band n77 power class 2 operation.

8.1.2	Receiver characteristics

No changes to receiver characteristics are needed as a result of Band n77 power class 2 operation.

<Unchanged sections omitted>

----- End of TP -----