3GPP TSG-RAN4 Meeting #86 	R4-1801436

# Athens, Greece, 26th February – 2nd March, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5526 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR PSCell addition and release delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-2-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR PSCell addition and release delay requirements for EN-DC were completed in RAN4#85 meeting; however, there is some TBD values need to be determined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the TBD to the proposed value for activation time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR PSCell addition and release delay requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

7.34	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity

7.34.1	Introduction

This section defines requirements for the delay within which the UE shall be able to configure an NR PSCell in E-UTRA – NR dual connectivity. The requirements in this section apply for UE capable of E-UTRA-NR dual connectivity with PCell which can be FDD or TDD.

7.34.2	NR PSCell Addition Delay Requirement

The requirements in this section shall apply for the UE which is configured with PCell, and may also be configured with one or more SCells.

Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n + Tconfig PSCell:

Where:

Tconfig_PSCell = TRRC_delay + Tactivation_time + TSFN_acquisition + TPCell_ DU + TPSCell_ DU

TRRC_delay is the RRC procedure delay as specified in [2].

Tactivation_time is the NR PSCell activation delay.

| Tactivation_time | SCell unknown | SCell known |
| --- | --- | --- |
| FR1 | [1+X]*SMTC_period + [Y]*TRS_period | [X]*SMTC_period + [Y]*TRS_period |
| FR2 |  |  |
| Note 1: The exact values of X and Y depends on the configured SMTC period and TRS period. Note 2: The sum of X and Y shall be [5]. Note 3: When TRS is not configured by gNB, Y can be zero. |  |  |

TPCell_ DU is the delay uncertainty due to PCell PRACH preamble transmission. TPCell_ DU is up to 20 ms if NR PSCell activation is interrupted by an PCell PRACH preamble transmission, otherwise it is 0.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU is up to [1 PRACH occasion period] as defined in [2].

TSFN_acquisition is the time used for SFN acquisition. If the SFN is unknown, TSFN_acquisition is [1 SMTC period] provided that the SFN can be successfully decoded on the first attempt. If the SFN is known, TSFN_acquisition is 0 ms.

In FR1, NR PSCell is known if it has been meeting the following conditions:

During the last [5] seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3of TS 38.133,

-	NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3 of TS 38.133.

otherwise it is unknown.

The PCell interruption specified in section 7.32is allowed only during the RRC reconfiguration procedure [2].

Editor’s note: Condition of known NR PSCell in FR2 is FFS;

Editor’s note: It is FFS whether SRS carrier based switching during NR PSCell addition procedure impacts NR PSCell addition delay requirement.

7.34.3	NR PSCell Release Delay Requirement

The requirements in this section shall apply for a UE which is configured with PCell and NR PSCell, and may also be configured with one or more SCells and/or NR SCells.

Upon receiving NR PSCell release in subframe n, the UE shall accomplish the release actions specified in [2] no later than in subframe n+ TRRC_delay:

Where

TRRC_delay is the RRC procedure delay as specified in [2].

The PCell interruption specified in section 7.32 is allowed only during the RRC reconfiguration procedure [2].

Editor’s note: It is FFS whether SRS carrier based switching during NR PSCell release procedure impacts NR PSCell release delay requirement.

<< End of Change >>