3GPP TSG-RAN WG4 Meeting #86                                                                                         R4-1801501

Athens, Greece, 26 Feb - 02 Mar 2018

Agenda item:            7.9.9.1.1

Source:	MediaTek Inc.

Title:	PDCCH Link Level Simulation Results

Document for:          Discussion

1	Introduction

According to the approved PDCCH simulation assumptions [1], PDCCH link level simulation results are provided. In this contribution, we further discuss the settings of hypothetical PDCCH parameters, including

- Ratio of hypothetical PDCCH RE energy to average SSS RE energy

- Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy

2     	Simulation Results

Table 1 summarizes the required SINRs to achieve 2% BLER and 10% BLER. The corresponding simulation parameters and performance curves can be found in Appendix.

| FR1 | Required SINR (unit: dB) |  |  |  |
| --- | --- | --- | --- | --- |
| Channel | 8CCE@BLER10% | 8CCE@BLER2% | 4CCE@BLER10% | 4CCE@BLER2% |
| Static | -7.73 | -7.15 | -5.38 | -4.76 |
| EPA 3 Km/hr | -4.97 | -2.86 | -2.34 | 0.016 |
| EPA 42 Km/hr | -4.76 | -2.37 | -2.04 | 0.64 |
| ETU 3 Km/hr | -4.84 | -3.41 | -2.15 | -0.51 |
| ETU 42 Km/hr | -4.73 | -3.29 | -2.08 | -0.53 |

Table 1: Required SNR for INS and OOS target BLER

3	Hypothetical PDCCH parameters

Under static channel,

- LTE OOS required SNR is -11.5 dB to achieve 10% BLER with 8CCEs [2], where 4 dB power boosting on PDCCH is assumed.

- NR OOS required SNR is -7.73 dB to achieve 10% BLER with 8 CCEs, where 0 dB power boosting on PDCCH is assumed.

It is observed that the required SINR of NR is 3.8 dB worse than that of LTE to achieve 10% BLER for OOS indication. The main reason would be that NR has no power boosting on PDCCH data and DMRS REs but LTE has 4dB power boosting on PDCCH and PCFICH REs. To keep the NR DL coverage comparable to LTE, there are two approaches could be jointly used to lower OOS required SNR:

- Approach #1: Increasing to 16 CCEs

- Approach #2: Power boosting on PDCCH data and DMRS REs

However, it would be a good starting point to leverage LTE hypothetical settings, i.e. 8 CCEs and 4 dB power boosting, for NR hypothetical OOS PDCCH parameters.

Proposal 1: Using 8CCEs and [4] dB power boosting on PDCCH data and DMRS REs for hypothetical PDCCH OOS parameters.

When determining PDCCH transmission parameters, from UE implementation of view, the following two considerations shall be taken into account.

- Consideration 1: Accuracy of SINR link quality estimation

- Consideration 2: Sufficient gap between OOS required SNR and INS required SNR, among all possible channels

Regarding consideration 1, because NR RLM-RS time domain density is far less than LTE CRS, a larger variance of NR radio link quality estimation (i.e. SINR span) can be expected [3].

Thus, it would lead to unnecessary RLF if the SINR estimation is not sufficiently accurate and the gap between OOS required SNR and INS required SNR is too small. As shown in Figure 1, the RLF could be false alarmed at the test SINR point 1, and that will be troublesome to design RLM test cases.

Figure 1: unexpected RLF when INS and OOS required SNRs are too close

UE implementation complexity will increase rapidly for small SINR gap between OOS and INS among all possible channels. For example, assume no power boosting on PDCCH for OOS,

- INS required SINR   = -4.76dB @ Static channel

- OOS required SINR = -4.97dB @ EPA 3Km/hr channel

The SINR difference is only 0.2 dB, and then UE will be forced to detect the channel type and power delay profile under such low SNR with high implementation cost.

Proposal 2: RAN4 shall consider SINR estimation accuracy and UE implementation cost when designing PDCCH parameters.

4	Conclusion

In this contribution, PDCCH simulation results are provided. Based on the discussion in section 3, we have the following proposals:

Proposal 1: Using 8CCEs and [4] dB power boosting on PDCCH data and DMRS REs for hypothetical PDCCH OOS parameters.

Proposal 2: RAN4 shall consider SINR estimation accuracy and UE implementation cost when designing PDCCH parameters.

5	Reference

[1] R4-1801068, “Simulation assumption for NR RLM”, Nokia, Nokia Shanghai Bell

[2] TS 36.133., ``Requirements for support of radio resource management’’

[3] R4-1712398, ``Link Level Simulation Results for RLM’’, MediatTek

6	Appendix

| Parameter | Value |
| --- | --- |
| Carrier frequency | FR1, 4 GHz // no FR2: RF impairment |
| DCI format / payload size | Format 1-0 Number of bits: 58 (48 PRBs) |
| Number of control OFDM symbols | {2 symbols, 15 kHz, 48 PRB} |
| Bandwidth (MHz) |  |
| Sub-carrier spacing (kHz) |  |
| Aggregation level (CCE) | 4 and 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB |
| Precoder granularity in frequency domain | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| SNR | [-12, -11, … 4, 5, 6] |
| Frequency error | 0 |
| Propagation condition | AWGN, EPA, ETU (only for 15kHz SCS) |
| Antenna configuration | 2x2 Note |
| UE speed | 3km/h, 42km/h |
| Note: Companies are encouraged to clarify whether the 2Tx simulation results are based Tx diversity or not. |  |

Table 2: Simulation Setting

Figure 2: PDCCH performance under static channel

Figure 3: PDCCH performance under EPA channel

Figure 4: PDCCH performance under ETU channel