3GPP TSG-RAN WG4 Meeting #86	R4-1802157

Athens, Greece, 26 Feb - 2 Mar, 2018

Agenda item:	7.3.1

Source:	Dish Network

Title:	Discussion on bandwidths for FR1

Document for:	Discussion

# 1	Introduction

This contribution aims to raise discussion on if >20MHz single CC bandwidth or intra-band contiguous CA is more beneficial for FR1.

# 2	Discussion

In LTE, intra-band CA was easily justified because operators have >20MHz contiguous spectrum and the maximum single CC bandwidth is 20MHz. In NR the situation is different; the maximum single CC bandwidth is 100MHz which in practice poses no limits for LTE re-farming spectrum usage.

This contribution discusses if it were more beneficial to define NR single CC bandwidths up to 40MHz with band specific granularity for the LTE re-farming bands which would otherwise use intra-band contiguous CA to implement up to 40MHz aggregated bandwidth. There are >10 bands in LTE for which CA class C is defined.

Let’s use band B1 as an example. B1 has 5/10/15/20 MHz bandwidths. It has intra-band contiguous CA defined with 25/30/35/40MHz aggregated bandwidths. In LTE re-farming, it has been used as a baseline that at least the same bandwidths equal or above 5MHz are supported in NR. Hence n1 bandwidths are currently 5/10/15/20MHz. In current specifications, some new bandwidths >20Mhz have been added for some LTE re-farming bands.

NR 2CC intra-band contiguous CA for band n1 would be defined according to n1 operator specific requests, possibly leading into 25/30/35/40MHz aggregated bandwidth granularity. The question we would like RAN4 to discuss is whether it would be more attractive to define new single CC bandwidths for LTE re-farming bands instead of using intra-band contiguous CA from day one? If band specific <20MHz single CC bandwidths up to 40MHz were defined, then there would be no need for intra-band contiguous CA for aggregated bandwidths 40MHz.

Question: Should RAN4 define band specific >20MHz single CC bandwidths up to 40MHz for LTE re-farming bands instead of defining “LTE BW class C” type intra-band contiguous CA for these bands?

There are several aspects that need to be accounted for. We mention a couple of them just to initiate discussion.

- New single CC bandwidths would bring more complexity at least in single CC specifications because of potential introduction of 25/30/35/40MHz bandwidths and would need to be considered in conformance testing; however not introducing 2CC intra-band CA for these bands would reduce the complexity and likely, reduce overall conformance testing.

- Another aspect is whether it is more practical from both network and UE perspective to have wider single CC bandwidths as the SCELL configuration/reconfiguration time is quite short.

- Third aspect is UE baseband impact, where different FFT’s could be used for CA compared with a single FFT for single CC.

- Fourth aspect relates to aggregated bandwidths with current NR bands where some wider bandwidths are added. For instance, n66 has single CC bandwidths 5/10/15/20/40. Hence CA_n66C would result as aggregated bandwidths 25/30/35/45/50/55/60/80. Does RAN4 see any issues with having some aggregated bandwidths (25/30/35) being smaller than maximum single carrier bandwidth?

All in all, it is not obvious which approach is better in the long run.

This question is important, because at least we initially think that if RAN4 concludes adding wider single CC bandwidths is more beneficial than defining intra-band CA, the additions should be done early, preferably during release 15. This is because it would likely not be practical to define intra-band contiguous CA first and then in later releases define wider bandwidths which would make CA defined in earlier releases redundant.

# 3	Conclusions

Whether intra-band contiguous class C CA or defining additional bandwidths for LTE re-farming bands is more beneficial was discussed. We hope RAN4 would discuss the following question.

Question: Should RAN4 define band specific >20MHz single CC bandwidths up to 40MHz for LTE re-farming bands instead of defining “LTE BW class C” type intra-band contiguous CA for these bands?

# 4	References