3GPP TSG-RAN WG4 Meeting#86	R4-1802326

# Athens, Greece, 26th Feb – 2nd March January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for NR NSA Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements needs to be added for contention free random access procedure for beam failure recovery purpose, contention free CSI-RS based random access, and necessary change due to multiple preamble transmission. Further description for SS/PBCH block selection and supplementary UL are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (1) Add contention free random access procedure for beam failure recovery; (2) Add contention free CSI-RS based random access procedure; (3) Necessary change for multiple preamble transmission; (4) Delete editorial notes for not applicable; (5) Add the condition of SS-RSRP threshold for SS/PBCH block selection; (6) Correct the description for non-contention based RA, i.e., after failed reception of RAR, UE could either transmit the origianl preamble index or use another one based on the selected SSB or CSI-RS, if the association configured; (7) Correct the condition desciption for applying RA over supplementary UL carrier, i.e., only applying for the cell for the random access procedure configured with supplementary UL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for NSA random access is not completed and error exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

### 6.2.2	Random access

#### 6.2.2.1		Introduction

This clause contains requirements on the UE regarding random access procedure. The random access procedure is initiated to establish uplink time synchronization for a UE which either has not acquired or has lost its uplink synchronization, or to convey UE’s request Other SI, or to request beam failure recovery. The random access is specified in clause 8 of TS 38.213[3] and the control of the RACH transmission is specified in clause 5.1 of TS 38.321[7].

#### 6.2.2.2		Requirements

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula defined in TS 38.213 [3] and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1[18] for frequency range 1 [and in Table x.x of TS 38.101-2 [19] for frequency range 2]. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1[18] for frequency range 1 [and in Table x.x of TS 38.101-2 [19] for frequency range 2].

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PSCell as specified in clause 5.1.4 in TS 38.321 [7].

Editor’s note: The requirement for random access procedure on PCell is not completed because of the priority on E-UTRA-NR Dual Connectivity (EN-DC) by December, and is targeted for completion in June 2018.

##### 6.2.2.2.1	Contention based random access

# 6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble

With the UE selected SS/PBCH block with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to select a Random Access Preamble randomly with equal probability from the random access preambles associated with the selected SS/PBCH block if the association between Random Access Preambles and SS blocks is configured, as specified in clause 5.1.2 in TS 38.321 [7].

With the UE selected SS/PBCH block with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SS/PBCH block, if the association between PRACH occasions and SS/PBCH blocks is configured, as specified in clause 5.1.2 in TS 38.321 [7].

# 6.2.2.2.1.2	Correct behaviour when receiving Random Access Response

The UE may stop monitoring for Random Access Response(s), shall stop transmitting remaining preambles if ‘multiple preamble transmission’ signalled, and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

# 6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [7].

# 6.2.2.2.1.4	Correct behaviour when receiving a NACK on msg3

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3.

# 6.2.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI

The UE shall send ACK if the Contention Resolution is successful.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

# 6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

##### 6.2.2.2.2	Non-Contention based random access

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble

With the UE selected SS/PBCH block with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS/PBCH blocks, UE shall have the capability to select the Random Access Preamble corresponding to the selected SS/PBCH block, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SS/PBCH block, if the association between contention free PRACH resources and SS/PBCH blocks is configured, as specified in clause 5.1.2 in TS 38.321 [7].

With the UE selected CSI-RS with CSI-RSRP above rsrp-dedicatedRACH-Threshold amongst the associated CSI-RSs, UE shall have the capability to select the Random Access Preamble corresponding to the selected CSI-RS, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected CSI-RS, if the association between contention free PRACH resources and CSI-RSs is configured, as specified in clause 5.1.2 in TS 38.321 [7].

If the random access procedure is initialized by a beam failure indication from lower layer and if the contention free PRACH resources for beam failure recovery request associated with any of the SS/PBCH blocks and/or CSI-RSs is configured, UE shall have the capability to select the Random Access Preamble corresponding to the selected SS/PBCH block with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS/PBCH blocks or the selected CSI-RS with CSI-RSRP above rsrp-dedicatedRACH-Threshold amongst the associated CSI-RSs, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SS/PBCH block or selected CSI-RS, as specified in clause 5.1.2 in TS 38.321 [7].

# 6.2.2.2.2.2	Correct behaviour when receiving Random Access Response

The UE may stop monitoring for Random Access Response(s) and shall stop transmitting remaining preambles if ‘multiple preamble transmission’ signalled, if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE may stop monitoring for Random Access Response(s) and shall monitor the Other SI transmission if the Random Access Response only contains a Random Access Preamble identifier which is corresponding to the transmitted Random Access Preamble and the random access procedure is initialized for SI request from UE, as specified in clause 5.1.4 in TS 38.321 [7].

The UE may stop monitoring for Random Access Response(s), if the contention free Random Access Preamble for beam failure recovery request was transmitted and if the PDCCH addressed to UE’s C-RNTI is received, as specified in clause 5.1.4 in TS 38.321 [7].

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

# 6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window or if no PDCCH addressed to UE’s C-RNTI is received within the BFR Response window,as defined in clause 5.1.4 in TS 38.321 [7].

##### 6.2.2.2.3	UE behaviour when configured with supplementary UL

In addition to the requirements defined in clause 6.2.2.2.1 and 6.2.2.2.2, a UE configured with supplementary UL carrier shall use RACH configuration for the supplementary UL carrier contained in RMSI and RRC dedicated signalling. If the cell for the random access procedure is configured with supplementary UL, the UE shall transmit or re-transmit PRACH preamble on the supplementary UL carrier if the SS-RSRP measured by the UE on the DL carrier is lower than the sul-RSRP-Threshold as defined in TS 38.331 [2].

### >>>>>End of Change 1<<<<<