3GPP TSG-RAN WG4 Meeting #86	R4-1802731

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	7.6.2.5.1.2

Source: 	Nokia, Nokia Shanghai Bell, ZTE

Title: 	TP to TR 38.817-02: NR CACLR requirement for non-continuous allocation in FR1 (6.6.3)

Document for:	Approval

# 1.	Introduction

The way forward on NR CACLR requirement for non-continuous allocation was agreed in RAN4 AH-1801 [1].

This contribution provides a text proposal to record the agreements in FR1 into TR 38.817-02 [2] for future reference.

# 2.	Text Proposal

<Start of change>

### 6.6.3	Adjacent Channel Leakage Power Ratio (ACLR)

The following have been agreed for conducted ACLR requirement in FR1:

-	Adopt the E-UTRA BS (45dB) ACLR1 and ACLR2 to all NR BS classes.

-	Adopt each E-UTRA BS absolute limit to the corresponding NR BS class, whichever is less stringent compared to the ACLR.

-	The same channel bandwidth with wanted signal can be assumed as the adjacent channel bandwidth.

-	Transmission bandwidth configuration of wanted channel (it would differ depend on subcarrier spacing and/or channel bandwidth) should be used as a measurement bandwidth for wanted signal power measurement.

-	Maximum transmission bandwidth configuration of the channel bandwidth (between subcarrier spacing) specified in Release 15 should be used as a measurement bandwidth for adjacent channel power measurement, i.e. the measurement bandwidth should also apply to future releases regardless of whether new SU is introduced or not.

-	For bands defined also for E-UTRA or UTRA:

-	Specify BS 45 dB ACLR conducted requirement for 5MHz adjacent channel bandwidth.

-	The same measurement bandwidth is used for adjacent channel power measurement with existing E-UTRA specification.

-	Not to specify against other E-UTRA channel bandwidth than 5MHz.

-	No need to specify ACLR against UTRA in core specification since it is already covered by ACLR against E-UTRA case.

Based on above agreements, the ACLR requirement is specified in Table 6.6.3-1.

Table 6.6.3-1: Base Station ACLR in FR1

| BS Channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA and/or UTRA. |  |  |  |  |

The following have been agreed for conducted CACLR requirement in FR1:

-	Adopt the E-UTRA BS relative (45dBc) and absolute (for each BS class) CACLR limits for NR BS.

-	Only specify CACLR against NR in core specification.

-	Apply the 5 MHz adjacent channel carrier to NR channel bandwidth ≤20 MHz and the 20 MHz adjacent channel carrier to NR channel bandwidth >20 MHz.

-	Apply different gap sizes for different combinations of NR channel bandwidth (≤20 or >20 MHz) at the edges of the gap, as shown in figures 6.6.3-1 to 6.6.3-8.

Figure 6.6.3-1. CACLR1 scenario for 20MHz channel bandwidth (min frequency separation)

Figure 6.6.3-2. CACLR1 scenario for 20MHz channel bandwidth (max frequency separation)

Figure 6.6.3-3. CACLR2 scenario for 20MHz channel bandwidth (min frequency separation)

Figure 6.6.3-4. CACLR2 scenario for 20MHz channel bandwidth (max frequency separation)

Figure 6.6.3-5. CACLR1 scenario for 25MHz channel bandwidth (min frequency separation)

Figure 6.6.3-6. CACLR1 scenario for 25MHz channel bandwidth (max frequency separation)

Figure 6.6.3-7 CACLR2 scenario for 25MHz channel bandwidth (min frequency separation)

Figure 6.6.3-8. CACLR2 scenario for 25MHz channel bandwidth (max frequency separation)

Based on above agreements, the CACLR requirement is specified in Table 6.6.3-2.

Table 6.6.3-2: Base Station CACLR in FR1

| BS Channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | 5 ≤ Wgap < 15 (Note 3) 5 ≤ Wgap < 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | 10 < Wgap < 20 (Note 3) 10 ≤ Wgap < 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| 25, 30, 40, 50, 60, 70, 80, 90,100 | 20 ≤ Wgap < 60 (Note 4) 20 ≤ Wgap < 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | 40 < Wgap < 80 (Note 4) 40 ≤ Wgap < 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

Based on above agreements, the ACLR requirement in non-contiguous spectrum or multiple bands is specified in Table 6.6.3-3.

Table 6.6.3-3: Base Station ACLR in non-contiguous spectrum or multiple bands in FR1

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

<End of change>

# References

[1]	R4-1801020, “WF on NR CACLR requirement for non-continuous allocation”, Nokia, Nokia Shanghai Bell, ZTE, NTT DOCOMO.

[2]	R4-1800270, “TR 38.817-02 v0.6.0: General aspects for BS RF for NR”, Ericsson.