3GPP TSG-RAN4 #85 	R4-1802825

# Athens, GR, February 26th , March 2nd 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0006 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pcmax for Rel-15 EN-DC FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, LGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Pcmax definition for EN-DC FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Added the Pcmax text to 6.2.5 sub-clause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 EN-DC is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

### 6.2.5	Configured maximum output power for EN-DC

For inter-band and intra-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX c,E-UTRA(p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L,c,E-UTRA (p) ≤  PCMAX,c,E-UTRA (p) ≤  PCMAX_H,c,E-UTRA (p)

where PCMAX_L,c,E-UTRA and PCMAX_H,c,E-UTRA are the limits for a serving cell c as specified in 36.101 sub-clause 6.2.5.

The configured maximum output power PCMAX c,NR (q) in slot q for the configured NR carrier shall be set within the bounds:

PCMAX_L,c,NR (q) ≤  PCMAX,c,NR (q) ≤  PCMAX_H,c,NR (q)

where PCMAX_L,c,NR and PCMAX_H,c,NR are the limits for a serving cell c as specified in 38.101-1.

ΔTIB,c specified in sub-clause 6.2.7 for EN-DC and any other additional power reductions parameters specified in sub-clauses 6.2.3 and 6.2.4 for EN-DC are applicable to PCMAX,c,E-UTRA and PCMAX,c,NR evaluations.

If the transmissions from NR and E-UTRA do not overlap, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions for inter-band and intra-band, the UE is allowed to set its configured maximum output power PCMAX,c,E-UTRA and PCMAX,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its total configured maximum output power PCMAX.

When an UL subframe transmission p from E-UTRA overlap with a slot q from the NR, then for PCMAX (p,q)  evaluation, the E-UTRA subframe p is taken as TREF period and always considered as the reference measurement duration and the following rules are applicable.

The total UE configured maximum output power PCMAX (p,q) in a subframe p of CG 1 and a scheduling unit (slot) q of CG 2 that overlap in time shall be evaluated for each Teval over the TREF and set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,c,E-UTRA (p) + pCMAX_L,c,NR (q)], PPowerClass, EN-DC}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c, E-UTRA (p) + pCMAX_H,c,NR (q)], PPowerClass, EN-DC}

where

- pCMAX_L,c,E-UTRA (p) , pCMAX_H,c, E-UTRA (p) , pCMAX_L,c,NR (q), and pCMAX_H,c,NR (q) are their respective high and low limits expressed in linear scale;

- PPowerClass, EN-DC is defined in sub-clause 6.2.2.1 for intra-band EN-DC and 6.2.2.2 for inter-band EN-DC respectively;

TREF and Teval are specified in Table 6.2.5-1 when same or different subframes and slots durations are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF.

PPowerClass ,EN-DC shall not be exceeded by the UE during any period of time.

Table 6.2.5-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | NR slot |

P’CMAX_L    and P’CMAX_H  are the limits per UE for TREF duration and are defined as follows

P’CMAX_L   = MIN {PCMAX_L   (p,q) , PCMAX_L  (p,q+1), … , PCMAX_L  (p,q+n) }

P’CMAX_H  = MAX {PCMAX_H   (p,q) , PCMAX_H  (p,q+1), … , PCMAX_H  (p,q+n), }

where PCMAX_L   and PCMAX_H are the applicable limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference unit duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

P’CMAX_L   –  TLOW (P’CMAX_L)  ≤  PUMAX  ≤  P’CMAX_H + THIGH (P’CMAX_H)

with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5-2.

Table 6.2.5-2: PCMAX tolerance for Dual Connectivity LTE-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |

### ---------------------------------------End of Change 1-------------------------------