3GPP TSG-RAN WG4 Meeting 86	R4-1803137

Athens, Greece, Feb. 26-March 02, 2018

Title:	Acquisition of NTA_Offset for Uplink Transmission

Release:	Release 15

Work Item:	NR_newRAT

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:	Muhammad Kazmi

E-mail Address:	muhammad.kazmi@ericsson.com

Attachments:	None

- Overall Description:

RAN4 has specified different values of NTA_Offset for uplink transmission on a TDD carrier in frequency range#1 (FR1) for these two cases: when no NR-LTE coexistence is used and when NR-LTE coexistence is used on the TDD carrier in FR1. The corresponding values of NTA_Offset expressed in Tc are reproduced below from Section 7.1.2-2, Table 7.1.2-2 [TS 38.133]:

# Table 7.1.2-2: The Value of [TS 38.133]

| Duplex Mode of cell used for uplink transmission |  |
| --- | --- |
| FDD in FR1 or FR2 | 0 (Note) |
| TDD in FR1 without LTE-NR coexistence | 25560 |
| TDD in FR1 for LTE-NR coexistence | 39936 |
| TDD in FR2 | 13763 |
| Note: Regardless of whether LTE-NR coexistence is configured or not in FR1 |  |

The UE needs to apply an appropriate value of  NTA_Offset for random access (RA) transmission on a TDD carrier in FR1 during initial access procedure. It is therefore beneficial for the UE to know before the initial RA transmission on a TDD carrier in FR1 whether NR-LTE coexistence is used or not on that TDD carrier. RAN4 therefore requests RAN2 to provide this information in the system information to the UE in RRC idle state.

RAN4 is also investigating the feasibility of having two values of NTA_Offset in FR2 and two values of NTA_Offset in FR1 FDD. If this is considered feasible then RAN4 will inform RAN2 about the need for similar signalling for NTA_Offset in FR2 and two values of NTA_Offset in FR1 FDD in the future meeting.

2. Actions:

To RAN1: RAN4 kindly requests RAN2 to specify signalling to inform the UE, before the initial access procedure, whether NR-LTE co-existence is used or not on a TDD carrier in FR1.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #86bis					April 16-20, 2018, Melbourne, Australia

TSG RAN WG4 Meeting #87					May 21-25, 2018, Busan, South Korea