3GPP TSG-RAN WG4 Meeting #86	R4-1803377

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Treatment of Scell MSD which is not from highest IM

Agenda Item: 			7.2.2

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting WF on general MSD principles for EN-DC band combinations was agreed and some MSD requirements were introduced into specifications. In this contribution we discuss further refinement of MSD principles.

# 2	Discussion

As a example we use Table 7.3.2.1.5-1 (presented in Annex) which was introduced into TS 38.101-3 in previous RAN4 meeting. This table has 4 different EN-DC configurations and 21 different MSD requirements. Looking the amount of MSD requirements it seems that all MSD cases were specified although the WF [2] stated

- Proposal1: Basic principle for EN-DC UE should be follow the agreed WF [1] as a baseline in LTE 2UL CA. It means only the highest MSD is specified automatically in TS38.101-3.

  - Some exceptions should be accepted based on request of the proponent of the EN-DC band combination.

  - Multiple IMDs for each CA band combos will be studied and analyzed in technical report.

Content of Table 7.3.2.1.5-1 is summarized in Table 1. MSD highlighted in red are the ones where MSD affects Pcell reception. It was our understanding that PCell MSD is specified only for 2UL/2DL table and not repeated in 2UL/xDL table. Furthermore DC_5A-7A-7A_n78A specifies three different MSD requirements IMD2, IMD4 and IMD5.

Proposal 1: RAN4 to agree that PCell MSD is only specified for 2UL/2DL configurations and not repeated for 2UL/xDL configuration.

Second issue is that for DC_1A-5A-7A_n78A and DC_3A-5A-7A_n78A Scell MSD is specified for IMD2 and IMD5. Considering that basic principle even for Pcell MSD is that only highest MSD is specified we propose that for Scell MSD it is agreed that ONLY highest MSD is specified.

Proposal 2: In case of Scell MSD in 2UL/xDL configuration only the highest MSD is specified.

Table 1: Analysis of MSD requirements

|  | victim | agressor | IM |
| --- | --- | --- | --- |
| DC_1A-5A-7A_n78A | 1 | 1+78 | IMD4 |
|  | 5 | 5+78 | IMD4 |
|  | 1 | 5+78 | IMD3 |
|  | 5 | 1+78 | IMD5 |
|  | 7 | 1+78 | IMD4 |
|  | 1 | 7+78 | IMD4 |
|  | 7 | 5+78 | IMD2 |
|  | 5 | 7+78 | IMD2 |
|  | 5 | 7+78 | IMD5 |
| DC_3A-5A-7A_n78A | 5 | 5+78 | IMD4 |
|  | 7 | 5+78 | IMD2 |
|  | 5 | 7+78 | IMD2 |
|  | 5 | 7+78 | IMD5 |
|  | 3 | 3+78 | IMD2 |
|  | 3 | 3+78 | IMD4 |
| DC_3A-7A-7A_n78A | 3 | 3+78 | IMD2 |
|  | 3 | 3+78 | IMD4 |
| DC_5A-7A-7A_n78A | 5 | 5+78 | IMD4 |
|  | 7 | 5+78 | IMD2 |
|  | 5 | 7+78 | IMD2 |
|  | 5 | 7+78 | IMD5 |

# 3	Conclusion

In this contribution we have discussed further refinement of MSD principles and made following proposals.

Proposal 1: RAN4 to agree that PCell MSD is only specified for 2UL/2DL (including LTE and NR configurations) and not repeated for 2UL/xDL configuration.

Proposal 2: Scell MSD is only specified in 2UL/3DL configuration(including LTE and NR configurations)..

# 4	Reference

[1] R4-1800544, WF on general MSD principles for EN-DC band combinations, LG Electronics, RAN4 AH1801

# Annex

Table 7.3.2.1.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA or NR band | UL Fc (MHz) |  |  | UL/DL BW (MHz) | UL CLRB |  |  | DL Fc (MHz) |  |  | MSD (dB) |  |  |  |  |  | Duplex mode |  | IMD order |  |  |  | Single UL allowed |
| DC_1A-5A-7A_n78A | 1 |  |  | 1930 |  |  |  | 5 |  |  | 25 |  |  | 2120 |  |  |  | 8.3 |  | FDD |  | IMD4 ¦fB78-3*fB1¦ |  |  |  |
|  | 5 |  |  | 844 |  |  |  | 5 |  |  | 25 |  |  | 889 |  |  |  | N/A |  | FDD |  | N/A |  |  |  |
|  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  |  |  | N/A |  | FDD |  | N/A |  |  |  |
|  | n78 |  |  | 3670 |  |  |  | 10 |  |  | 52 |  |  | 3670 |  |  |  | N/A |  | TDD |  | N/A |  |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1950 |  |  |  | 5 |  |  | 25 |  |  | 2140 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 5 |  |  | 844 |  |  |  | 5 |  |  | 25 |  |  | 889 |  |  |  | 8.3 |  | FDD |  |  | IMD4 ¦fB78-3*fB5¦ |  |  |
|  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | n78 |  |  | 3421 |  |  |  | 10 |  |  | 52 |  |  | 3421 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1932 |  |  |  | 5 |  |  | 25 |  |  | 2122 |  |  |  | 18.1 |  | FDD |  |  | IMD3 ¦fB78-2*fB5¦ |  |  |
|  | 5 |  |  | 829 |  |  |  | 5 |  |  | 25 |  |  | 874 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | n78 |  |  | 3780 |  |  |  | 10 |  |  | 52 |  |  | 3780 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1975 |  |  |  | 5 |  |  | 25 |  |  | 2165 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 5 |  |  | 840 |  |  |  | 5 |  |  | 25 |  |  | 885 |  |  |  | 3.1 |  | FDD |  |  | IMD5 ¦2*fB78-3*fB1¦ |  |  |
|  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | n78 |  |  | 3405 |  |  |  | 10 |  |  | 52 |  |  | 3405 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1977.5 |  |  |  | 5 |  |  | 25 |  |  | 2167.5 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 5 |  |  | 840 |  |  |  | 5 |  |  | 25 |  |  | 885 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 7 |  |  | 2507.5 |  |  |  | 5 |  |  | 25 |  |  | 2627.5 |  |  |  | 9.1 |  | FDD |  |  | IMD4 ¦fB78-3*fB1¦ |  |  |
|  | n78 |  |  | 3305 |  |  |  | 10 |  |  | 52 |  |  | 3305 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1950 |  |  |  | 5 |  |  | 25 |  |  | 2140 |  |  |  | 8.7 |  | FDD |  |  | IMD4 ¦2*fB78-2*fB7¦ |  |  |
|  | 5 |  |  | 840 |  |  |  | 5 |  |  | 25 |  |  | 885 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 7 |  |  | 2510 |  |  |  | 10 |  |  | 50 |  |  | 2630 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | n78 |  |  | 3310 |  |  |  | 10 |  |  | 52 |  |  | 3310 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1950 |  |  |  | 5 |  |  | 25 |  |  | 2140 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 5 |  |  | 844 |  |  |  | 5 |  |  | 25 |  |  | 889 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 7 |  |  | 2525 |  |  |  | 5 |  |  | 25 |  |  | 2645 |  |  |  | 30.1 |  | FDD |  |  | IMD2 ¦fB78-fB5¦ |  |  |
|  | n78 |  |  | 3489 |  |  |  | 10 |  |  | 52 |  |  | 3489 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1950 |  |  |  | 5 |  |  | 25 |  |  | 2140 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 5 |  |  | 834 |  |  |  | 5 |  |  | 25 |  |  | 879 |  |  |  | 30.2 |  | FDD |  |  | IMD2 ¦fB78-fB7¦ |  |  |
|  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | n78 |  |  | 3429 |  |  |  | 10 |  |  | 52 |  |  | 3429 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_1A-5A-7A_n78A | 1 |  |  | 1950 |  |  |  | 5 |  |  | 25 |  |  | 2140 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | 5 |  |  | 830 |  |  |  | 5 |  |  | 25 |  |  | 875 |  |  |  | 3.3 |  | FDD |  |  | IMD5 ¦2*fB78-3fB7¦ |  |  |
|  | 7 |  |  | 2525 |  |  |  | 5 |  |  | 25 |  |  | 2645 |  |  |  | N/A |  | FDD |  |  | N/A |  |  |
|  | n78 |  |  | 3350 |  |  |  | 10 |  |  | 52 |  |  | 3350 |  |  |  | N/A |  | TDD |  |  | N/A |  |  |
| DC_3A-5A-7A_n78A | 3 |  |  | 1730 |  |  |  | 5 |  |  | 25 |  |  | 1825 |  |  | N/A |  |  | FDD |  |  | N/A |  |  |
|  | 5 |  |  | 844 |  |  |  | 5 |  |  | 25 |  |  | 889 |  |  | 8.3 |  |  | FDD |  |  | IMD4 ¦fB78-3*fB5¦ |  |  |
|  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  |  | N/A |  |  | FDD |  |  | N/A |  |  |
|  | n78 |  |  | 3421 |  |  |  | 10 |  |  | 52 |  |  | 3421 |  |  | N/A |  |  | TDD |  |  | N/A |  |  |
| DC_3A-5A-7A_n78A | 3 |  |  | 1730 |  |  |  |  | 5 |  |  | 25 |  |  | 1825 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | 5 |  |  | 844 |  |  |  |  | 5 |  |  | 25 |  |  | 889 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | 7 |  |  | 2525 |  |  |  |  | 5 |  |  | 25 |  |  | 2645 |  | 30.1 |  |  | FDD |  |  |  | IMD2 ¦fB78-fB5¦ |  |
|  | n78 |  |  | 3489 |  |  |  |  | 10 |  |  | 52 |  |  | 3489 |  | N/A |  |  | TDD |  |  |  | N/A |  |
| DC_3A-5A-7A_n78A | 3 |  |  | 1730 |  |  |  |  | 5 |  |  | 25 |  |  | 1825 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | 5 |  |  | 834 |  |  |  |  | 5 |  |  | 25 |  |  | 879 |  | 30.2 |  |  | FDD |  |  |  | IMD2 ¦fB78-fB7¦ |  |
|  | 7 |  |  | 2550 |  |  |  |  | 5 |  |  | 25 |  |  | 2670 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | n78 |  |  | 3429 |  |  |  |  | 10 |  |  | 52 |  |  | 3429 |  | N/A |  |  | TDD |  |  |  | N/A |  |
| DC_3A-5A-7A_n78A | 3 |  |  | 1730 |  |  |  |  | 5 |  |  | 25 |  |  | 1825 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | 5 |  |  | 830 |  |  |  |  | 5 |  |  | 25 |  |  | 875 |  | 3.3 |  |  | FDD |  |  |  | IMD5 ¦2*fB78-3fB7¦ |  |
|  | 7 |  |  | 2525 |  |  |  |  | 5 |  |  | 25 |  |  | 2645 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | n78 |  |  | 3350 |  |  |  |  | 10 |  |  | 52 |  |  | 3350 |  | N/A |  |  | TDD |  |  |  | N/A |  |
| DC_3A-5A-7A_n78A | 3 |  |  | 1740 |  |  |  |  | 5 |  |  | 25 |  |  | 1835 |  | [26] |  |  | FDD |  |  |  | IMD2 ¦fB78-fB3¦ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [28.75] |  |  |  |  |  |  |  |  |
|  | 5 |  |  | 840 |  |  |  |  | 5 |  |  | 25 |  |  | 885 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | 7 |  |  | 2550 |  |  |  |  | 5 |  |  | 25 |  |  | 2670 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | n78 |  |  | 3575 |  |  |  |  | 10 |  |  | 25 |  |  | 3575 |  | N/A |  |  | TDD |  |  |  | N/A |  |
|  | n78 |  |  | 3710 |  |  |  |  | 10 |  |  | 25 |  |  | 3710 |  | N/A |  |  | TDD |  |  |  | N/A |  |
| DC_3A-5A-7A_n78A | 3 |  |  | 1770 |  |  |  |  | 5 |  |  | 25 |  |  | 1865 |  | [8.0] |  |  | FDD |  |  |  | IMD4 ¦fB78-3*fB3¦4 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [10.75] |  |  |  |  |  |  |  |  |
|  | 5 |  |  | 840 |  |  |  |  | 5 |  |  | 25 |  |  | 885 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | 7 |  |  | 2550 |  |  |  |  | 5 |  |  | 25 |  |  | 2670 |  | N/A |  |  | FDD |  |  |  | N/A |  |
|  | n78 |  |  | 3445 |  |  |  |  | 10 |  |  | 52 |  |  | 3445 |  | N/A |  |  | TDD |  |  |  | N/A |  |
| DC_3A-7A-7A_n78A | 3 |  |  | 1740 |  |  |  |  | 5 |  |  | 25 |  | 1835 |  | [26] |  |  |  | FDD |  |  |  | IMD2 ¦fB78-fB3¦ |  |
|  | 7 |  |  | TBD |  |  |  |  | TBD |  |  | TBD |  | TBD |  | [28.75] |  |  |  | FDD |  |  |  | TBD |  |
|  | n78 |  |  | 2550 |  |  |  |  | 5 |  |  | 25 |  | 2670 |  | N/A |  |  |  | FDD |  |  |  | N/A |  |
|  | n78 |  |  | 3575 |  |  |  |  | 10 |  |  | 25 |  | 3575 |  | N/A |  |  |  | TDD |  |  |  | N/A |  |
| DC_3A-7A-7A_n78A | 3 |  |  | 1770 |  |  |  |  | 5 |  |  | 25 |  | 1865 |  | [8.0] |  |  |  | FDD |  |  |  | IMD4 ¦fB78-3*fB3¦4 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [10.75] |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  | 2550 |  |  |  |  | 5 |  |  | 25 |  | 2670 |  | N/A |  |  |  | FDD |  |  |  | N/A |  |
|  | n78 |  |  | 3445 |  |  |  |  | 10 |  |  | 52 |  | 3445 |  | N/A |  |  |  | TDD |  |  |  | N/A |  |
| DC_5A-7A-7A_n78A |  | 5 |  |  | 844 |  |  |  | 5 |  |  | 25 |  |  | 889 |  | 8.3 |  |  | FDD |  |  | IMD4 ¦fB78-3*fB5¦ |  |  |
|  |  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  | N/A |  |  | FDD |  |  | N/A |  |  |
|  |  | n78 |  |  | 3421 |  |  |  | 10 |  |  | 52 |  |  | 3421 |  | N/A |  |  | TDD |  |  | N/A |  |  |
| DC_5A-7A-7A_n78A |  | 5 |  |  | 844 |  |  |  | 5 |  |  | 25 |  |  | 889 |  | N/A |  |  | FDD |  |  | N/A |  |  |
|  |  | 7 |  |  | 2525 |  |  |  | 5 |  |  | 25 |  |  | 2645 |  | 30.1 |  |  | FDD |  |  | IMD2 ¦fB78-fB5¦ |  |  |
|  |  | n78 |  |  | 3489 |  |  |  | 10 |  |  | 52 |  |  | 3489 |  | N/A |  |  | TDD |  |  | N/A |  |  |
| DC_5A-7A-7A_n78A |  | 5 |  |  | 834 |  |  |  | 5 |  |  | 25 |  |  | 879 |  | 30.2 |  |  | FDD |  |  | IMD2 ¦fB78-fB7¦ |  |  |
|  |  | 7 |  |  | 2550 |  |  |  | 5 |  |  | 25 |  |  | 2670 |  | N/A |  |  | FDD |  |  | N/A |  |  |
|  |  | n78 |  |  | 3429 |  |  |  | 10 |  |  | 52 |  |  | 3429 |  | N/A |  |  | TDD |  |  | N/A |  |  |
| DC_5A-7A-7A_n78A |  | 5 |  |  | 830 |  |  |  | 5 |  |  | 25 |  |  | 875 |  | 3.3 |  |  | FDD |  |  | IMD5 ¦2*fB78-3fB7¦ |  |  |
|  |  | 7 |  |  | 2525 |  |  |  | 5 |  |  | 25 |  |  | 2645 |  | N/A |  |  | FDD |  |  | N/A |  |  |
|  |  | n78 |  |  | 3350 |  |  |  | 10 |  |  | 52 |  |  | 3350 |  | N/A |  |  | TDD |  |  | N/A |  |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |