3GPP TSG-RAN4 Meeting #86 	R4-1803503

# Athens, Greece, 26th February – 2nd March, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0002 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MTTD and MRTD requirements for synchronous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-2-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The MTTD and MRTD requirements for EN-DC operation are completed in RAN4#85 meeting; however, there is some TBD values need to be determined. The MTTD and MRTD requirements in current specification need more organized to make it clear and easy to read. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the TBD to the proposed value for MTTD and MRTD requirements Update some changes for MTTD and MRTD requirements, and add one sub-section of MTTD and MRTD requirements for intra-band EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MTTD and MRTD requirements are not complete. The specification for MTTD and MRTD requirements are not easy to read. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.5 and 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

7.5	Maximum Transmission Timing Difference

7.5.1	Introduction

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated EN-DC.

7.5.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1. The requirements for asynchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA FDD- NR FDD, E-UTRA FDD-NR TDD and E-UTRA TDD-NR FDD inter-band asynchronous EN-DC.

Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| Note1 : For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-2 provided that the UE indicates that it is capable of synchronous EN-DC [16]. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

Table 7.5.2-2 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120Note1 | 35.21 |
| Note 1: For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

7.5.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 provided the UE indicates that it is capable of asynchronous EN-DC  [16]. The requirements for asynchronous EN-DC are applicable for E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band asynchronous EN-DC.

No uplink transmission timing difference is applicable for synchronous EN-DC.

7.6   Maximum Receive Timing Difference

7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated for EN-DC.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of different carriers to be aggregated NR carrier aggregation.

7.6.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1. The requirements for asynchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA FDD- NR FDD, E-UTRA FDD- NR TDD and E-UTRA TDD- NR FDD inter-band EN-DC.

Table 7.6.2-1 Maximum receive timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120 | 62.5 |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2 : For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-2 provided that the UE indicates that it is capable of synchronous EN-DC[16]. The requirements for synchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA TDD- NR FDD and E-UTRA FDD- NR TDD inter-band EN-DC.

Table 7.6.2-2 Maximum receive timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2: For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

7.6.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 provided the UE indicates that it is capable of asynchronous EN-DC [16]. The requirements for asynchronous EN-DC are applicable for E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-3 provided the UE indicates that it is only capable of synchronous EN-DC [16]. The requirements for synchronous EN-DC are applicable for E-UTRA TDD- NR TDD and E-UTRA FDD- NR FDD intra-band EN-DC.

Table 7.6.2-3 Maximum receive timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

<< End of Change >>