3GPP TSG-RAN WG4 Meeting 86	R4-1803570

Athens, Greece, Feb. 26-March 02, 2018

Title:	Acquisition of NTA_Offset for Uplink Transmission

Release:	Release 15

Work Item:	NR_newRAT

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:	Muhammad Kazmi

E-mail Address:	muhammad.kazmi@ericsson.com

Attachments:	None

- Overall Description:

RAN4 has specified different values of NTA_Offset for uplink transmission on a TDD carrier in frequency range#1 (FR1) for these two cases: when LTE cell and NR cell are not on the same TDD carrier in FR1 and when NR cell and LTE cell are on the same TDD carrier in FR1. The corresponding values of NTA_Offset for these two cases shall be defined in Section 7.1.2-2, Table 7.1.2-2 of TS 38.133.

The UE needs to apply an appropriate value of  NTA_Offset for random access (RA) transmission on a TDD carrier in FR1 during initial access procedure. It is therefore beneficial for the UE to know before the initial RA transmission on a TDD carrier in FR1 whether NR cell and LTE cell are on the same TDD carrier in FR1 or not. RAN4 therefore requests RAN2 to provide this information in the system information to the UE in RRC idle state.

2. Actions:

To RAN1: RAN4 kindly requests RAN2 to specify signalling to inform the UE, before the initial access procedure, whether NR cell and LTE cell are on the same TDD carrier in FR1 or not.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #86bis					April 16-20, 2018, Melbourne, Australia

TSG RAN WG4 Meeting #87					May 21-25, 2018, Busan, South Korea