3GPP TSG-RAN WG4 Meeting #86                                                                                  R4-1801582

# Athens, GR, 26th Feb  – 2nd March, 2018

Title:                           Discussion on Pcompensation type parameters in S-criteria for NR

Source: 	CMCC

Agenda Item:	7.9.5.1

Document for:	Discussion and decision

1    Introduction

RAN2 has discussed cell quality measurements for NR Idle/Inactive mobility in January adhoc meeting. The agreements are listed as follows. In general, LTE idle mode cell selection/reselection mechanisms are applied to NR.

- Cell selection criterion (S criterion) is applied to cell selection and reselection. - Both RSRP and RSRQ are considered in S criterion. - From RAN2 perspective compensation parameter(s) is needed to S criterion. - Cell reselection for intra frequency case and equal priority inter frequency case, criterion (R criterion) is applied to cell reselection. - Qhyst and Qoffset (including both cell specific and carrier specific one) can be applied to R criterion - RSRP is applied to R criterion

Regarding the compensation parameter (s) in Agreement#3, RAN2 would like to ask RAN4 whether a Pcompensation type parameter(s) is also needed in S-criteria for NR as in LTE and if there are any additional use cases and how it is applicable, e.g. for SUL.

In this contribution, we discuss the Pcompensation type parameter(s) that needed to be considered in S-criteria for NR.

2    Compensation parameter for different UE power classes

In E-UTRA, the cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 36.101 [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 36.101 [33] |

In E-UTRA, Pcompensation parameter is introduced in order to accommodate multiple UE power classes (e.g. power class 2 and power class 3).

In NR, UE power class 2 and 3 are introduced for n41, n77, n78 and n79 in Rel-15. The use case and scenario is same as LTE. Hence the Pcompensation parameter as LTE is needed for NR cell selection S-criteria in order to accommodate multiple UE power classes. The applicable way is also same as LTE, which is:

| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |
| --- | --- |

Proposal 1: Pcompensation type parameter for different UE power classes is needed in S-criteria for NR, and the applicable way is same as LTE.

3    Compensation parameter for SUL

According to LS [1], RAN2 also discussed the SUL impact on cell selection S-criteria for NR. SUL is a supplementary uplink frequency, which transmits only uplink resource from NR perspective. When SUL is deployed, there are 2ULs configured for one DL of the same cell. In this section, we will discuss the compensation parameter in S-criteria for SUL.

## 3.1 Uplink and downlink imbalance in NR system

In Table1 and 2, we preliminary calculated the MAPL of control channel and traffic channel for both uplink and downlink based on 3.5GHz frequency.

The calculation is based on +26dBm UE maximum output power and 2Tx, the results show that even with the UE maximum output power of +26dBm and 2Tx, NR system is still UL coverage limited, both for the Control Channel and the Traffic Channel. The obvious imbalance between UL and DL coverage will definitely impact the subscribers’ experience.

## Observation 1:  In NR system, the uplink coverage is limited compared to downlink coverage for both control and traffic channel.

Table 1:  The PRACH MAPL and PDCCH MAPL of 3.5GHz 5G NR’s Control Channel

| Control Channel | PRACH MAPL (dB) | PDCCH MAPL (dB) |
| --- | --- | --- |
| AntennaConfig. | 2Tx*64Rx | 64Tx*4Rx |
| Noise Power (dB) | -128.7 | -125.2 |
| Sensitivity - Composite | -140.5 | -130.7 |
| Total System loss Margin (dB)(Dense Urban/Urban) | 39.6 | 42.6 |
| SNR for MCS Level - 1%BLER(dB) | -11.8 | -5.5 |
| 3.5GHz | 125.87@26dBm/20MHz | 129.85@46dBm/20MHz |

Table 2:  The UL MAPL and DL MAPL of 3.5GHz 5G NR’s Traffic Channel

| Traffic Channel | Time slot allocation (UL:DL) | UL MAPL (dB) | DL MAPL (dB) |
| --- | --- | --- | --- |
| 3.5GHz DL 8Mbps + UL 512Kbps, UE: 2T4R, BS: 64T64R | 1:3 | 118.62 @26dBm/20MHz | 128.92 @46dBm/20MHz |
|  | 2:2 | 121.59 @26dBm/20MHz | 126.79 @46dBm/20MHz |

## 3.2 SUL impact on cell selection

In order to improve the uplink coverage and throughput, SUL is introduced in Rel-15 NR. SUL is a supplementary uplink frequency, which transmits only uplink resource from NR perspective. When SUL is deployed, there are 2ULs configured for one DL of the same cell.

For initial access in a cell configured with SUL, the UE selects the SUL carrier if and only if the measured quality of the DL is lower than a broadcast threshold. Once started, all uplink transmissions of the random access procedure remain on the selected carrier.

Figure 1 shows an example of supplementary uplink. For UE that supports SUL, the uplink coverage is improved and the cell edge is shown as the yellow circle in Figure 1. For UE that does not support SUL or supports SUL but not serving SUL band combination, the cell edge is shown as the small green circle in Figure 1. Hence, if SUL is deployed, SUL UE and non-SUL UE will see different cell edge for the same cell in real network.

Observation 1: If SUL is deployed, SUL UE and non-SUL UE will see different cell edge for the same cell.

Figure 1: Example of supplementary uplink (TS 38.300)

According to RAN2 agreement, Cell selection criterion (S criterion) is applied to cell selection and reselection for NR. As we discussed in observation 1, SUL UE and non-SUL UE will see different cell edge depending on its capabilities. Hence, SUL impact on cell selection criterion S needs to be considered, e.g. compensation parameter should be considered to accommodate multiple UE capabilities (i.e. SUL and non-SUL) for cell selection criterion.

Proposal 2: Pcompensation type parameter for SUL is needed in S-criteria for NR.

Pcompensation type parameter for SUL is used to compensate the coverage gap between regular uplink and the supplementary uplink. Depending on the SUL band combination that will be deployed in real network, the exact compensation value will be different.

Regarding how to apply the SUL compensation parameter in S-criteria, one approach could be adding another Pcompensation value, e.g. Pcompensation_SUL.  For example, as shown in Figure 2, Qrxlevmin is the minimum required Rx level in the cell for normal UE that does not support SUL. Pcompensation_SUL is the compensation value used for UE that supports SUL. Pcompensation_SUL can be transmitted in SIB1. For UE that supports SUL, Srxlev_SUL is calculated as follows:

Srxlev_SUL = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp   +Pcompensation_SUL

Figure 2: SUL compensation in cell selection criterion

Another approach could be transmitting multiple “Qrxlevmin” values corresponding to different UE capabilities. For example, as shown in Figure 3, two  “Qrxlevmin” are transmitted in SIB1. Qrxlevmin is the minimum required Rx level in the cell for normal UE that does not support SUL, Qrxlevmin_SUL is the minimum required Rx level in the cell for SUL UEs. Different UE will look into different values when performing cell selection procedure. The cell selection criterion S is the same as LTE.

Figure 2: Multiple “Qrxlevmin” values for SUL and non-SUL UE

Proposal 3: Two alternatives can be considered to apply the Pcompensation type parameter for SUL:

- Alternative 1:  Add another compensation parameter for SUL in cell selection S-criteria, e.g.

The cell selection criterion S in normal coverage is fulfilled when: Srxlev > 0 AND Squal > 0. where: Srxlev_SUL = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp + Pcompensation_SUL Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

- Alternative 2:  Use the same cell selection S-criteria as LTE, and network transmits two “Qrxlevmin” values for SUL and non-SUL UEs.

The cell selection criterion S in normal coverage is fulfilled when: Srxlev > 0 AND Squal > 0. where: Srxlev_SUL = Qrxlevmeas – (Qrxlevmin_SUL + Qrxlevminoffset) – Pcompensation - Qoffsettemp Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

Other alternatives are not precluded. How to apply the SUL compensation parameter in S-criteria is up to RAN2 decision.

4   Conclusions

In this contribution, we discuss the Pcompensation type parameter(s) that needed to be considered in S-criteria for NR. The observations and proposals are provided as follows:

Proposal 1: Pcompensation type parameter for different UE power classes is needed in S-criteria for NR, and the applicable way is same as LTE.

Observation 1: If SUL is deployed, SUL UE and non-SUL UE will see different cell edge for the same cell.

Proposal 2: Pcompensation type parameter for SUL is needed in S-criteria for NR.

Proposal 3: Two alternatives can be considered to apply the Pcompensation type parameter for SUL:

- Alternative 1:  Add another compensation parameter for SUL in cell selection S-criteria, e.g.

The cell selection criterion S in normal coverage is fulfilled when: Srxlev > 0 AND Squal > 0. where: Srxlev_SUL = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp + Pcompensation_SUL Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

- Alternative 2:  Use the same cell selection S-criteria as LTE, and network transmits two “Qrxlevmin” values for SUL and non-SUL UEs.

The cell selection criterion S in normal coverage is fulfilled when: Srxlev > 0 AND Squal > 0. where: Srxlev_SUL = Qrxlevmeas – (Qrxlevmin_SUL + Qrxlevminoffset) – Pcompensation - Qoffsettemp Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

5   References

- [1] R2-1801658, LS on NR Idle Mode Measurements, RAN2.