3GPP TSG-RAN WG4#86 Meeting	R4-1801816

Athens, Greece, 26 February – 2 March 2018

Agenda item:	7.9.4.1.1

Title: 	Summary of Cell Identification Requirements without Gap

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In TS 38.133 there are two sets of requirements for intra-frequency cell identification and measurements without gap [1].

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if the UE has already identified an SS block with different time index and the same physical cell ID, or the UE has been indicated that the neighbour cell is synchronous with the serving cell. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index Tidentify_intra_without_index = TPSS/SSS_sync + T SSB_measurement_period ms Tidentify_intra_with_index = TPSS/SSS_sync + T SSB_measurement_period + TSSB_time_index ms TPSS/SSS_sync is the PSS/SSS detection delay time T SSB_measurement_period is the SSB-based measurement time period TSSB_time_index is the SSB index acquisition delay time

In RAN4 AH 1801 Meeting, a WF [2] has been agreed for the requirements in scenarios of intra-frequency “without gap”, without considering the SMTC occasions that are partially/fully overlapped with measurement gap and without considering multiple CC scenarios. The formula’s format for the requirements are given by:

For FR1, consensus could be reached to use a unified formula form for TPSS/SSS_sync, T SSB_measurement_period

and TSSB_time_index as below:

max([a constant value], [number of SSB] × SMTC_period ).

As indicated in the WF [2], in order to ensure acceptable mobility performance from system perspective and acceptable power consumption from UE perspective, it is important to guarantee a constant value in the time delay requirement without the impact by other factors. Moreover, the [number of SSB] corresponds to the number of SSBs required to meet the detection and measurement accuracy requirement.

For FR2, consensus could be reached to use an additional parameter N related to Rx beamforming and Rx beam sweeping for requirement definition purpose, but the value of N is TBD. And the unified formula form is given by:

max([a constant value], [number of SSB] × N × SMTC_period )

In order to better align the proposed values from different companies, interested companies are encouraged to provide requirement values according to the listed parameters in the above formula’s format for FR1 and FR2.

In this contribution, we provide the summary of interested companies’ proposals on the cell identification and measurement requirements.

# Discussion

## PSS/SSS Detection

Table 1 and Table 2 below list interested companies’ proposals on PSS/SSS detection requirement for FR1 and FR2, respectively.

Table 1 Summary of PSS/SSS Detection Requirement Proposals in FR1

| PSS/SSS detection in FR1 | Constant value | Number of SSB |
| --- | --- | --- |
| Intel [R4-1801815] | 600 | 6 |
| MediaTek [R4-1801498] | 600 | 6 |
| DoCoMo [R4-1802536] | 600 | 5 |
| Huawei [R4-1802589] | 600 | 6 |
| Ericsson [R4-1802835] | 600 | 5 |
| Note: FR1 formula format is given by max([a constant value], [number of SSB] × SMTC_period ) |  |  |

Table 2 Summary of PSS/SSS Detection Requirement Proposals in FR2

| PSS/SSS detection in FR2 | Constant value | Number of SSB | N |
| --- | --- | --- | --- |
| Intel [R4-1801815] | 600 | 5 | 8 |
| MediaTek [R4-1801498] | N/A | 6 or 7 | N/A |
| LG [R4-1802428] | 800 | 4 or 5 | [4] for UE with low number of Rx beams [12] for UE with high number of Rx beams |
| DoCoMo [R4-1802536] | 200 | 5 | N ≤ 4 |
| Huawei [R4-1802589] | 600 | 6 | 16 |
| Ericsson [R4-1802835] | 600 | 5 | 2 and 1 (optional) |
| Note: FR2 formula format is given by max([a constant value], [number of SSB] × N × SMTC_period ) |  |  |  |

## SSB Index Detection

Table 3 and Table 4 below list interested companies’ proposals on SSB index detection requirement for FR1 and FR2, respectively.

Table 3 Summary of SSB Index Detection Requirement Proposals in FR1

| SSB index detection in FR1 | Constant value | Number of SSB |
| --- | --- | --- |
| Intel [R4-1801815] | 120 | 3 |
| MediaTek [R4-1801498] | 160 | 4 |
| DoCoMo [R4-1802536] | 120 | 3 |
| Huawei [R4-1802589] | 160 | 4 |
| Qualcomm [R4-1802162] | N/A | 3 |
| Note: FR1 formula format is given by max([a constant value], [number of SSB] × SMTC_period ) |  |  |

Table 4 Summary of SSB Index Detection Requirement Proposals in FR2

| SSB index detection in FR2 | Constant value | Number of SSB | N |
| --- | --- | --- | --- |
| Intel [R4-1801815] | 280 | 7 | 1 |
| MediaTek [R4-1801498] | N/A | 5 | N/A |
| LG [R4-1802428] | 140 | 4 | [4] for UE with low number of Rx beams [12] for UE with high number of Rx beams |
| DoCoMo [R4-1802536] | 160 | 4 | N ≤ 4 |
| Huawei [R4-1802589] | 160 | 6 | 16 |
| Qualcomm [R4-1802162] | N/A | 4 | N/A |
| Note: FR2 formula format is given by max([a constant value], [number of SSB] × N × SMTC_period ) |  |  |  |

## SSB Measurement Period

Table 5 and Table 6 below list interested companies’ proposals on SSB measurement requirement for FR1 and FR2, respectively.

Table 5 Summary of SSB Measurement Requirement Proposals in FR1

| SSB measurement in FR1 | Constant value | Number of SSB |
| --- | --- | --- |
| Intel [R4-1801815] | 200 | 5 |
| MediaTek [R4-1801498] | 200 | 5 |
| DoCoMo [R4-1802536] | 200 | 5 |
| Huawei [R4-1802589] | 200 | Г 5×K ˥ ( K is related to gap impact, and can be {1, 4/3, 2} ) |
| Ericsson [R4-1802835] | 200 | 5 |
| Note: FR1 formula format is given by max([a constant value], [number of SSB] × SMTC_period ) |  |  |

Table 6 Summary of SSB Measurement Requirement Proposals in FR2

| SSB measurement in FR2 | Constant value | Number of SSB | N |
| --- | --- | --- | --- |
| Intel [R4-1801815] | 200 | 5 | 8 |
| MediaTek [R4-1801498] | N/A | 5 | N/A |
| LG [R4-1802428] | 200 | 4 | [4] for UE with low number of Rx beams [12] for UE with high number of Rx beams |
| DoCoMo [R4-1802536] | 120 | 3 | N ≤ 4 |
| Huawei [R4-1802589] | 200 | Г 5×K ˥ ( K is related to gap impact, and can be {1, 4/3, 2} ) | 16 |
| Ericsson [R4-1802835] | 200 | 5 | 2 and 1 (optional) |
| Note: FR2 formula format is given by max([a constant value], [number of SSB] × N × SMTC_period ) |  |  |  |

# Conclusion

In this contribution, we provide the summary of interested companies’ proposals on the cell identification and measurement requirements in Table 1 to Table 6.

# References

- TS 38.133, “NR; Requirements for support of radio resource management”.

- R4-1801055, “WF on cell identification requirements in Intra-frequency without gap”, Intel, January 2018.