3GPP TSG-RAN WG4 Meeting #86	R4-1802140

Athens, Greece, 26 February – 2 March 2018

Agenda item:	7.1

Source: 		NTT DOCOMO, INC.

Title: 	Updated UE feature list for NSA

Document for:	Approval

# 1. Introduction

ASN.1 for NSA will be frozen in Mar. 2018, the specification related to capability signaling should be finalized in RAN4 #86 and RAN2 #101 in Feb.

RAN2 needs time to implement the capability signaling in Feb. meeting. Hence, RAN4 needs decide type of capability described as below (R4-1712119) as soon as possible.

  - Type 1:	Layer-1 features relevant to RF characteristics

    - They are reported per band (not per band combination).

  - Type 2:	Layer-1 features that influence baseband processing when UE is configured with NR CA/MR-DC/SUL

    - They are reported in the baseband capability combination signaling.

  - Type 3:	Layer-1 features having both Type 1 and Type 2 characteristics (like the MIMO capability and the other LTE UE capabilities included per band in the band combination or per band combination)

    - It is noted that for type-3 features RAN2 aims to follow the above agreements and working assumptions made for the MIMO capability.

  - Type 4:	Layer-1 features independent from the other features and not categorized into any of the above types

    - In LTE, they are defined as physical layer parameters in TS 36.306. They are reported without linkage to band combination signalling.

To decide type of capability, RAN4 had email discussion from 2/1 - 2/9 as summarized in [1]. In this contribution, we update the UE feature list based on the proposals in [1]

# 2. Updated UE feature list

In this section, we update the UE feature list based on the proposals in [1].

We note that the features are re-ordered and re-categorized as the first two columns. Also, the main updated parts are indicated by red.

| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (See R4-17121 19) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | 1) 60kHz subcarrier spacing for data channel in FR1 |  | Yes | UE does not support 60kHz of subcarrier spacing for data channel | Type 4 if needed | No Need | Applicable only to FR1 |  |  | RAN4 | TBD |  |
|  | 1-2 | Extended CP | 1) Extended CP | 2-1 | Yes |  | Type 4 if needed | [No need] | Yes |  |  | RAN4 | TBD |  |
|  | 1-3 | 64QAM modulation for FR2 PDSCH | 1) 64QAM modulation for FR2 PDSCH |  | Yes |  | NA | No need | Applicable only to FR2 |  |  | RAN4 | Mandatory without capability |  |
|  | 1-4 | 64QAM for PUSCH | 1) 64QAM for PUSCH |  | Yes | 64QAM for PUSCH is not possible | NA | No need | No need |  |  | RAN4 | Mandatory without capability |  |
|  | 1-5 | 256QAM for PDSCH | 1) 256QAM for PDSCH |  | Yes | 256QAM for PDSCH is not possible | For FR1, Type4 if needed For FR2, Type 1 or Type 4 | No need | Yes |  |  | RAN4 | TBD for FR1 Optional for FR2 |  |
|  | 1-6 | 256QAM for PUSCH | 1) 256QAM for PUSCH |  | Yes | 256QAM for PUSCH is not possible | For FR1, Type1 if needed For FR2, Type 1 | No need | Yes |  |  | RAN4 | TBD for FR1 Optional for FR2 |  |
|  | 1-7 | pi/2-BPSK for PUSCH | [1) Pulse-shaped pi/2-BPSK 2) Non pulse-shaped pi/2-BPSK. pi/2-BPSK for PUSCH] |  | Yes | pi/2-BPSK for PUSCH is not possible | Type 4 | No need | Yes |  |  | RAN4 | Optional |  |
|  | 1-8 | pi/2-BPSK for PUCCH format 3/4 | 1) pi/2-BPSK for PUCCH format 3/4 |  | Yes | pi/2-BPSK for PUCCH format 3/4 is not possible | Type 4 | No need | Yes |  | Same capability as pi/2-BPSK for PUSCH | RAN4 | Optional |  |
|  | 1-9 | BWP switching delay |  |  |  |  | TBD in RAN4 #86 |  |  |  |  | RAN4 | TBD |  |
|  | 1-10 | 1-symbol GP for 120KHz SCS in unpaired spectrum | 1) Slot formats with 1-symbol GP(s) for 120KHz SCS in unpaired spectrum |  | Yes |  | Type 1 or Type 4 | Applicable only to TDD | Applicable only to FR2 |  | For 60kHz SCS? | RAN4 | Optional |  |
|  | 1-11 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) ~0us 2) <20us |  | Yes | UE does not support UL subcarrier alignment between LTE and NR for EN-DC with LTE-NR coexistence in UL | Type 3 (per BC) or Type 4 | No Need | Applicable only to FR1 |  |  | RAN4 | TBD |  |
|  | 1-12 | [Support of UL sharing from UE perspective] |  |  |  |  | TBD |  |  |  | Necessity of this feature will be discussed in RAN4 #86 | RAN4 | TBD |  |
|  | 1-13 | 7.5kHz UL raster shift | 1) 7.5kHz UL raster shift |  | Yes |  | Type 1 or Type 4 if needed |  |  |  |  | RAN4 | TBD |  |
|  | 1-14 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL |  | Yes | UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL | TBD |  |  |  | Details will be discussed in RAN4 #86 | RAN4 | TBD |  |
|  | 1-15 | Supplemental uplink with different numerology | 1) Numerology other than that of associated DL |  | Yes | The UE will not be able to access or operate on a SUL carrier | TBD | Need Support not required for an FDD-only UE |  |  | Details will be discussed in RAN4 #86 | RAN4 | TBD |  |
|  | 1-16 | Supplemental uplink with dynamic switch | 1) DCI based selection of PUSCH carrier |  | Yes |  | TBD | Need Support not required for an FDD-only UE |  |  | Details will be discussed in RAN4 #86 | RAN4 | TBD |  |
| UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 TBD |  | Yes | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC For FR2, UE does not support some UE channel bandwidths | Type 1 TBD | No Need | No Need |  | Decision was made in RP-172832 RAN4 will discuss UE mandatory channel bandwidth for FR2 Overall EN-DC/NR CA capability framework will be discussed in RAN4 #86 | RAN2/4 | Optional |  |
|  | 2-2 | Mixed numerologies for CA | TBD |  |  |  | TBD | TBD | TBD |  |  | RAN4 | TBD |  |
|  | 2-3 | Non-contiguous intra-band CA frequency span | TBD |  |  |  | Combine with CA class, Type 1 or Type4 | TBD | TBD |  |  | RAN4 | TBD |  |
|  | 2-4 | Simultaneous reception and transmission for inter band CA or EN-DC (TDD-TDD or TDD-FDD) | 1) Simultaneous reception and transmission for inter CA or EN-DC (TDD-TDD or TDD-FDD) |  | Yes | UE does not support simultaneous reception and transmission for inter CA or EN-DC (TDD-TDD or TDD-FDD) | Type 3 (per BC) or only exceptions are signaled (such as single switched UL) | No Need | No Need |  | For band combination within FR1 (or FR2) bands Per band combination signaling | RAN4 | TBD |  |
|  | 2-5 | Asynchronous FDD-FDD intra-band LTE-NR DC | Asynchronous FDD-FDD intra-band LTE-NR DC |  |  |  | Type 3 (per BC) or Type 4 | Applicable only to FDD | Applicable only to FR1 |  |  | RAN4 | Optional |  |
|  | 2-6 | Non-contiguous UL CP-OFDM |  |  | Yes |  | Type 1 | No | No need |  |  | RAN4 | Mandatory with capability |  |
|  | 2-7 | PA calibration gap |  |  |  |  | TBD | TBD | TBD |  | Necessity of this feature will be discussed in RAN4 #86 | RAN4 | TBD |  |
| Baseband | 3-1 | Independent measurement gap configurations for FR1 and FR2 | 1) measurement gaps for FR1 and FR2 are configured independently |  | Yes | UE does not support independent gap configuration between FR1 and FR2 | Type 4 | No Need | No Need |  |  | RAN4 | Optional |  |
|  | 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | 1) Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement |  | Yes | UE does not support simultaneous reception of data and SS block with different numerologies | Type 4 | No Need | Applicable only to FR1 |  | FFS: inter-band CA | RAN4 | Optional |  |
|  | 3-3 | Number of UE RX ports |  |  |  |  | TBD |  |  |  | FR1: UE supports 2 or 4 RX chains [RP-172788]. FR2: Number of transceiver chains for FR2 is FFS FFS if separate capability signalling is needed or can be coupled with number of MIMO layers Necessity of this feature will be discussed in RAN4 #86 | RAN4 | TBD |  |
|  | 3-4 | Number of UE TX ports |  |  |  |  | TBD |  |  |  | FFS if separate capability signalling is needed or can be coupled with number of MIMO layers Necessity of this feature will be discussed in RAN4 #86 | RAN4 | TBD |  |
| Removed features from RAN4 feature list | r-1 | Single switched UL transmission in EN-DC | 1) single switched UL transmission in EN-DC |  | Yes | UE does not support single switched UL | Type 4 | No Need | Applicable only to FR1 |  | For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5 of RP-1722833) is mandatory with capability signaling. (Decision was made in RAN#78, RP-172833) | RAN4 | Optional |  |
|  | r-2 | Dynamic transmission power sharing between NR and LTE in EN-DC | 1) dynamic transmission power sharing between NR and LTE in EN-DC |  | Yes | Support of single UL operation (operation A with Case 1) is mandatory | [Type 4] | No Need | Applicable only to FR1 |  | This feature is not mandatory in Rel.15. For UEs without this capability, the support of single UL operation (Operation A with Case 1 in Slide 5 of RP-1722833) is mandatory with capability signaling. (Decision was made in RAN#78, RP-172833). | RAN 1/4 | Optional |  |
|  | r-3 | DL MIMO layers (PDSCH MIMO layers) |  |  |  |  |  |  |  |  | Maximum number of MIMO layers shall be supported |  |  |  |
|  | r-4 | UL MIMO layers (PUSCH MIMO layers) |  |  |  |  |  |  |  |  | Maximum number of MIMO layers shall be supported |  |  |  |

# Reference

[1]. R4-1802139, Summary of email discussion on UE feature list for NSA, NTT DOCOMO, Jan. 2018.