3GPP TSG-RAN WG4 Meeting #86	R4-1802365

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	6.19.4.3

Source:	Ericsson

Title:	Simulation assumption of CGI reading for eFeMTC UE

Document for:	Approval

# 1	Introduction

This contribution proposes the simulation assumption for CGI reading requirement for eFeMTC.

# 2	Simulation assumption

Table 1 is the simulation assumption to derive the CGI reading time. Companies are encouraged to provide the simulation results (required CGI acquisition time) with 90% decoding success rate assuming Rel-15 BL UE.

Table 1	Simulation parameters for CGI reading for CE Mode B UE.

| Parameters | Values |
| --- | --- |
| Serving cell SCH Es/Iot | -6dB |
| Target cell SCH Es/Iot | -15dB |
| UE receive antennas | 1 |
| BS transmit antennas | 2 |
| Fading channel | ETU1, EPA1 |
| PBCH repetition | Configured as specified in TS36.211 6.6.4 |
| SIB1-BR repetition level | 16 (schedulingInfoSIB1-BR = 18) |
| TBS for SIB1-BR | 936 bits (schedulingInfoSIB1-BR = 18) |
| MIB assumption | The network does not change the MIB payload information across MIB TTI (40ms) except for the system frame number (SFN) |
| SIB1-BR assumption | The network does not change the SIB1-BR information across SIB1-BR TTI (80ms). |
| Evaluation | 90% of target cell SI decoding success rate. |

# 3	References

R4-157723, “SI reading requirements for eMTC”, Ericsson.

R4-160875, “Simulation assumption for CGI requirements for eMTC”, Huawei, HiSilicon.

R4-1802364, “CGI reading requirements for eFeMTC UE”, Ericsson.