3GPP TSG-RAN WG4 Meeting #86	R4-1802885

Athens, Greece, 26 Feb – 2 Mar, 2018

Source:	NTT DOCOMO, INC.

Title:	Output power level of interference signal for out of band blocking

Agenda item:	6.27.2.2

Document for:	Approval

1	Introduction

In the RAN4 meeting #85, the output power level of interference signal for out of band blocking was decided as a fixed EIRP level at a fixed distance between an AAS-BS and an aggressor antenna. The value of the level and distance is currently [36 dBm] and [30 m] that is calculated under the assumption that antenna gain of receiving antenna is 17 dBi and blocker level at the input port of TRX unit is -15 dBm. However, there is a probability that the antenna gain of the antenna that is connected to TRX unit is much lower than 17 dBi and the blocker level at the input port is also much lower than -15 dBm. This contribution proposes the level of the blocker signal and distance should be increased to achieve the same protection as that for Rel.-13.

2	Discussion

2.1 Level of blocker signal for OOB

In TR37.843 v15.0.0, the output level (EIRP) of blocking signal and distance between AAS BS and aggressor antenna are set to [36 dBm] and [30m] to achieve the -15 dBm at the input port of TRX unit [1].

Observation 1:  [36 dBm] and [30 m] are determined to acheve the -15 dBm of blocking singal at the input port of TRX unit of AAS BS.

These values were obtained under the condition that the gain of the receiving antenna is 17 dBi [1].

Observation 2: Antenna gain of victim is assumed to be 17 dBi.

In AAS BS, the antenna array and TRX units are connected via the RDN as shown in Figure 1[1].

Figure 1. General architecture of OTA AAS BS.[1]

Since the RDN is not specified, there is a probability that an antenna element is directly connected to TRX unit as shown in Figure 2. In this case, the antenna gain will be much smaller than 17 dBi, and this will be the most over-estimation case of TRX unit’s blocking functionality.

Figure 2. An example of actual AAS BS configuration.

Observation 3: It is probable that antenna gain of the antenna connected to the TRX unit in AAS is much smaller than 17 dBi.

For example, the antenna gain of a patch antenna that has horizontal 3-dB beam width of 70 degree that is close to the beam width of co-location reference antenna, 65 degrees, is approximately 6 dB from the results of electromagnetic simulation as shown in Figure 3(a). Figure 3 (b) shows the patch antenna simulation model. The electromagnetic simulation was done by employing Method of Moment (MoM). Frequency is 2 GHz and return loss is more than 15 dB.

(a) Beam pattern 　　　　　　　　　　　　　　　　　 (b) patch antenna simulation model

Figure 3 Simulated antenna gain of patch antenna.

Observation 4: The antenna gain of the antenna that has the similar horizontal beam width as that of the co-location reference antenna is ~6 dBi. This is ~10 dB smaller than 17 dBi.

From above observations, the blocker level should be increased to keep the -15 dBm at input port of TRX unit. The gain of patch antenna is ~10 dB smaller than 17 dBi. Therefore, we propose the new blocker level and distance as follows.

Proposal:  Blocker level and distance should be 46 dBm and 30 m.

3	Summary

In this document, we have discussed the output level of interference signal for OOB requirements except for co-location scenario, and made the following proposal based on the following observations.

Observation 1:  [36 dBm] and [30 m] are determined to acheve the -15 dBm of blocking singal at the input port of TRX unit of AAS BS.

Observation 2: Antenna gain of victim is assumed to be 17 dBi.

Observation 3: Antenna gain of the antenna connected to TRX unit in AAS is probable to be much smaller than 17 dBi.

Observation 4: The antenna gain of the antenna that has the similar horizontal beam width as that of the co-location reference antenna is ~6 dBi. This is 11 dB smaller than 17 dBi.

Proposal:  Blocker level and distance should be 46 dBm and 30 m.

References

- 3GPP TR 37.843 V15.0.0 (2017-12).